3GPP TSG RAN WG1 #121			   R1-2504851

St. Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0663 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### CR on timeline for 8TX PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Samsung), Ericsson, Oppo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following two agreements made in RAN1#120 meeting are not captured in the specifications. Agreement: Introduce following FG for UE 8Tx PUSCH processing capability for codebook. - FG name - UE 8Tx PUSCH processing capability for codebook - Component - 1. Require additional timeline to process two TBs for 8Tx PUSCH - Prerequisite - 40-7-1 or 40-7-2 - Consequence if not supported - UE should process two TBs within legacy timeline - Type - Per FS - Need of FR1/FR2 differentiation - n/a - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64, 128} - For 960kHz SCS: {32,64,128, 256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH - Note that the proposed UE capability is reported by UE only when UE reports the Maximum number of PUSCH MIMO layers to be {5,6,7,8} - Mandatory or optional - Optional with capability signaling Agreement: Introduce following FG for UE 8Tx PUSCH processing capability for non-codebook. - FG name - UE 8Tx PUSCH processing capability for non-codebook - Component - 1. Require additional timeline to process two TBs for 8Tx PUSCH - Prerequisite - 40-7-1 or 40-7-2 - Consequence if not supported - UE should process two TBs within legacy timeline - Type - Per FS - Need of FR1/FR2 differentiation - n/a - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64, 128} - For 960kHz SCS: {32,64,128, 256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH - Note that the proposed UE capability is reported by UE only when UE reports the Maximum number of PUSCH MIMO layers to be {5,6,7,8} - Mandatory or optional - Optional with capability signaling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the additional processing timeline for a PUSCH with 2 TBs in the specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not have enough time to prepare the PUSCH with 2 TBs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.4	UE PUSCH preparation procedure time

If the first uplink symbol in the PUSCH allocation for up to two transport blocks, including the DM-RS, as defined by the slot offset K2 and Koffset, if configured, and the start S and length L of the PUSCH allocation indicated by 'Time domain resource assignment' of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting Tproc,2=max(N2+d2,1+d2+d3+d4)(2048+144)⋅κ2-μ⋅TC+Text+Tswitch,d2,2 after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the PUSCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, the PDCCH candidate that ends later in time is used.

-	N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	For operation with shared spectrum channel access in FR1, is calculated according to [4, TS 38.211], otherwise =0.

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1.

-	If the UE is configured with multiple active component carriers, the first uplink symbol in the PUSCH allocation further includes the effect of timing difference between component carriers as given in [11, TS 38.133].

-	If the scheduling DCI triggered a switch of BWP, d2,2 equals to the switching time as defined in [11, TS 38.133], otherwise d2,2=0.

-	If a PUSCH of a larger priority index would overlap with a PUCCH of a smaller priority index and the PUCCH and PUSCH are not simultaneously transmitted, and the UE is not provided uci-MuxWithDiffPrio for the primary PUCCH group or uci-MuxWithDiffPrioSecondaryPUCCHgroup for the secondary PUCCH group, d2 for the PUSCH of a larger priority is set as reported by the UE; otherwise d2 = 0.

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PUSCH-ServingCellConfig is configured for the cell and set to 'enable'.

-	If the PUSCH indicated by the DCI is overlapping with one or more PUCCH channels, then the transport block(s) is(are) multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the transport block(s) is(are) transmitted on the PUSCH indicated by the DCI.

-	If uplink switching gap is triggered as defined in clause 6.1.6,  equals to the switching gap duration and for the UE configured with higher layer parameter uplinkTxSwitchingOption set to 'dualUL' for uplink carrier aggregation µUL=min(µUL,carrier1, µUL,carrier2), otherwise .

-	d3 is set as reported by UE if the UE is configured with sTx-2Panel and the PUSCH associated with a value of coresetPoolIndex is fully/partially overlapping in time domain and is fully/partially/non-overlapping in frequency domain with another PUSCH that is associated with a different value of coresetPoolIndex on a same serving cell; otherwise d3 = 0.

-	d4 is set as reported by UE if the number of transport blocks in the PUSCH is 2; otherwise d4 = 0.

Otherwise the UE may ignore the scheduling DCI.

The value of  is used both in the case of normal and extended cyclic prefix.

Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |
| 5 | 144 |
| 6 | 288 |

Table 6.4-2: PUSCH preparation time for PUSCH timing capability 2

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 5 |
| 1 | 5.5 |
| 2 | 11 for frequency range 1 |

## *** Unchanged parts are omitted ***