3GPP TSG RAN WG1 #121																		     R1-2504911

St Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0669 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on starting slot for typeII Doppler CSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (NEC), Samsung, Ericsson, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current TS, the earliest of the N4 slot intervals for typeII doppler CSI is defined as slot l=n+δ, and n is the uplink slot in which the CSI is reported. There are several issues unclear, also discussed in our companion contribution [1]: - No clear definition of the starting slot l as uplink slot or downlink slot - No clear definition of subcarrier spacing for the predictiondelay δ - At least when δ=-nCSI_ref, it’s clearly defined nCSI_ref corresponding to downlink subcarrier spacing in section of CSI reference resource, then it’s not suitable to directly add slot n (defined as uplink slot) with δ=-nCSI_ref (which corresponds to downlink) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifiying the definition of starting slot l corresponding to downlink slot - Reusing the definition of the slot n for CSI reference resource, as +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL - Clarifying the predictiondelay δ corresponding to subcarrier spacing configuration for downlink |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Ambiguous on UE to determine the starting slot l for typeII doppler CSI |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.1.4.2	Report quantity configurations

<Unchanged part omitted>

Subject to UE capability, a UE configured with a CodebookConfig with the higher layer parameter vectorLengthDD and a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter vectorLengthDD. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter unitDurationDD, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where  +Nslot,offset,ULCA2μoffset,UL-Nslot,offset,DLCA2μoffset,DL∙2μDL , where n' n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter predictionDelay and corresponds to subcarrier spacing configuration for DL, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability. In the above, μDL and μUL are the subcarrier spacing configurations for DL and UL, respectively, and Nslot, offsetCA and μoffset are determined by higher-layer configured ca-SlotOffset for the cells transmitting the uplink and downlink, as defined in clause 4.5 of [4, TS 38.211].

<Unchanged part omitted>