3GPP TSG RAN WG1 #121			R1-2504952

St Julian’s, Malta, May 19th – 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0704 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CG PUSCH transmission in RACH-less LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During RACH-less LTM cell switch, UE may transmit the first UL via a CG PUSCH to complete the LTM cell switch procedure. For determining valid PUSCH occasion of CG PUSCH, UE checks whether a PUSCH occasion is at least Ngap symbol after a SSB. However, current SPEC requires a UE to obtain ssb-PositionsInBurst from SIB1 of candidate cell, which unnecessarily complicates UE implementation complexity and prolongs interruption time during LTM cell switch. Instead, the UE should obtain rrc-SSB-Subset in cg-LTM-Configuration or ssb-PositionsInBurst from ServingCellConfigCommon, if rrc-SSB-Subset is not provided. It also aligns with the SSBs associated with PUSCH occasion defined in previous paragraph. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For determining valid PUSCH occasion of CG PUSCH, UE obtains indexes of SSBs from rrc-SSB-Subset in cg-LTM-Configuration or ssb-PositionsInBurst in ServingCellConfigCommon, if rrc-SSB-Subset is not provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect UE behavior which increases unnecessary delay of completing LTM cell switch. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 21.1	Configured-grant PUSCH transmission in RACH-less LTM cell switch

< Unchanged parts are omitted >

A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1.

For unpaired spectrum and for SS/PBCH blocks with indexes provided by rrc-SSB-Subset in cg-LTM-Configuration, or ssb-PositionsInBurst in ServingCellConfigCommon if rrc-SSB-Subset is not provided

-	if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	is within UL symbols

-	starts at least Ngap symbols after a last downlink symbol, and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2

< Unchanged parts are omitted >