3GPP TSG-RAN WG1 Meeting #121	R1-2504845

St Julian’s, Malta, May 19th – 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0699 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SSSG reset with BWP switch triggered by RA procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE’s PDCCH monitoring behavior in a target BWP (configured with search space set group switching) due to BWP switch requested by higher layer (e.g. triggered by the RA procedure as in TS 38.321 Clause 5.15) is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the spec text to include the BWP switch requested by higher layer as an additional condition under which UE shall reset the PDCCH monitoring in a BWP configured with search space set group switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential mis-understanding between UE side and NW side on whether UE should reset to PDCCH monitoring behavior in a BWP configured with search space set group switching after BWP switch requested by higher layer (e.g. triggered by the RA procedure as in TS 38.321 Clause 5.15). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis: This CR only impacts UE’s PDCCH monitoring behavior in a BWP configured with search space set group switching after BWP switch requested by higher layer (e.g. triggered by the RA procedure as in TS 38.321 Clause 5.15). If only the UE (or NW) implemented this CR while the NW (UE) does not implement according to this CR, UE and NW may have different assumption on the PDCCH monitoring behavior in a BWP configured with search space set group switching after BWP switch requested by higher layer (e.g. triggered by the RA procedure as in TS 38.321 Clause 5.15). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.4      Search space set group switching and skipping of PDCCH monitoring

<Unchanged Text Omitted>

If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer, or by RA procedure requested by higher layers [11, TS 38.321], or by RRC configuration, the UE

-	resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell when UE is in a PDCCH skipping duration, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP

-	monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell, if the UE is provided searchSpaceGroupIdList-r17.

<Unchanged Text Omitted>