3GPP TSG RAN WG1 #121	R1-2505057

# St Julian’s, Malta, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0574 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of IoT_NTN_Ph3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-06-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of IoT_NTN_Ph3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defining OCC-based reference signals |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for IoT_NTN_Ph3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.6, 10.1.4.1.1.1, 10.1.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 10.1.3.6	Mapping to physical resources

Each NPUSCH codeword can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted  times.

The block of complex-valued symbols  shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [4], and mapped in sequence starting with  to subcarriers assigned for transmission of NPUSCH. The mapping to resource elements  corresponding to the subcarriers assigned for transmission and not used for transmission of reference signals, shall be in increasing order of first the index , then the index, starting with the first slot in the assigned resource unit.

After mapping to slots, the  slots shall be repeated  additional times, before continuing the mapping of  to the following slot, where

For a UE communicating over NTN in FDD operation, if the higher layer parameter npusch-OCC-Enabled is configured, OCC is indicated as activated in DCI Format N0 as described in [3], and MrepNPUSCH≥2,

-	For Δf=15 kHz, after mapping to a slot, the slot shall be repeated NidenticalNPUSCH-1 additional times and pairs of slots shall be multiplied by q(m) before continuing the mapping of z(∙) to the following slot, the process repeats for all the slots in the NPUSCH format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI as described in TS36.212 [3].

-	For Δf=3.75 kHz, after mapping to a data symbol, the data symbol shall repeated NidenticalNPUSCH-1 additional times and pairs of data symbols shall be multiplied by q(m) while skipping the DMRS symbol and guard period within the slot, before continuing the mapping of z(∙) to the following data symbol, the process repeats for all the data symbols in the NPUSCH Format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI as described in TS36.212 [3].

For NPUSCH Format 1 and 2 on frame structure type 2 with ,

-	the NPUSCH transmission is carried out in the first set of  slots spanning over two contiguous uplink subframes not overlapping with any uplink subframe configured as invalid;

-	for TDD configuration 1 and 4, if the starting position for the NPUSCH is indicated as the second of the two contiguous uplink subframes, the NPUSCH transmission is postponed until the start of two consecutive uplink subframes.

If a mapping to  slots or a repetition of the mapping contains a resource element which overlaps with

-	any configured NPRACH resource according to nprach-ParametersList in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigList in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigListMixed in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH and mixedOperationMode as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 in SystemInformationBlockType2-NB and if the UE indicates nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigList in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigListMixed in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH, mixedOperationMode and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource configured for Early Data Transmission and if the NPUSCH transmission is during an Early Data Transmission procedure [12, Clause 7.3b],

then,

-	for  the NPUSCH transmission in overlapped slots is postponed until the next  slots not overlapping with any configured NPRACH resource.

-	for  the NPUSCH transmission in overlapped  slots is postponed until the next  slots starting with the first slot satisfying ns mod 2=0 nsmod 2=0and not overlapping with any configured NPRACH resource.

NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of  is then repeated until MrepNPUSCHNRUNslotsUL slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, for frame structure type 1, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

When higher layer parameter npusch-AllSymbols is set to false, resource elements in SC-FDMA symbols overlapping with a symbol configured with SRS according to srs-SubframeConfig shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH. When higher layer parameter npusch-AllSymbols is set to true, all symbols are transmitted.

If higher layer parameter resourceReservationConfigUL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space with the Resource reservation field in the DCI set to 1 including NPUSCH format 1 transmission without a corresponding NPDCCH, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space,

-	In a subframe for  or a slot for that is overlapping with any fully reserved uplink subframe as defined in clause 16.5 in [4],

-	for , the NPUSCH transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved.

-	for , the NPUSCH transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved.

-	In a subframe for  or a slot for that is not overlapping with any fully reserved uplink subframe, any SC-FDMA symbols overlapping with reserved symbols shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH.

For a UE communicating over NTN, after transmissions (and/or postponements due to NPRACH) of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH according to the UE capability ntn-SegmentedPrecompensationGaps-r17 as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by higher layers based on the UE capability if signalled.

### 10.1.4	Demodulation reference signal

#### 10.1.4.1	Reference signal sequence

##### 10.1.4.1.1	Reference signal sequence for

The reference signal sequence for  is defined by

where the binary sequence  is defined by clause 7.2 and shall be initialised with  at the start of the NPUSCH transmission. The quantity  is given by Table 10.1.4.1.1-1 where  for NPUSCH format 2, and for NPUSCH format 1if group hopping is not enabled, and by clause 10.1.4.1.3 if group hopping is enabled for NPUSCH format 1.

Table 10.1.4.1.1-1: Definition of

| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 |
| 2 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 |
| 3 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 |
| 4 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 |
| 5 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 6 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 |
| 7 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 9 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 |
| 10 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 |
| 11 | 1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 |
| 12 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 |
| 13 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 14 | 1 | 1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 |
| 15 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 |

The reference signal sequence for NPUSCH format 1 is given by:

The reference signal sequence for NPUSCH format 2 is given by

where is defined in Table 5.5.2.2.1-2 with the sequence index chosen according to i=07c8n'+i∙2imod 3 with . For frame structure type 1, n'=ns. For frame structure type 2, n'=ns+nfmod 5 for and n'=ns+nfmod 20 for .

###### 10.1.4.1.1.1	OCC reference signal sequence for NscRU=1 with Δf=15 kHz

For a UE communicating over NTN, the OCC reference signal sequence ru,OCC(n) for NscRU=1 is defined by

ru,OCCMOCCn+m=runqm

n=0,1,…,MrepNPUSCHNslotsULNRUMOCC-1

where

-	MOCC=2

-	MrepNPUSCH≥2

-	m=0, 1

-	the subcarrier indication field in the DCI selects one of

-	q(m)=11

-	q(m)=1-1

For NPUSCH format 1 transmission with Δf=15 kHz, the start of the NPUSCH Format 1 transmission is as described in clause 16.5.1 of [4], when OCC is applied, after the start of the NPUSCH Format 1 transmission the DMRS symbols are transmitted according to ru,OCCMOCCn+m.

###### 10.1.4.1.1.2	OCC reference signal sequence for NscRU=1 with Δf=3.75 kHz

For a UE communicating over NTN, the OCC reference signal sequence ru,OCC(n) for NscRU=1 is defined by

ru,OCCn=run'if nMOCC mod MOCC=m 0otherwise

n=0,1,…,MrepNPUSCHNslotsULNRU-1

n'=n-nMOCC-m

where

-	MOCC=2

-	MrepNPUSCH≥2

-	m=0, 1

-	the DCI selects one of

-	q(m)=11

-	q(m)=1-1

For NPUSCH Format 1 transmission with Δf=3.75 kHz, the start of the NPUSCH Format 1 transmission is as described in clause 16.5.1 of [4] which when OCC is applied is postponed to the next slot whose index satisfies 5nf+ns mod 4=0. After the start of the NPUSCH Format 1 transmission the DMRS symbols are transmitted according to

-	if n mod 4∈0,1

-	ru,OCCn for q(m)=11

-	0 for q(m)=1-1

-	if n mod 4∈2,3

-	0 for q(m)=11

-	ru,OCCn for q(m)=1-1