3GPP TSG-RAN WG1 Meeting #121 	R1-2504966

St. Julian’s, Malta, May 19-23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0377 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-19 OCC for IoT-NTN for LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FUTUREWEI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of Rel-19 OCC for IoT-NTN for LTE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Rel-19 OCC enhancement for IoT-NTN for LTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 IoT-NTN for LTE will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 36.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.4.3.1	DCI Format N0

DCI format N0 is used for the scheduling of NPUSCH and operation on preconfigured UL resources in one UL cell.

The following information is transmitted by means of the DCI format N0:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	Modulation and coding scheme – 4 bits as defined in clause 16.5.1.2 of [3]. This field is only present if format N0 CRC is scrambled by PUR-RNTI.

If format N0 CRC is scrambled by PUR-RNTI and Modulation and coding scheme is set to '1110', the remaining fields are set as follows:

-	ACK or Fallback indicator – 1 bit, where value 0 indicates ACK and value 1 indicates fallback as defined in clause 16.6.4 of [3]

-	NPUSCH repetition adjustment – 3 bits refer to  in Table 16.5.1.1-3 of [3]

-	Timing advance adjustment – 6 bits as defined in clause 16.1.2 of [3]. The field is only present if ACK or Fallback indicator is set to 0.

-	All the remaining bits in format N0 are set to one

Otherwise

-	Modulation and coding scheme and Subcarrier indication for OCC – 10 bits. This field is only present if  and OCC enabled/disabled is OCC enabled.

-	OCC sequence index – 1 bit, where value 0 indicates OCC sequence [1 1] and value 1 indicates OCC sequence [1 -1].

-	Modulation and coding scheme and Subcarrier indication – 9 bits, as defined in 16.5.1.1 and 16.5.1.2 of [3].

-	Subcarrier indication – 6 bits as defined in clause 16.5.1.1 of [3]. This field is not present if  and OCC enabled/disabled is OCC enabled. If higher layer parameter npusch-OCC-Enabled is configured and Repetition number field has a value > 0 and

-	OCC enabled/disabled – 1 bit, where value 0 indicates OCC disabled and value 1 indicates OCC enabled.

-	OCC sequence index – 1 bit, where value 0 indicates OCC sequence [1 1] and value 1 indicates OCC sequence [1 -1]. This field is only present if OCC enabled/disabled is OCC enabled.

-	Subcarrier indication for OCC – 4 bits (for OCC enabled/disabled is OCC enabled) or 5 bits (for OCC enabled/disabled is OCC disabled).

-	Resource assignment – 3 bits as defined in clause 16.5.1.1 of [3]

-	Scheduling delay – 2 bits as defined in clause 16.5.1 of [3]

-	Modulation and coding scheme – 4 bits as defined in clause 16.5.1.2 of [3]. This field is not present if format N0 CRC is scrambled by PUR-RNTI, or if  and OCC enabled/disabled is OCC enabled. If npusch-16QAM-Config is configured and the value is '1111', it functions as 16QAM indicator.

-	Redundancy version – 1 bit as defined in clause 16.5.1.2 of [3]. If higher layer parameter npusch-OCC-Enabled is configured and Repetition number field has a value > 0 and , this field functions as OCC enabled/disabled, where value 0 indicates OCC disabled and value 1 indicates OCC enabled.

-	Repetition number – 3 bits as defined in clause 16.5.1.1 of [3]. If 16QAM is indicated, it functions as Modulation and coding scheme for 16QAM as defined in 16.5.1.2 of [3].

-	New data indicator – 1 bit. If multiple TB are scheduled, it functions as New data indicator for the first TB.

-	DCI subframe repetition number – 2 bits as defined in clause 16.6 in [3]

-	Number of scheduled TB for Unicast – 1 bit, where value 0 indicates a single TB is scheduled and value 1 indicates multiple TB are scheduled. This field is only present if higher layer parameter npusch-MultiTB-Config is enabled and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI as defined in [3]. The field is set to 0 if the CRC of the DCI is scrambled by SPS C-RNTI.

-	HARQ process number – 1 bit. This field is only present if 2 HARQ processes are configured and the corresponding DCI format is mapped onto the UE specific search space given by the C-RNTI as defined in [3], or if Number of scheduled TB for Unicast is present. If multiple TB are scheduled, it functions as New data indicator for the second TB.

-	Resource reservation – 1 bit as defined in clause 16.5 of [3]. This field is only present if higher layer parameter resourceReservationConfigUL is configured and the DCI is mapped onto the UE-specific search space given by C-RNTI as defined in [3].

If the number of information bits in format N0 mapped onto the UE specific search space given by the C-RNTI as defined in [3] is less than that of format N1 in the same search space, zeros shall be appended to format N0 until the payload size equals that of format N1.