3GPP TSG RAN Meeting #108					RP-251610

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.2

Source:	Vodafone

Title:	Considerations on 6G Working Group SID. Aspects related to: 
        - interworking with 4G;  
        - interim milestone for Energy Consumption model, and
        - need for early SA3 input on security for L2 /L1 signalling

Document for:	Agreement

# 1	Introduction

Owing to the agenda’s restriction to only have one document per company under agenda item 16.2, this document covers several diverse topics.

# 2	Interworking of 6G with 4G

## 2.1	Use cases

While other companies have mentioned other use cases (e.g. handover from 6G to a 4G-VoLTE coverage layer; and, direct migration from 5G-NSA to 6G), this document focuses on 2 use cases for handover from 6G to 4G:

- Handover from 6G Terrestrial Network to a satellite system supporting unmodified (c.f. Release 9) 4G devices with 4G core network.

  - Currently deploying commercial satellite systems support the use of unmodified 3GPP mobiles and present unmodified interfaces to a 3GPP core network.

  - “Digital Inclusivity” and the support for IoT means full coverage is needed for 4G.

  - Satellite systems are limited in capacity, hence the use of 4G-5G DSS on a satellite system is undesirable.

  - The good ambition of 3GPP to avoid 5G-6G Dual Connectivity implies that 6G must be able to be rolled out easily on a low frequency band.

  - With the use of a low band for 6G, it might be expected that 6G, 5G and 4G terrestrial coverage runs out at same point….

  - however, the ambition of building an IoT “kernel” into the day-1 6G specifications implies that 6G will support network based “coverage recovery” mechanisms for “6G-RedCap” devices with a single antenna. This would mean that 6G TN coverage would exceed that of 4G TN and 5G TN….

  - thus, at the edge of TN coverage, a 6G mobile would be on 6G and not on 4G or 5G.

  - Hence, at least for (emergency) voice calls, there is a need for 6G TN to 4G handover (with the 4G satellite system appearing to be a TN to the 6G system)

Note: handover from 4G-satellite to 6G TN is not essential. Instead 4G-satellite to 4G/5G terrestrial
          could be used and then followed by 5G to 6G handover.

- Network Energy Saving

- While some operators foresee the shutdown of their 4G network, other operators foresee that 4G IoT will be very long lived.

- The commencement of 4G eCall in only 2026 (for new car models) and 2027 (for existing car models) also indicates that 4G will be maintained in (at least) Europe for a long while.

- Future voice-only devices are likely to support 4G but not support 5G or 6G. These devices need to be able to make emergency calls (and with consistently good indoor coverage -> i.e. satellite coverage is not sufficient)

- In a multi-frequency cell site, the most complete way to save energy is to power down cells with light load.

- Currently, it does not look like 4G-6G DSS will be specified/supported.

- 4G-5G DSS consumes more base station power than 4G-only operation, and, significantly reduces network capacity.

- Thus, when the traffic load on a 4G/5G/6G site decreases, carriers can get switched off until just a 4G carrier remains.

- Adjacent to this lightly loaded site may be a more heavily loaded site that is still operating 6G.

- As a mobile moves from the 6G site into the energy saving site, a 6G (emergency) voice call will drop without handover from 6G to 4G.

Note: handover from a 4G-only Network Energy Saving site to 6G is not essential as the 6G site can be
          expected to also have 4G operational.

Proposal 1: the 6G RAN WG study should study how to support 6G TN to 4G TN handover. Support for 4G to 6G handover need not be considered in the first phase of 6G.

Independently, it is important that a chain of mobility events from 6G to 5G to 4G to 5G to 6G shall work without dropping a (emergency) voice call at the final 5G to 6G handover. However, this is primarily an issue for SA2 to solve and ought not to impact RAN directly.

# 3	Interim Milestones

In addition to proposals from other companies, the contributing company believes that the following issues are important to include in 3GPP’s project planning for 6G:

- Network Energy Saving

It seems to be broadly accepted that “energy consumption” should be considered during all aspects of the 6G Study. To do this, we need a base station energy consumption model that is valid for the 6G bands and technologies, and, this model needs to be available in-time to be used during the 6G study phase.

Hence there is a need for an updated Base Station Energy consumption model to be finished by mid-2026.

Proposal 2: an interim milestone is set at June-2026 for completion of a 6G base station energy consumption model.

- Radio interface User Plane protocol study

In order for this study to be completed successfully, early input from SA3 is needed on the security requirements for Layer 2 and Layer 1 signalling.

Proposal 3: TSG RAN should co-ordinate with TSG SA and SA3 in order for the security requirements for Layer 2 and Layer 1 radio interface signalling to be delivered by June 2026.

# 4	Proposals

In this contribution we propose:

Proposal 1: the 6G RAN WG study should study how to support 6G TN to 4G TN handover. Support for 4G to 6G handover need not be considered in the first phase of 6G.

Proposal 2: an interim milestone is set at June-2026 for completion of a 6G base station energy consumption model.

Proposal 3: TSG RAN should co-ordinate with TSG SA and SA3 in order for the security requirements for Layer 2 and Layer 1 radio interface signalling to be delivered by June 2026.