3GPP TSG RAN Meeting #108                                                        RP-251612

Prague, June 9 – 13, 2025

Source:	CEWiT

Title:	Views on the study of IMT-2030 requirements

Agenda Item:	16.1 – Study on 6G Scenarios and Requirements

Document for:	Discussion and Decision

Introduction

In RAN #107:

- Progress was made on the list and definitions of some of the TPRs, along with discussions on target values

- It was agreed to hold two conference calls (CC#1 and CC#2) to facilitate discussions between RAN #107 and RAN #108

In RAN #107, we provided our views in RP-250153, followed by further inputs in RT-250017 for CC#1. In this contribution, we reiterate our views on:

- Possible target values for legacy TPRs

- Definition and possible evaluation methodology for the Coverage TPR

Target Values for Legacy TPRs

Our views on the target values for the legacy TPRs are shown in the table below.

Table 1: Our views on target values for legacy TPRs

| ITU IMT-2030 TPR |  | IMT-2020 Value | Values proposed in this meeting | Moderator proposal | Our view |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | TBD | 22 Gbps for an aggregated bandwidth of 400 MHz and 45 Gbps for an aggregated bandwidth of 800 MHz. The proponent shall report the bandwidth used in the calculation. |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | [2x] of IMT-2020 | 1-1.5x compared to IMT-2020 |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | DL: TBD UL: Revisit the evaluation methodology before setting values Further discuss applicable deployment scenarios | For Immersive communication (IC): DL: 1-1.5x compared to IMT-2020 UL: 1.5-2x compared to IMT-2020 For Ubiquitous Connectivity (UC), DL: >=2 Mbit/s, UL: >=2 Mbit/s Note 2: In general, we agree with the potential issues of bandwidth scaling, particularly in the uplink. Having said that, we think that bandwidth scaling can be applied in scenarios where the bands being aggregated are in the same frequency range. |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | DL: 1.5x compared to IMT-2020 UL: 1.5-2x compared to IMT-2020 For Ubiquitous Connectivity DL: >0.1 bps/Hz UL: >0.1 bps/Hz |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | For IC: DL: 1.5x compared to IMT-2020 DL: 1.5-2x compared to IMT-2020 For UC DL: >=2 bps/Hz UL: >=1 bps/Hz |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [10] | 2-3x compared to IMT-2020 |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | eMBB: [1~4] HRLLC: [0.1~1] | TBD | IC: 2-4 ms HRLLC: 0.5-1 ms |
|  | Control plane latency (ms) | 20 | [5~20] | TBD | 10-20 ms |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 | 5-10x compared to IMT-2020 |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] | 1-10−5 to 1-10−6 |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 | Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Low Speed Vehicular: 10 km/h to [30-60] km/h Medium Speed Vehicular: [30-60] km/h to 120 km/h High speed vehicular: 120 km/h to 500 km/h, as a special case for high-speed train |
| Mobility interruption time (ms) |  | 0 | 0 ms | [0 ms] | 0 ms |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | 400MHz for system bandwidth (same definition as IMT-2020) | At least 200 MHz (As per our present understanding, Indian regulator is unlikely to give 400 MHz per operator in 7GHz band) |

Coverage TPR – Definition and Evaluation Steps

Our goal is increased and improved terrestrial coverage with larger cells. We envision terrestrial coverage to be an important component of Ubiquitous Connectivity, and it should evolve with every new generation. In IMT-2030, TN will be complemented by NTN wherever terrestrial coverage is missing. TN is expected to provide coverage to 95% of the population and with a higher user experienced data rate and cell edge data rate than NTN.

While NTN can be a big driver for serving under connected areas where there is not much habitation of people, we strongly believe that enhanced terrestrial coverage is still necessary to serve under connected areas which do have significant habitation of people (example: rural areas including villages and small towns such as those in the Indian subcontinent). NTN is not expected to handle the very large population density seen in rural environments of developing countries including India, and is also not expected to provide the above-mentioned user experienced data rate and cell edge data rate.

One type of deployment that we envision includes sparse deployment of large cells without contiguous multi-cell coverage. We believe that this type of deployment is to offer broadband services and is therefore best suited for a new test environment, which we call Rural-UC. The evaluation parameters should be revisited (compared to Rural-eMBB in IMT-2020) to include different UE heights and different UE power classes.

This test environment can be used to evaluate the coverage TPR, which we define as follows:

Figure 1: Rural-UC test environment covering multiple small villages with a large standalone cell.

Coverage refers to the ability to provide access to communication services for users in a desired service area. In the context of this capability of the RIT, coverage is defined as the cell edge distance of a single cell through link budget analysis.

This requirement is defined for the purpose of evaluation in the Ubiquitous Connectivity Usage Scenario.

This requirement is defined for the purpose of evaluation in the Rural-UC test environment.

| Test Environment | Cell Edge Distance |
| --- | --- |
| Rural-UC | 7 km for a single cell |

- Note: Cell edge distances beyond the minimum requirement of 7km, if supported, should be reported.

Evaluation methodology for the Coverage TPR:

- Run link level simulation for downlink and uplink data channels with the given configuration

- Evaluate the MCL for the downlink and uplink using link budget.

- Evaluate the corresponding distance for LOS and NLOS, for the associated channel model, for this test environment.

- Report whether this distance exceeds the minimum coverage requirement as specified in TPR.

Conclusion

Proposal 1: Consider the target values presented in column 5 of Table 1, towards the discussion on legacy TPR values.

Proposal 2: For peak data rate calculations, the RIT proponent shall report the bandwidth used in the calculation.

Proposal 3: For test environments related to Ubiquitous Connectivity, lower values compared to IMT-2020 (Rural-eMBB) should be considered for the following TPRs: User Experienced Data Rate, Avg Spectral Efficiency and 5th Percentile Spectral Efficiency.

Proposal 4: Bandwidth scaling can be applied in scenarios where the bands being aggregated are in the same frequency range.

Proposal 5: For the Ubiquitous Connectivity usage scenario, the study should consider coverage, user experienced data rate, 5th percentile and average spectral efficiency as TPRs. A new test environment should be defined for Ubiquitous Connectivity which includes at least the Rural geographical area, with an emphasis on large cells.

Proposal 6: Rural geographies should be included in a new test environment called Rural-UC for the evaluation of TPRs related to Ubiquitous Connectivity. The Rural-UC test environment should include the sparse deployment of large cells without contiguous multi-cell coverage, as shown in Figure 1.

Proposal 7: The evaluation parameters for the Rural-UC test environment should be revisited (compared to Rural-eMBB in IMT-2020) to include different UE heights and different UE power classes.

Proposal 8: The evaluation of coverage in Rural-UC can done with the following steps:

- Run link level simulation for downlink and uplink data channels with the given configuration.

- Evaluate the MCL for the downlink and uplink using link budget.

- Evaluate the corresponding distance for LOS and NLOS, for the associated channel model, for this test environment.

- Report whether this distance exceeds the minimum coverage requirement as specified in TPR.