3GPP TSG-RAN Meeting #108	RP-251614

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.2.2

Source:	Ericsson

Title:	Views on Rel-20 IoT NTN

Document for:	Discussion

# 1	Introduction

The conclusions from RAN#107 in Incheon for IoT NTN are:

Conclusions The following enhancements will be considered as baseline for IoT NTN: - Voice over NB-IoT-NTN via GEO - Study phase for 1-2Quarters to downselect between CP or UP by RAN#1xy (TBD) - Work on detailed objectives for RAN#108 ~max TU estimate for IoT NTN : RAN2 [1], RAN1 [0.5], RAN3 [no impact at this point]

Below we discuss the details and propose mode detailed objectives for IoT NTN for Rel-20.

# 2	Discussion

## 2.1	Scenarios

NB-IoT NTN is built on terrestrial NB-IoT, which has a reduced set of features compared to LTE or even LTE-M. For example, it lacks mobility support in RRC Connected. Rather than attempting to address these limitations, the work should be restricted to supporting stationary UEs communicating with GEO satellites, which we anyway expect to be the most interesting scenario business-wise.

Only stationary UEs communicating over GEO satellites is in scope.

## 2.2	Transfer of user plane data

For NB-IoT, the technical specifications support transfer of user plane data in UP and CP modes. As discussed in Incheon, a short study should be conducted to select between these two options. It is open if the study should be one or two quarters. Because coordination with SA groups is needed, we think two quarters are needed.

The study phase to evaluate if the UP or CP approach should be selected, there should be two quarters allowing coordination with SA2.

## 2.3	Access and connection procedures

RAN specs do not support real-time services such as voice and emergency services for terrestrial NB-IoT and, consequently, also not for IoT-NTN either (See clause 4.1 in LTE RRC). Supporting voice requires changes to initial access and RRC connection procedures in order to differentiate voice services which would be needed to handle overload situations.

For example, RRC connection establishment in NB-IoT does not include voice nor emergency as shown in Table II.

Table II: Comparison between the “establishment causes” for LTE and NB-IoT, the latter not including voice nor emergency [9].

| Establishment Cause for NB-IoT (Not including voice nor emergency) | Establishment Cause for LTE (Including voice and emergency) |
| --- | --- |
| -- ASN1START EstablishmentCause-NB-r13 ::= ENUMERATED { mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1} -- ASN1STOP | EstablishmentCause ::= ENUMERATED { emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1} |

Further, the concept of “acceptable cell” does not exist in NB-IoT due to lack of emergency services and that may need to be addressed.

RAN specifications currently exclude support for real-time services such as voice and emergency services for terrestrial NB-IoT and, consequently, for IoT-NTN.

Supporting voice in the RAN will require changes to the specifications in the access control, cell classifications and connection procedures.

## 2.4	PHY-layer aspects

Voice frames are typically delivered every 20 ms in each link direction (note that for reducing the protocol overhead several voice frames can be bundled e.g., 2 or 4 voice frames leading to deliveries every 40 or 80 ms), therefore rather than using dynamic scheduling for handling those speech frames, a semi-persistent scheduling (SPS) is expected to be a more suitable scheme to cope with such transmissions.

Overall, an SPS-based transmission does not require an NPDCCH to provide the characteristics of the transmission/reception of the data nor for pointing to the granted time-frequency resources, since instead the transmission/reception characteristics will be pre-established whereas the time-frequency resources will be granted in advance which will become available persistently with certain periodicity. To our knowledge, SPS is supported for LTE-MTC in coverage enhancement mode A (CE Mode A), but it is not supported for LTE-MTC in CE Mode B nor for NB-IoT.

For the support of voice-over-GEO, a semi-persistent scheduling (SPS) scheme is foreseen to be suitable to deal with voice frames which are typically delivered every 20 ms in each link direction (bundled voice frames for reducing the protocol overhead can lead to longer deliveries, e.g., every 40 or 80 ms).

Whereas the design details of the SPS scheme should be discussed in RAN1, it would be beneficial defining the scope and objectives in RAN Plenary which will facilitate the technical discussions. In our view, and as it was done for some Rel-19 objectives, there should be at least two meetings dedicated to study voice-over-GEO scenarios within a NB-IoT NTN context, and thereafter RAN1 can proceed to discuss the required solutions to specify the support of voice-over-GEO for NB-IoT NTN.

In our view, as part of the Rel-20 for the support of voice-over-GEO using NB-IoT NTN, there should be a study phase preceding the discussions on what to specify.

As part of the study phase, RAN1 should:

- Reach consensus on the simulation assumptions to evaluate a SPS-based scheme for voice-over-GEO scenarios in NB-IoT NTN.

- Perform an analysis including both voice frames (delivered every 20 ms in each link direction) and bundled voice frames (e.g., 2 or 4 bundled voice frames leading deliveries every 40 or 80 ms).

- Perform UL evaluations:

  - NPUSCH Format 1 with 15 kHz SCS using single-tone and/or multi-tone.

  - NPUSCH Format 1 with 3.75 kHz SCS using single-tone.

  - Assuming one or more than one Resource Unit (RU) depending on Transport Block size deemed to be suitable for transmitting a voice packet.

  - Assuming the use of repetitions (e.g., the number of required repetitions can be based on a link-budget analysis).

  - Achievable data rate.

- Perform DL evaluations:

  - NPDSCH with 15 kHz SCS.

  - Assuming one or more than one NPDSCH subframes depending on Transport Block size deemed to be suitable for transmitting a voice packet.

  - Assuming the use of repetitions (e.g., the number of required repetitions can be based on a link-budget analysis).

  - Assuming transmissions on an anchor carrier and non-anchor carrier.

  - Achievable data rate.

  - Any NPDCCH/NPDSCH scheduling required e.g., for IMS.

Then, based on the observations and conclusions reached during the study phase of this objective, RAN1 can proceed to discuss the design details and components of the SPS scheme intended to be used for the support of voice-over-GEO for NB-IoT NTN.

## 2.5	Draft WID

Based on the above we suggest a work item for voice support for IoT NTN with these objectives:

Study and specify necessary enhancements to support Voice over NB-IoT-NTN via GEO: - Until RAN#111, study and down select between UP or CP mode for supporting voice over IoT-NTN. Coordinate with SA2 as needed. [RAN2] - Modifications to the initial access and RRC connection procedures including how to handle overload situations [RAN2] - Support semi-persistent scheduling (SPS) for voice frames (e.g., 20 ms) and bundled voice frames (e.g., 40 ms, 80 ms). [RAN1, RAN2] - Simulation assumptions and evaluations: - NPUSCH Format 1 with 3.75 kHz and/or 15 kHz SCS using single-tone and/or multi-tone/NPDSCH with 15 kHz SCS. - Assuming one or more than one Resource Unit (RU)/NPDSCH subframes depending on Transport Block size deemed to be suitable for transmitting a voice packet. - Assuming the use of repetitions (e.g., the number of required repetitions can be based on a link-budget analysis). - Assuming transmissions on an anchor carrier and non-anchor carrier. - Any NPDCCH/NPDSCH scheduling required e.g., for IMS. - Note: Based on the feasibility identified from the evaluation, the options in the bullets above may be subject to down-selection. Note: Only stationary UEs communicating over GEO satellites is in scope meaning for example enhancement of mobility procedures is not in scope. TU allocation: RAN2 [0.5], RAN1 [0.5]

We judge that with these objectives we require 0.5 TUs in RAN1 and RAN2.

Approve a Rel-20 work item based on the above draft WID text.

# 3	Conclusion

Based on the discussion in the previous sections we made the following observations:

Observation 1	RAN specifications currently exclude support for real-time services such as voice and emergency services for terrestrial NB-IoT and, consequently, for IoT-NTN.

Observation 2	Supporting voice in the RAN will require changes to the specifications in the access control, cell classifications and connection procedures.

Observation 3	For the support of voice-over-GEO, a semi-persistent scheduling (SPS) scheme is foreseen to be suitable to deal with voice frames which are typically delivered every 20 ms in each link direction (bundled voice frames for reducing the protocol overhead can lead to longer deliveries, e.g., every 40 or 80 ms).

Observation 4	In our view, as part of the Rel-20 for the support of voice-over-GEO using NB-IoT NTN, there should be a study phase preceding the discussions on what to specify.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only stationary UEs communicating over GEO satellites is in scope.

Proposal 2	The study phase to evaluate if the UP or CP approach should be selected, there should be two quarters allowing coordination with SA2.

Proposal 3	Approve a Rel-20 work item based on the above draft WID text.