3GPP TSG RAN Meeting #108	                                            RP-251623

Prague, Czech Republic, June 9-13, 2025

Source:	CATT

Title:	Views on 5G-A Rel-20 NR NTN

Agenda Item:	15.1.5

Document for:	Discussion and Decision

Introduction

In RAN #107 plenary, there was extensive discussion for NR NTN R20 study. One important thing is to determine whether and how to make a study item on the GNSS resilient operation.

In this contribution, we provide our views on GNSS resilient operation in 5G-A Rel-20 NTN.

Discussion

Scenarios of GNSS resilient operation

In Rel-17 NTN, it is assumed that UE is able to acquire GNSS position information and make suitable time/frequency compensation. In real application, in most times, GNSS provides a highly accurate position and time reference. However, it is possible that GNSS jamming and spoofing will occur. Then, it may cause UE access denial or degradation of service in some time.

Another use case is that UE may not acquire accurate GNSS position information due to power saving or GNSS module not working simultaneously with communication model. In this case, the GNSS location information may have certain error, and will degrade the communication performance.

Hence, we think there are two levels of GNSS information relaxation:

- Case 1:  GNSS information exists some errors with less accuracy, but GNSS information is available

- Case 2:  GNSS information is temporarily not available in certain time period, and historical position information may have some errors

No matter which case occurs, the GNSS position that UE has acquired may or may not be able to support UE to access network without performance loss. In this sense, one important issue is to define the range of GNSS accuracy. Different range will have different impact to UE access procedure and synchronization maintenance after RRC connection.

Additional benefit for GNSS resilient operation is to alleviate the dependency to GNS position information. For example, sometime UE may reduce the power assumption if performing GNSS measurement with less frequency.

Proposal 1: When GNSS accuracy is relaxed, the study on detailed evaluation assumption should be included in Rel-20 NTN.

Potential technical impact

When the GNSS position information in UE side is not available, there are some impacts for UE operation:

- Issue 1:  UL synchronization is not correct and may cause connection failure or data transmission is not reliable

- Issue 2:  Network can’t get accurate UE location information, and it will cause beam steering or handover decision incorrect.

In general, the first issue is critical since UL synchronization may only tolerate very limited error. When UE sends PRACH signal, gNB will detect PRACH signal assuming a reasonable range of time or frequency shift. When the time error is out of the predefined range, detection performance is degraded significantly. It is same for frequency error. Therefore, necessary change for initial access procedure should be considered. When UE is being in RRC-Connected mode, tight synchronization of time and frequency should be kept in the whole time duration of RRC connection. If GNSS position is not accurate, it may impact the synchronization performance in RRC-Connected mode, and as a consequence, it will cause data transmission. Hence, necessary procedure enhancement to maintain synchronization in RRC-Connected mode should be studied and specified.

Proposal 2: Study on synchronization enhancement for initial access procedure and RRC-Connected mode is to be included in Rel-20 NTN.

Proposed working scope

Based on the agreement from RAN #107 plenary, it seems the impact to RAN1 for GNSS resilient operation is limited.

Conclusion for GNSS resilient operation Study Item with checkpoint to determine whether we have normative work and the scope based on results of study item phase. - Study should consider Rel-16 as baseline for discussion. Study should focus on NR NTN and be backward compatible with legacy UEs. Aim to minimize physical layer procedures impact to 5GA and avoid physical layer channel/signal changes. - Objective to be narrowed down for RAN#108 No additional NR NTN enhancements with RAN1 impact can be considered at this point given limited 5GA TUs

In this situation, the network assisted UE positioning may not be suitable for GNSS resilient operation due to that it will have more impacts to RAN1 channel and procedure.

Following the guidance of last meeting, RAN2 impact can be allowed. It means the network assisted information can be considered if it is justified to help UE to access the network.

Based on above discussion, the scope of GNSS operation can be limited and RAN1 channel structure is not expected to change. But the procedure and network assisted information can be modified. From section 2.2, the study on synchronization enhancement for initial access procedure and RRC-Connected mode is critical for GNSS resilient operation.

Another issue is how to define study phase and normal phase. In general, we hope in Rel-20 there is one normal phase for GNSS resilient operation. Then clear specification impact can be identified and specify necessary technical enhancement. Within this, Rel-20 NTN network can support UE to work in GNSS relaxed scenario.

Proposal 3: Rel-20 NR NTN working scope can be considered as follows:

  - Study evaluation assumption and technical impact for GNSS resilient operation

  - Study and specify (if needed) when UE is in GNSS resilient case:

    - Support necessary enhancement for PRACH transmission procedure

    - Support necessary enhancement to maintain time and frequency synchronization for RRC-Connected mode

  - Study and specify relative requirements in RAN4

Proposal 4: It is suggested that after 6-months study, RAN plenary will check study status and revise the scope of normal phase.

Conclusion

According to the discussions above, we provide a few proposals to elaborate potential enhancements for NR NTN in 5G-A Rel-20:

Proposal 1: When GNSS accuracy is relaxed, the study on detailed evaluation assumption should be included in Rel-20 NTN.

Proposal 2: Study on synchronization enhancement for initial access procedure and RRC-Connected mode is to be included in Rel-20 NTN.

Proposal 3: Rel-20 NR NTN working scope can be considered as follows:

  - Study evaluation assumption and technical impact for GNSS resilient operation

  - Study and specify (if needed) when UE is in GNSS resilient case:

    - Support necessary enhancement for PRACH transmission procedure

    - Support necessary enhancement to maintain time and frequency synchronization for RRC-Connected mode

  - Study and specify relative requirements in RAN4

Proposal 4: It is suggested that after 6-months study, RAN plenary will check study status and revise the scope of normal phase.