3GPP TSG RAN Meeting #108										                RP-251633

Prague, Czech Republic, 9th – 13th June, 2025

Agenda Item:	15.4.3

Source:	CATT

Title:	Views on Rel-20 5G-A Demod/RRM topics

Document for:	Discussion and decision

# Introduction

With the imminent arrival of the 6G era, Release 20 will encompass efforts for both the evolution of 5G-A and the study of 6G within the RAN WGs. In particular, RAN4 has been under heavy load for a long time, highlighting the critical need for careful planning in the Release 20 efforts.

In this contribution, we present our initial perspectives on the RAN4 RRM activities for this upcoming release.

# Discussion

As usual, RAN4 RRM is responsible for the following tasks in each release:

(1) RAN4-related work required by RAN1/2/3-led items,

(2) RAN4-led non-spectrum new items,

(3) Maintenance of previous releases.

Notably, maintenance work for previous releases, including Rel-15/16/17/18, continues to demand significant effort within RAN4, with more than 40% of the total contributions submitted in Q4 2024 dedicated to this [1]. With the potential additional Rel-19 maintenance, it is expected that a substantial amount of effort will still need to be reserved for maintenance tasks within the Rel-20 timeline.

Observation 1: In the Rel-20 timeline, a substantial amount of effort will still need to be reserved for maintenance tasks for releases from Rel-15 to Rel-19 and items led by other WGs.

Additionally, RAN4 needs to work on Rel-20 in two parallel tracks: 5G-A evolution and the 6G study. As a result, the release is expected to be smaller than Rel-19, requiring careful planning for the 5G-A evolution. In our opinion, RAN4's focus will be on addressing commercial demands and interests, technical issues identified in real-world scenarios, and making necessary corrections to the current specifications for both NTN and TN in Rel-20.

Observation 2: From RAN4 perspective Rel-20 5G-A would likely be a smaller release than Rel-19 with a focus on addressing commercial demands and interests, resolving technical issues identified in real-world scenarios, and making necessary corrections to the current specifications for both NTN and TN.

Based on the above observations, even though the companies are interested in many items, considering the limited time left for 5G-A in Rel-20, we suggest only considering 2 or 3 enhanced aspects. From our point of view, we propose to consider the following two items for RAN4-led non-spectrum new items with the justifications demonstrated respectively.

(1) NTN enhancement: mobility from E-UTRA TN to NR NTN in the RRC_CONNECTED state.

(2) RLM/BFD relaxation for RedCap UE.

## NTN RRM enhancement

Regarding NTN enhancement, it is noted that only the scenario of "E-UTRA TN to NR NTN" is lacking in existing RRM requirements for NTN, including requirements for IDLE/INACTIVE/CONNECTED mode. Actually, the requirements for CONNECTED mode in all existing "TN to NTN" and "NTN to TN" scenarios already introduced are also lacking, as shown in Fig. 1.

From the perspective of commercial development and demand, in order to enable seamless handover for "E-UTRA TN to NR NTN" in potential scenarios and improve perceived user experience, after further considering the time that Rel-20 will allocate for 5G-A as per the endorsed conclusion in [2], RAN4 may need to specify necessary RRM requirements at least for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode to enhance mobility performance.

Fig. 1, TN and NR NTN mobility enhancement

Proposal 1: For NTN enhancement, RAN4 need to specify necessary RRM requirements at least for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode to enhance mobility performance.

## TN RRM enhancement

On TN aspect, the following RRM enhancements were discussed in RAN#107 [3].

- RLM/BFD relaxation for RedCap UE - Consider to reuse Rel-17 solution as baseline - Consider both 1Rx and 2Rx. - Fast SCell activation [and PSCell addition] enhancement - Consider the reduction of the delay of measurement and/or procedure - FFS on the detailed solutions - Whether to have this item depends on the detailed proposals of solution - One unified solution is expected - Uplink only SCell (NES SSB-less SCell enhancement) - FR2 multi-Tx and multi-Rx - Enhanced RRM requirements for NES - Enhanced RedCap with LTM - R18 leftover on LTM RRM - Interruption at NR SRS antenna port switching - Parallel measurement - Inter-frequency measurement enhancement for LTM - On-demand deactivated SCell measurement - A-TRS based RRM enhancement - Dynamic RTD/TTD status update - AL/ML beam prediction testing - Handover interruption minimization - Interruption-free DCI operation - Extend OD-SSB to PSCell in NR-DC

According to the discussions in RAN#107, the first topic apparently attracted the most of interests among companies, while the opinions on the rest of topics were quite divergent. From the perspective of commercial development and demand, the cost of RedCap UEs is gradually decreasing and the amount is rising rapidly in an exponential trend. Therefore, it is necessary to specify solutions for RedCap UE to achieve the purpose of power saving, which can further lower the cost and prolong the lifetime of RedCap UEs in the deployment.

In addition, considering the relaxed RLM/BFD requirements for non-RedCap UEs have already been specified in Rel-17, controllable work efforts are required to complete the RLM/BFD relaxation requirements for RedCap UEs. In regard to the number of Rx antennas, the relaxed RLM/BFD requirements defined for non-RedCap UEs can be basically reused for 2Rx RedCap UEs. For 1 Rx RedCap UEs, not only the existing relaxation due to reduction of the number of Rx antennas but also the relaxations defined for non-RedCap UEs are needed.

Proposal 2: The enhancement of 1Rx/2Rx RedCap UEs with relaxed RLM/BFD requirements could be considered in Rel-20 for RRM-related tasks.

## TN demodulation enhancement

UE performance requirements for 2Rx/4Rx/8Rx SU-MIMO and MU-MIMO with MMSE-IRC receiver have been introduced, and UE performance requirements for 2Rx/4Rx SU-MIMO and MU-MIMO with R-ML receiver has been introduced. Since the 8Rx is widely used for CPE and FWA, so better performance 8Rx SU-MIMO with R-ML receiver compared to MMSE-IRC receiver are needed.

Proposal 3: Introduce the UE performance requirements for 8Rx SU-MIMO with R-ML receiver.

In addition, there is also one Rel-19 leftover with the introduction of 6Rx support, hence, performance requirements for 6Rx UE can also be considered.

Proposal 4: Introduce the UE performance requirements for 6Rx UE.

# Conclusions

In this contribution, we share our views on with the following observations Rel-20 RAN4 RRM works:

Observation 1: In the Rel-20 timeline, a substantial amount of effort will still need to be reserved for maintenance tasks for releases from Rel-15 to Rel-19 and items led by other WGs.

Observation 2: From RAN4 perspective Rel-20 5G-A would likely be a smaller release than Rel-19 with a focus on addressing commercial demands and interests, resolving technical issues identified in real-world scenarios, and making necessary corrections to the current specifications for both NTN and TN.

Proposal 1: For NTN enhancement, RAN4 need to specify necessary RRM requirements at least for mobility scenario of "E-UTRA TN to NR NTN" in CONNECTED mode to enhance mobility performance.

Proposal 2: The enhancement of 1Rx/2Rx RedCap UEs with relaxed RLM/BFD requirements could be considered in Rel-20 for RRM-related tasks.

Proposal 3: Introduce the UE performance requirements for 8Rx SU-MIMO with R-ML receiver.

Proposal 4: Introduce the UE performance requirements for 6Rx UE.

# References

RP-242424, “Status Report RAN WG4”, RAN4 Chair (Huawei).

RP-250812, Summary for RAN Release 20 5G-Adv

RP-250817, Moderator's summary for RAN4 led REL-20 topics, RAN4 Chair (Huawei).