3GPP TSG RAN Meeting #108		                                                      RP-251640

Prague, Czech Republic, June 9th-13th, 2025

Source:	CATT

Title:	Views on Rel-20 NR MIMO

Agenda Item:	15.1.2

Document for:	Discussion and Decision

- Introduction

In RAN #107 meeting, a large amount of proposals were provided on Rel-20 NR MIMO. The proposed objectives with good level of support are mainly in the area of DL CSI enhancement, UL enhancement and beam management enhancement [1]. Regarding the TU for Rel-20 NR MIMO, the majority support 1 TU for RAN1 and 0.5 TU for RAN2. In this contribution, we provide our views on the objectives for Rel-20 NR MIMO.

- Discussion

  - DL CSI enhancement

Early SRS/CSI/CSI-RS triggering

For a UE transits from IDLE/INACTIVE mode to CONNECTED mode, there is a delay for utilize MIMO since CSI measurement and report can only takes place after the UE receives an RRC configuration for CSI measurement and report. An early DL CSI acquisition is an effective way to enhance system efficiency since it can avoid performance loss caused by stale CSI. For the enhancement, both FDD and TDD should be considered. Therefore early triggering for both SRS and CSI-RS should be supported.

For the early DL CSI acquisition, the channel quality should be estimated before or in parallel with the RRC setup/ reconfiguration procedure, then MIMO can be used when the UE is ready to transmit or receive data. To have a reasonable workload for the enhancement, the scope of early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE mode to CONNECTED mode can be restricted to early triggering via MSG4 of 4-step RACH.

Proposal 1：On enhancing DL CSI acquisition, support specifying the following objective:

- Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH in FR1.

Sub-band scalar interference plus noise reporting

The motivation of sub-band scalar interference-plus-noise (IpN) measurement and report is to make MU-MIMO operation using channel reciprocity more efficient and simplify operation in TDD. However, for IpN estimation, the performance gain of scalar IpN reporting compared to CQI based solution is unclear, therefore more study is needed. Since there is only 1 TU in RAN1 for Rel-20 NR MIMO, the topics need study should not be considered in Rel-20.

Proposal 2：Do not support enhancement for sub-band scalar interference-plus-noise (IpN) measurement and report in Rel-20.

CSI-RS overhead reduction

In Rel-19, 48, 64, and 128 CSI-RS ports are aggregated by the legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), and the overhead of CSI-RS can be large. It is helpful to improve the spectrum efficiency if the overhead of CSI-RS is reduced. It was proposed to reduce the overhead of CSI-RS in frequency domain or spatial domain. For CSI-RS overhead reduction in frequency domain, the specification impact can be not large if a reasonable objective is adopted. The workload for CSI-RS overhead reduction in spatial domain can be quite large since varies patterns of subset configuration of CSI-RS ports should be discussed. Besides, advanced UE receiver is needed for CSI-RS overhead reduction in spatial domain. Therefore it should not be considered in Rel-20.

Proposal 3：Do not support enhancement for CSI-RS overhead reduction in spatial domain in Rel-20.

  - UL enhancement

SRS enhancement

For scenarios with only two symbols in S slot for TDD system, the accuracy of channel estimation from SRS cannot be ensured if the SRS is transmitted in the S slot and the channel condition is poor. To improve the coverage, the SRS transmission has to be delayed to the next UL slot, which increases time delay. To improve the SRS channel estimation in S slot, cross-slot SRS between one U slot and one adjacent S slot for SRS repetition can be considered.

In current system, RB-level partial sounding can be used to improve SRS capacity and coverage. Enabling interpolation for channel estimation would be benefit for further improving the SRS channel estimation accuracy for RB-level partial frequency sounding.

Proposal 4： On enhancing UL capacity and coverage, support specifying the following objectives:

- Cross-slot SRS between one U slot  and one adjacent S slot within a single SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots.

- Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding.

UL 2Tx codebook enhancement

Since the gain of introducing more precoders for 2Tx coherent codebook by utilizing the unused DCI codepoint for TPMI indication is unclear and the major current deployed UEs are non-coherent UEs, it is not urgent to enhance 2Tx full-coherent UL codebook with 8/16PSK alphabets in Rel-20.

Proposal 5：Do not support enhancement for 2Tx full-coherent UL codebook with 8/16PSK alphabets in Rel-20.

  - Beam management enhancement

UE-initiated/Event-driven beam switching

In Rel-19, UE-initiated/Event-driven beam reporting is supported. With the enhancement in Rel-19, the efficiency of beam management has been improved. UE-initiated/Event-driven beam switching is UE controlled beam management. Its performance is unclear. Besides, it would increase the complexity of gNB. In Rel-20, UE-initiated/Event-driven beam switching should be not supported.

Proposal 6：Do not support enhancement for UE-initiated/Event-driven beam switching in Rel-20.

- Conclusions

In this contribution, we provided our views on the objectives of Rel-20 NR MIMO. We have the following proposals:

DL CSI enhancement

Proposal 1：On enhancing DL CSI acquisition, support specifying the following objective:

- Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH in FR1.

Proposal 2：Do not support enhancement for sub-band scalar interference-plus-noise (IpN) measurement and report in Rel-20.

Proposal 3：Do not support enhancement for CSI-RS overhead reduction in spatial domain in Rel-20.

UL enhancement

Proposal 4： On enhancing UL capacity and coverage, support specifying the following objectives:

- Cross-slot SRS between one U slot  and one adjacent S slot within a single SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots.

- Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding.

Proposal 5：Do not support enhancement for 2Tx full-coherent UL codebook with 8/16PSK alphabets in Rel-20.

Beam management enhancement

Proposal 6：Do not support enhancement for UE-initiated/Event-driven beam switching in Rel-20.

- References

- RP-250802, Moderator's summary for RAN1 led REL-20 topics, RAN1 Chair (Samsung), RAN#107, Incheon, Korea, March 12th – 14th, 2025.