3GPP TSG-RAN Meeting #108                                              	RP-251642

Prague, Czech Republic, June 9-13, 2025

Source:	CATT, SK Telecom, China Unicom, Kyocera, LG Electronics Inc. China Telecom, LG Uplus, CMCC

Title:	Discussion on UAV/UAM enhancements in Rel-20 5GA

Agenda Item:	15.2.5

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss UAV/UAM enhancements for Rel-20 5GA based on the conclusions endorsed in RAN#107 as follows [1]:

Also, the detailed justifications and objectives for the potential Rel-20 WI for this topic are also provided.

Proposed Objectives for UAV/UAM enhancements in Rel-20

Specific Objectives for IDLE/INACTIVE mobility enhancements

From our perspective, the specific objectives on IDLE/INACTIVE mobility enhancements based on the above endorsed scope include two component parts, Frequency/cell prioritization and beam prioritization, which are discussed respectively as follows.

- Frequency/cell prioritization for IDLE/INACTIVE mobility

This part of the endorsed scope involves the impacts to reselection priority handling for measurements and/or cell reselection criterion during cell reselection procedure. For example, the UAV/UAM dedicated frequency may be considered as highest priority frequency for cell reselection measurements by a UE performing UAV/UAM service, or as another cell resection criterion, the UE may prioritize the cell(s) with specific deployment for UAV/UAM support (e.g. with up-tilted antenna).

- Beam prioritization for IDLE/INACTIVE mobility

In our understanding, this part of the endorsed scope corresponds to the support of altitude-based SSB in measurements for cell reselection. Specifically, the altitude-based SSB can be configured in system information (similar to what is already supported in measurement configuration in CONNECTED mode). Then, an IDLE/INACTIVE UE configured with UAV/UAM services may prioritize the measurements on the SSB(s) that correspond to its actual height during cell reselection. This can enable the UE to find the cell(s) suitable for the support of UAV/UAM services more efficiently.

Based on the above discussions, the specific objectives for IDLE/INACTIVE mobility enhancements for UAV/UAM in Rel-20 are proposed as follows:

Proposal 1: Specify UAV/UAM specific IDLE/INACTIVE mobility enhancement: [RAN2]

  - Specify the mechanism prioritizing frequency(ies)/cell(s) with enhanced UAV/UAM based service support for cell reselection.

  - Specify altitude-based SSB for UE measurements during IDLE/INACTIVE mobility.

Specific Objectives for CHO enhancements

Whether to support UAV/UAM specific CHO enhancements was left as FFS in the last RAN plenary. During the discussion, there were interests from some operators in this regards. Considering the endorsed TU budget, i.e. a 2-quarter work item, if CHO enhancement is pursued, only the support of height based CHO trigger event should be within the scope, i.e. extending the height based events which are already specified for measurements (i.e. Hx and AxHy) to CHO. This can enable the characteristics of UAV/UAM mobility to be sufficiently considered during handover procedure, envisioning performance gain for the support of UAV/UAM mobility.

Proposal 2: Specify the height-based CHO trigger events for UAV/UAM services (reusing measurements events Hx and/or AxHy already specified in TS 38.331). [RAN2]

Work item management

Whether to have a separate WI or TEI to handle UAV/UAM is to be decided. Considering the above objectives, potential work load and specifications to be impacted (e.g. TS 38.300/306/304/331), UAV/UAM enhancements in Rel-20 5GA is worth a (at least) two-quarter discussions in RAN2, instead of simply a TEI item. Therefore, it is proposed to approve a separate UAV/UAM enhancement work item in Rel-20, including the above proposed objectives.

Proposal 3: Approve a separate NR work item for UAV/UAM enhancements in Rel-20.

Proposed "Justification" and "bullet lists of objectives"

For the UAV/UAM enhancement WID, the TP to "Justification" part and "bullet lists of objectives" (which are based on above proposals) are provided in the Annex.  It is proposed that RAN plenary endorses the TP in the Annex.

Proposal 4: RAN plenary endorses the "Justification" and "bullet lists of objectives" in the Annex for the Rel-20 UAV/UAM enhancement WID.

Conclusion

This contribution discusses the potential Rel-20 work item for UAV/UAM enhancement. Observations and proposals are listed as follows:

On proposed objectives for UAV/UAM enhancements in Rel-20

Proposal 1: Specify UAV/UAM specific IDLE/INACTIVE mobility enhancement: [RAN2]

  - Specify the mechanism prioritizing frequency(ies)/cell(s) with enhanced UAV/UAM based service support for cell reselection.

  - Specify altitude-based SSB for UE measurements during IDLE/INACTIVE mobility.

Proposal 2: Specify the height-based CHO trigger events for UAV/UAM services (reusing measurements events Hx and/or AxHy already specified in TS 38.331). [RAN2]

On work item management

Proposal 3: Approve a separate NR work item for UAV/UAM enhancements in Rel-20.

On proposed "Justification" and "bullet lists of objectives"

Proposal 4: RAN plenary endorses the "Justification" and "bullet lists of objectives" in the Annex for the Rel-20 UAV/UAM enhancement WID.

Reference

- RP-250812	Summary for RAN Release 20 5G-Adv

- Annex: TP for UAV/UAM enhancement WID in Rel-20

# 3 Justification Increasing interests in uncrewed aerial vehicles (UAV) based services have been witnessed globally in recent years: - In some regions, there are great interests from governments and operators to build a “Low-altitude” network to support “Low-altitude” economy by UAV communications, with strong industrial radiations (e.g. logistics, transportation, agriculture, meteorology, etc.) and expectation of trillion-level industrial and market scale. - Great expansion and growth of commercial drone/UAV markets are witnessed in a variety of regions throughout the globe. In Rel-18, UAV specific enhancements were introduced in NR, aiming at more effective support of UAV based services in NR RAN but still with some limitations on mobility aspects. Specifically, there were no consideration on UAV/UAM specific factors in those advanced mobility mechanisms in NR (e.g. CHO, LTM, etc.) and no consideration on IDLE/INACTIVE mobility enhancements. These make the current 5G NR probably insufficient/ineffective to support the mobility requirements of UAV/UAM communication thus far. To this end, further enhancements in NR to support UAV/UAM based service are needed in 5G-A Rel-20, with the main objectives to support further UAV/UAM-specific mobility enhancements. # 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The objectives of the work item are the following: - Specify UAV/UAM specific IDLE/INACTIVE mobility enhancement: [RAN2] - Specify mechanisms prioritizing frequency(ies)/cell(s) with enhanced UAV/UAM based service support for cell (re)selection. - Specify altitude-based SSB for UE measurements during IDLE/INACTIVE mobility. - Specify height based CHO trigger events to support UAV/UAM based communication [RAN2] - Note: For this objective, the solutions shall be reusing the existing height based measurement events to the maximum extent. ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. ### 4.3 RAN time budget request (not applicable to RAN5 WIs/SIs) NOTE: For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings. The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI. One time unit (TU) corresponds to ~ 2 hours in the meeting. If no TU is needed, then leave the field empty otherwise enter a number >0 in the field. For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID. If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table. See the time budget request in the attached Excel table.