3GPP TSG RAN Meeting #108								   RP-251656

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.3.2

Source:	InterDigital, Inc.

Title:	Scope for ISAC for Rel-20 5G-Advanced

Document for:	Discussion/Decision

# Introduction

In RAN#107, the potential scope of the ISAC for Rel-20 5G-Adv was discussed, and the following guidance was provided for further discussion [1]:

the following were discussed and expected to be further checked in RAN#108:

- The discussion on sensing/ISAC in 5GA and 6G are independent.

- A set of possible objectives of Sensing/ISAC in Rel-20 are listed below:

  - Study on gNB mono-static mode and possibly bi-static mode for UAV use case. [RAN3, RAN1]

  - Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1]

  - Performance evaluation for gNB sensing. [RAN1]

  - Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected.

  - Note: Coordination with SA2 when needed

In this contribution, we discuss the potential scope for Rel-20 ISAC for NR in terms of objectives to be included based on market interests as well as allocated TU.

# Discussion

ISAC channel models will be specified in Rel-19 [2], paving the way for performance evaluation of ISAC use cases and deployment scenarios in 5G-Adv and 6G. The scope of the corresponding studies should be defined based on market interests as well as allocated TU, and importantly in such a way to to avoid overlapping and imposing unnecessary constraints for future additions.

Observation 1: The scope of potential studies on Sensing/ISAC for 5G-Adv and 6G in Rel-20 require careful consideration in view of the market interests and well as allocated TU.

The scope of the Rel-20 ISAC for NR should be defined such to have a reasonable workload in RAN1 based on the TU allocation.

Proposal 1: Minimize RAN1 workload for 5G-Adv Sensing/ISAC study in Rel-20 in view of the allocated TU.

The primary use case motivating incorporating Sensing/ISAC in 5G-A is ISAC-UAV, i.e., UAV detection, tracking, and navigation – intended to facilitate the demands for low-altitude economy through NR. Meanwhile, Sensing/ISAC in 6G can explore broader use cases.

The UAV-ISAC use case in 5G-Adv can be supported through gNB-only sensing. In particular, gNB-monostatic can be targeted, which requires reasonable workload with minimal specification impact. The additional consideration of gNB-bistatic significantly widens the scope of the study and the specification impact, and in turn increases the workload in RAN1.

Moreover, the underlying sensing methods and algorithms for UAV-ISAC use case can be left to implementation and up to companies to report.

Observation 2: The ISAC-UAV use case can be selected for performance evaluation in 5G-Adv, without including ISAC-Automative, ISAC-Human, and ISAC-AGV.

Observation 3: gNB-monostatic can be targeted to support UAV-ISAC use case in 5G-Adv, whereas the additional study of gNB-bistatic significantly widens the scope of the study and consequently workload in RAN1.

Observation 4: The sensing methods and algorithms for ISAC-UAV use case are left to implementation.

Proposal 2: The study of Sensing/ISAC in 5G-Adv should be limited to gNB-monostatic for supporting ISAC-UAV use case and deployment scenarios.

In order to avoid specification impact and RAN1 workload, 3GPP should retain the existing NR radio interface for the Sensing/ISAC study for 5G-Adv. In particular, reusing of existing RS such as CSI-RS and PRS can be considered for performance evaluation. This approach in turn avoids overlapping with potential study of waveform/RS design for 6G Sensing/ISAC.

Observation 5: Study of gNB-monostatic sensing/ISAC in 5G-Adv can be based on existing NR radio interface and reusing of the existing RS such as CSI-RS and PRS.

Proposal 3: Retain existing NR radio interface and RS for the 5G-Adv Sensing/ISAC study in Rel-20.

The scope of the Sensing/ISAC in Rel-20 should be focused on the procedures and signalling support between RAN and CN. To enable this study, different measurement types can be studied including possible reuse of existing measurements such as positioning, defining new measurement types, or leaving measurement type down to implementation.

Observation 6: The sensing measurement to support 5G-Adv Sensing/ISAC in Rel-20 study should be identified.

Proposal 4: RAN1 should identify the NR measurement types needed for gNB-monostatic sensing/ISAC in Rel-20 study.

The respective work in RAN3 on defining the procedures and signalling between RAN and CN can then be carried out based on sensing measurement type. At the same time, in order to avoid the overlap with 6G Sensing/ISAC study in Rel-20, defining a new network function for sensing should be avoided.

Observation 7: The procedures and signaling between RAN and CN can be studied to enable gNB-monostatic sensing measurements and reporting in 5G-Adv.

Proposal 5: Study signaling support between RAN and CN and corresponding sensing measurement framework to enable gNB-monostatic sensing in 5G-Adv without specifying a new network function.

# Summary

In this contribution, the following observations and proposals were made:

Observation 1: The scope of potential studies on Sensing/ISAC for 5G-Adv and 6G in Rel-20 require careful consideration in view of the market interests and well as allocated TU.

Observation 2: The ISAC-UAV use case can be selected for performance evaluation in 5G-Adv, without including ISAC-Automative, ISAC-Human, and ISAC-AGV.

Observation 3: gNB-monostatic can be targeted to support UAV-ISAC use case in 5G-Adv, whereas the additional study of gNB-bistatic significantly widens the scope of the study and consequently workload in RAN1.

Observation 4: The sensing methods and algorithms for ISAC-UAV use case are left to implementation.

Observation 5: Study of gNB-monostatic sensing/ISAC in 5G-Adv can be based on existing NR radio interface and reusing of the existing RS such as CSI-RS and PRS.

Observation 6: The sensing measurement to support 5G-Adv Sensing/ISAC in Rel-20 study should be identified.

Observation 7: The procedures and signaling between RAN and CN can be studied to enable gNB-monostatic sensing measurements and reporting in 5G-Adv.

Proposal 1: Minimize RAN1 workload for 5G-Adv Sensing/ISAC study in Rel-20 in view of the allocated TU.

Proposal 2: The study of Sensing/ISAC in 5G-Adv should be limited to gNB-monostatic for supporting ISAC-UAV use case and deployment scenarios.

Proposal 3: Retain existing NR radio interface and RS for the 5G-Adv Sensing/ISAC study in Rel-20.

Proposal 4: RAN1 should identify the NR measurement types needed for gNB-monostatic sensing/ISAC in Rel-20 study.

Proposal 5: Study signaling support between RAN and CN and corresponding sensing measurement framework to enable gNB-monostatic sensing in 5G-Adv without specifying a new network function.

# References

RP-250040, “Summary for RAN Rel-20 5G-Adv”, RAN Vice Chairs, RAN WG Chairs

3GPP TD RP-242348: " Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR ".