3GPP TSG RAN Meeting #108                                                              RP-251681

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.3.1

Source:	Tejas Networks

Title:	Views on scope of AI/ML for NG-RAN ph3

Document for:	Discussion

# Introduction

The objectives of AI/ML for NG-RAN in Rel-20 are listed below:

-	Study the following new AI/ML based use cases, i.e., QoE Optimization and energy saving (including finer granularity of energy saving), and the corresponding solutions within the existing architecture. [RAN3]

-	Continue to work on the following leftovers [RAN3]:

-  Multiple-hop target node UE trajectory

Note: Other Leftovers depend on Rel-19 outcome, if any.

Note: Coordination based on LSs with other working groups, when needed

In RAN#107, the following areas were downselected [1]:

# 2   Scope of AI/ML for NG RAN
For the new topics under consideration for Rel-20, we should prioritize areas which are of commercial interest and considering this, energy saving and 'multiple-hop target node UE trajectory' topic has strong interest from operators. Additionally, the energy saving and UE trajectory was also explored in previous releases and can be completed within the proposed timeline,

# Proposal 1: RAN3 to prioritize energy saving and multiple hop target node UE trajectory for Rel 20.

2.1 Scope of AI/ML for Network Energy Saving in Rel-20

In the scope of the Rel-18 Work Item “AI/ML for NG-RAN” [2], RAN3 specified enhancements for data collection and signaling support within the existing NG-RAN architecture and interfaces. Energy saving was one of these use cases. The work item defined Energy cost as a metric to consider for Network Energy saving use case and Node level EC parameter was introduced as part of Xn/F1 interface.

The main open issues during Rel-18 WID was the inability to establish a clear definition for 'additional load’ for requesting the predicted Energy Cost.  As a result, discussions related to predicted or estimated Energy Cost were halted.

Proposal 2: RAN3 to study “additional load” or other metrics during request for “predicted Energy Cost”

Release 18 introduced node-level EC but during the release 19 study it became evident that such a coarse granularity cannot adequately capture the impact of an offloading action on the Energy Cost of a target node. The following EC granularity were discussed:

- EC per group of Cells based on energy consumption measurements associated to the hardware serving the group of cells

- EC per Cell based on energy consumption estimations

- EC per one or more HO event based on energy consumption estimations

- EC per gNB-DU based on energy consumption measurements associated to the hardware serving the gNB-DU

However, the discussions did not lead to a consensus on selecting a specific option.

Observation 1: RAN3 could not conclude on AI/ML for energy savings WI due to not having consensus on finer granularity of the EC metrics.

In our view, it is nearly impossible to obtain Energy Cost (EC) measurements at the per-cell or per-group-of-cells level, as cells often share hardware resources in complex ways. However, measuring EC based on one or more handover (HO) events, or at the gNB-DU level, could be considered a finer granularity for EC measurements.

Proposal 3a: For AI/ML for Network energy saving, RAN3 to study issue of EC granularity level.

Proposal 3b: RAN3 to prioritize the following granularity levels. EC measurements per one or more handover (HO) events, and EC per gNB-DU. Note: Other granularity levels are not precluded

Additionally, several new features related to network energy saving were introduced in Rel-18 and Rel-19, such as OD-SIB1, cell-level DTX/DRX, OD-SSB etc. We propose exploring how these features can be optimally deployed using AI/ML-based decision-making to further enhance energy efficiency.

Proposal 4: We propose exploring how other NES features like OD-SIB1, cell-level DTX/DRX, OD-SSB etc can be optimally deployed using AI/ML-based decision-making to further enhance energy efficiency.

# 3. Conclusion

Following are our proposals:

Proposal 1: RAN3 to prioritize energy saving and multiple hop target node UE trajectory for Rel 20.

Proposal 2: RAN3 to study “additional load” or other metrics during request for “predicted Energy Cost”

Observation 1: RAN3 could not conclude on AI/ML for energy savings WI due to not having consensus on finer granularity of the EC metrics.

Proposal 3a: For AI/ML for Network energy saving, RAN3 to study issue of EC granularity level.

Proposal 3b: RAN3 to prioritize the following granularity levels. EC measurements per one or more handover (HO) events, and EC per gNB-DU. Note: Other granularity levels are not precluded

Proposal 4: We propose exploring how other NES features like OD-SIB1, cell-level DTX/DRX, OD-SSB etc can be optimally deployed using AI/ML-based decision-making to further enhance energy efficiency.

# 4. Reference

[1] RP- 250787 [report for Rel-20] 'Moderator's summary for RAN3 led REL-20 topics' by RAN3 Chair (ZTE)

[2] RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102