3GPP TSG-RAN Meeting #108									       RP-251682

Prague, CZ, 9 - 13 June, 2025

Agenda Item:		16.1

Source:		Deutsche Telekom, Vodafone, Orange, Telecom Italia, Telefonica, BT, Bouygues Telecom, Spark NZ, Odido, Turkcell, Telenor, Telus, CKH IOD UK, Jio Platforms, Reliance Jio, KPN, Boost Mobile Network

Title:		Key operator requirements for 6G

Document for:		Discussion and decision

## 1	Introduction

The RAN P SI on “Study on 6G scenarios and requirements” has been approved in December 2024 with a first phase focusing on IMT-2030 related topics especially for the interaction with ITU-R WP 5G [1].

At the RAN#107 meeting in March 2025 the SI has been amended to also cover “6G requirements and deployment scenarios” in a second phase of the SI until June 2026 [2].

With this contribution the cosigning operators list their key requirements for 6G.

## 2	Key requirements for 6G

The following key requirements should be used as the baseline for future RAN P and RAN WG work in Rel-20 and beyond and supported from day 1 unless stated otherwise.

General:

- 6G RAT & RAN shall be based on open standardised interfaces

- 6G RAT & RAN shall be energy efficient

- 6G RAT & RAN shall support UN sustainable development goals

- 6G RAT & RAN shall support automatic configuration and optimisation to the maximum extent possible

- 6G RAT & RAN shall make use of AI where justified in terms of gains and energy efficiency versus classic non-AI based solutions

- 6G RAT & RAN shall provide enhanced security, integrity and privacy, e.g. quantum-safe

- 6G RAT & RAN features shall target to deliver a Minimum Viable Product in a single release

  - No enhancements over multiple releases, unless commercially proven

Potential new 6G RAT:

- 6G RAT shall not require Radio Access Network HW refresh or expansion in existing bands without significant additional benefits being demonstrated in the SI phase

- 6G RAT shall provide significant gains in terms of spectrum efficiency and energy efficiency against 5G-Advanced in existing bands

  - The gains shall be assessed in the RAN WG SI based on the NGMN RPAF [3]

- 6G RAT shall support a streamlined protocol for each layer and procedure

- 6G RAT shall support a smooth migration path from NR connected to 5GCN

- 6G RAT shall support a “standalone” mode without DC with NR

- 6G RAT shall support Multi-RAT Spectrum Sharing (“MRSS”), including Carrier Aggregation of MRSS carriers, on new and existing Radio Access Network HW with minimal radio overhead and without the need for additional baseband resource

- 6G RAT shall support a non-standalone mode (DC between 6G RAT and NR) with 6G RAT as the Primary Cell (“6NDC”)

  - NOTE: The final decision if this mode (single solution) is standardised shall be based on the 6G RAN WG and SA WG SIs results

- 6G RAT shall support a non-standalone mode (DC between NR and 6G RAT) with NR as the Primary Cell (“N6DC”)

  - NOTE: The final decision if this mode (single solution) is standardised shall be based on the 6G RAN WG and SA WG SIs results

- 6G RAT shall support radio level aggregation using intra-RAT Carrier Aggregation, this includes non-collocated deployments

- 6G RAT shall support radio level aggregation using intra-RAT Dual Connectivity, this includes non-collocated deployments

- 6G RAT shall support Network Sharing (i.e. MOCN, MORAN and its enhancements)

- 6G RAT shall support improvements for the Uplink concerning data rate performance (especially at cell edge) and coverage

- 6G RAT shall support enhancements of MIMO technology for the scenario of passive antennas with up to 8T8R

  - Note: commercially other lower configurations might be relevant depending on the deployment scenario

- 6G RAT shall support massive MIMO with active antennas with up to at least 1024 antenna elements and up to at least 256T/256R

  - Note: commercially other lower configurations might be relevant depending on the deployment scenario

- 6G RAT shall support improvements of massive MIMO technology for both TDD and FDD

  - 6G RAT shall implement efficient SU-MIMO and MU-MIMO CSI acquisition techniques including low and high resolution CSI codebook techniques and channel reciprocity based CSI acquisition (SRS based) techniques from day 1

- 6G RAT shall support distributed MIMO with multiple TRPs at different locations

- 6G RAT shall support a simple framework for user and traffic flow differentiation that improves user experience given network resource constraints, while allowing flexibility, service and user customization, and service exposure (e.g. with APIs)

- 6G RAT shall support native IMS Voice, incl. emergency calls

- 6G RAT shall be designed to support 6G-NTN with LEO, MEO, GEO constellations with “hooks” considered in the design from day-1

  - Support of enhanced 6G-NTN interworking with 5G/6G-TN and NR-NTN as well as 6G-TN interworking with NR-NTN is FFS for potentially a later Phase of 6G

- 6G RAT shall work in FR1 spectrum between 410 and 7125 MHz

- 6G RAT shall work in FR2 spectrum between 24.25 and 71 GHz

- 6G RAT shall work in FR3 spectrum between 7.125 and 24.25 GHz

- 6G RAT shall allow easy, flexible and ideally automated (specification / testing) support of various bands, band configurations and band combinations in a non-constrained manner

- 6G RAT shall support multiple duplexing options including FDD, TDD, SDL incl. Half-Duplex (i.e. for IoT)

- 6G RAT shall support decoupling of the uplink and the downlink

- 6G RAT shall be designed to take into account the bursty nature of the traffic and the large amount of small data transmissions on the air interface

- 6G RAT shall support optimised RAN and CN mobility management and paging, and fast state transition to connected mode

- 6G RAT shall support UE-based mobility in IDLE (incl. redirection by the network)

- 6G RAT shall support fast network-based mobility in CONNECTED mode with small interruption time and a minimum number of options

- 6G RAT shall support IDLE and CONNECTED mode mobility to and from 5G NR (TN & NTN)

  - There is no need to support radio level mobility (i.e. cell reselection or handover) 
to 3G or 2G – interworking can be done via PLMN/Cell Selection

  - Radio level inter-working (i.e. cell reselection or handover) with 4G is FFS

  - Note: “Mobility chaining” from 6G to 5G to 4G and back from 4G to 5G to 6G shall be supported by the respective CN procedures to be studied in SA2

- 6G RAT shall define a limited set of diverse UE classes

- 6G UE shall support a device design of 3 MHz bandwidth for initial access

- 6G UEs shall meet the following minimum requirements:

  - bandwidth of at least >= 20 MHz in bands 600/700/800/900, incl. operation in all channel bandwidths

  - bandwidth of at least >= 40 MHz in bands 1500/1800/2100/2600, incl. operation in all channel bandwidths

  - bandwidth of at least >= 200 MHz in bands 3.x, incl. operation in all channel bandwidths

  - bandwidth of at least >= 200 MHz in bands u6GHz, incl. operation in all channel bandwidths

- 6G UEs shall support min. bandwidths as defined above and meet the following requirements:

    - 4 DL MIMO layers / 4 Rx for bands > 1 GHz

    - 2 Tx / 2 UL MIMO layers for bands > 1 GHz

- 6G RAT shall support a future proof design (“hooks”) for introduction of IoT devices with min. 3 MHz CBW in a later release

Potential new 6G RAN:

- 6G RAN architecture shall generally keep the 5G functional split (e.g. Authentication, Charging, etc in the CN, while Handover decisions, etc remain in the RAN), while optimising functions which have (partly) a duplication in today’s architecture (e.g. paging)

- 6G RAN architecture shall not require a Radio Access Network HW refresh

- 6G RAN architecture shall be software based and optimised, wherever possible

- 6G RAN architecture shall allow cloud implementation, allowing independent selection of HW and SW for digital elements

- 6G RAN architecture part of the 3GPP SI shall study protocols optimised for cloud native implementations

- 6G RAN architecture shall support standardised, open interfaces for multi-vendor interoperability

  - An open Lower Layer Split (LLS) between the RU and the DU shall be supported and standardised outside 3GPP

  - An open Xn-like interface between the 6GNBs shall be supported

  - As part of the 3GPP SI the need for open F1-like interface between the DU and the CU shall be studied

  - As part of the 3GPP SI the need for open E1-like interface between the CU-CP and the CU-UP shall be studied

  - An open N2-like interface between the 6G RAN and the CN for 6G shall be supported

  - An open N3-like interface between the 6G RAN and the CN for 6G shall be supported

  - An open multi-vendor interface from 6G RAN to the Service Management & Orchestration (“SMO”) like entity shall be supported

  - The 6G RAN shall support Open Application Programming Interfaces (“APIs”)

- 6G RAN shall support connectivity through multiple transmission points (multiple TRPs), either collocated or non-collocated

- 6G RAN shall support network sharing of the RAN between multiple operators

- 6G RAN shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services

- 6G RAN shall support automatic configuration and optimisation to the extent possible

- 6G RAN shall make use of AI, where justified in terms of performance gains and energy efficiency versus classic non-AI based solutions

## 3	Proposal

It is proposed by the cosigning operators to agree on related text for the above to be captured in the new TR for the RAN P SI on 6G during this meeting.

## References

[1] 	RP-243327 “Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#107

[2] 	RP-250810	“Revised SID: Study on 6G Scenarios and requirements”,	 CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#108

[3]	NGMN RPAF Paper https://www.ngmn.org/publications/ngmn-radio-performance-assessment-framework.html