3GPP TSG-RAN Meeting #108									       RP-251683

Prague, CZ, 9 - 13 June, 2025

Agenda Item:		16.1

Source:	Deutsche Telekom, Vodafone, Orange, Telecom Italia, Spark NZ, Odido, Turkcell, Telenor, Telus, Jio Platforms

Title:			Operator suggestions for functionalities not needed for 6G

Document for:		Discussion and decision

## 1	Introduction

The RAN P SI on “Study on 6G scenarios and requirements” has been approved in December 2024 with a first phase focusing on IMT-2030 related topics especially for the interaction with ITU-R WP 5G [1].

At the RAN#107 meeting in March 2025 the SI has been amended to also cover “6G requirements and deployment scenarios” in a second phase of the SI until June 2026 [2].

With this contribution the cosigning operators suggest which functionality from legacy generations and newly proposed R & D topics should not be considered for 6G.

## 2	Guidelines on what is not needed for 6G

NOTE: this section intends to provide guidelines on which features shall not be studied in RAN and the RAN WGs during the 6G Studies

- 6G shall not support multiple options to achieve the same functionality

- 6G shall be a simplified system that does not carry forward all the features specified for previous generations which have not seen widespread commercial use. e.g.

  - operation in unlicenced spectrum (5G can be used and was optimised for this)

  - (e)MBMS (4G/5G can be used and was optimised for this, not widely adopted by the market)

  - RAN-level aggregation with non-3GPP technologies (LAA, LWA, LWIP 4G, ..)

  - Sidelink services (5G can be used and was optimised for this)

  - Integrated Access & Backhaul (“IAB”), relay or mobile relay

- As part of the 3GPP SI a simplified system is targeted copying a subset of functionality from previous generation(s), where well justified. e.g.

  - Industrial IoT

  - multiple network-based positioning technologies

  - Non-Public-Networks/NPN

  - Closed Subscriber Group “CSG”/Home-NB

- 6G shall be a simplified system that does not need to support:

  - intra-3GPP CN-level aggregation by means of Dual-Stack

  - spectrum > 71 GHz (including sub-THz bands)

  - Visual Light Communication (“VLC”)

## 3	Proposal

It is proposed by the cosigning operators to agree on related text for the above to be captured in the new TR for the RAN P SI on 6G during this meeting.

## References

[1] 	RP-243327 “Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#107

[2] 	RP-250810	“Revised SID: Study on 6G Scenarios and requirements”,	 CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#108