3GPP TSG-RAN Meeting #108									       RP-251684

Prague, CZ, 9 - 13 June, 2025

Agenda Item:		16.1

Source:		Deutsche Telekom, Vodafone, Orange, Telecom Italia, Telefonica, Spark NZ, Odido, Turkcell, Telstra, Telenor, Telus, Jio Platforms, KPN, Boost Mobile Network

Title:			Operator deployment scenarios for 6G

Document for:		Discussion and decision

## 1	Introduction

The RAN P SI on “Study on 6G scenarios and requirements” has been approved in December 2024 with a first phase focusing on IMT-2030 related topics especially for the interaction with ITU-R WP 5G [1].

At the RAN#107 meeting in March 2025 the SI has been amended to also cover “6G requirements and deployment scenarios” in a second phase of the SI until June 2026 [2].

With this contribution the cosigning operators list their deployment scenarios for 6G, focusing first on macro scenarios. Other scenarios, e.g. indoor, will be addressed at a later stage.

## 2	General deployment scenarios for 6G

The following provides a list of deployment scenarios the co-signing operators propose to focus on and optimise 6G for:

- 6G shall be optimised for terrestrial (TN) deployment

  - TN deployment type includes

    - Outdoor: rural, sub-urban, urban and dense urban deployment, inc. deployment for outdoor-to-indoor coverage

    - Indoor: separate indoor installation of RAN equipment, advanced Distributed Antenna Systems (“aDAS”)

- 6G shall support Fixed-Wireless Access (“FWA”)

- 6G shall support non-terrestrial deployment (drones and satellites)

  - The design and especially the efficiency of TN 6G shall not be constrained by NTN design, if any specific

- 6G shall support HAPS deployments

  - The design and especially the efficiency of TN 6G shall not be constrained by HAPS design, if any specific

- 6G shall support deployment for high-speed trains up to 500 kmph for FR1

- 6G shall support deployment for low altitude < 3000 m) aviation up to 1000 kmph (i.e. this includes drones and light aircraft) for FR1

- 6G shall support deployment for high altitude (3000 to 12000 m) commercial aviation up to 1000 kmph for FR1

### 2.1	Deployment details

The aim of the cosigning operators is to reuse the existing 5G locations for initial 6G deployment and keep todays Inter-Site-Distance (ISD) for providing coverage due to the unavailability of new / additional macro sites especially in urban environments and the high cost of deploying a new small cell layer. For rural coverage few new sites might be possible, but a general densification is not anticipated. Further the review of the scenarios from 38.913 also proposes changes to deployment parameters reflecting todays typical deployment situation with a 10 to 15 years outlook taken into consideration. A longer-term densification is not precluded as a means of performance improvement, but this requires an economic case that is currently missing.

<THE FOLLOWING IS BASED ON 3GPP TR 38.913 FOR 5G AND ADAPTED TO 6G WHERE APPRORIATE>

### 2.1.1	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high-speed vehicles, outdoor pedestrians and indoor users. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 2.1.1-1.

Table 2.1.1-1: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz Around 700 MHz and Around 2 GHz combined |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: 2x30 MHz NOTE3 Around 2 GHz: 2x60 MHz NOTE3 |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m (optional, for comparison with IMT-2020/5G ISD 2: 5000m (baseline) |
| BS configuration NOTE4 | Around 700 MHz: 4T4R with passive antenna Around 2 GHz: 4T4R with passive antenna |
| UE configuration | Around 700 MHz: 1T2R Around 2 GHz: 2T4R |
| User distribution and UE speed | 15% outdoor vehicles (120km/h), 15% outdoor (3 km/h) and 70% indoor (3 km/h), 10/2 users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450MHz – 960MHz identified for WRC-15 are currently being considered and around 700MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

### 2.1.2	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 2.1.2-1.

Table 2.1.2-1: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz and Around 2 GHz and Around 4 GHz and Around 7 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: 2x30 MHz NOTE3 Around 2 GHz: 2x60 MHz NOTE3 Around 4 GHz: 100 MHz Around 7 GHz: 200 MHz |
| Layout | Single layer: - Hex. Grid |
| ISD | 500m |
| BS configuration | Around 700 MHz: 4T4R Around 2 GHz: 4T4R with passive (baseline) and 32T32R with up to 128 active antenna elements (optional) Around 4 GHz: 32T32R with up to 192 active antenna elements Around 7 GHz: up to 256T256R with up to 1024 active antenna elements |
| UE configuration | Around 700 MHz: 1T2R Around 2 GHz: 2T4R Around 4 GHz: 2T4R Around 7 GHz: up to 4T8R |
| User distribution and UE speed | 10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10/2 users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes.

NOTE4:	10 users per TRxP is the baseline with full buffer traffic. 20 users per TRxP with full buffer traffic is not precluded. For evaluation of UL user experienced data rate  the number of users per TRxP can be reduced to 2.

### 2.1.3	(new) Sub-Urban macro

The sub-urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in sub-urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 2.1.3-1.

Table 2.1.3-1: Attributes for sub-urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz and Around 2 GHz Around 700 MHz and Around 2 GHz and Around 4 GHz and Around 7 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: 2x30 MHz NOTE3 Around 2 GHz: 2x60 MHz NOTE3 Around 4 GHz: 100 MHz Around 7 GHz: 200 MHz |
| Layout | Single layer: - Hex. Grid |
| ISD | 1000m |
| BS configuration | Around 700 MHz: 4T4R antenna Around 2 GHz: 4T4R (for all 4 bands) or 32T32R with up to 128 active antenna elements (for only the “around 700 MHz and around 2 GHz” scenario) Around 4 GHz: 32T32R with up to 192 active antenna elements Around 7 GHz: Up to 256T256R with up to 1024 active antenna elements |
| UE configuration | Around 700 MHz: 1T2R Around 2 GHz: 2T4R antenna Around 4 GHz: 2T4R antenna Around 7 GHz: Up to 4T8R antenna |
| User distribution and UE speed | 10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 50km/h, 80% Indoor in houses: 3km/h 10/2 users per TRxP NOTE4 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 1427 – 2690MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of ...

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For UL User Experienced Data rate it is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes.

NOTE4:	10 users per TRxP is the baseline with full buffer traffic. 20 users per TRxP with full buffer traffic is not precluded. For evaluation of UL user experienced data rate  the number of users per TRxP can be reduced to 2.

## 3	Proposal

It is proposed by the cosigning operators to agree on related text for the above to be captured in the new TR for the RAN P SI on 6G during this meeting.

## References

[1] 	RP-243327 “Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#107

[2] 	RP-250810	“Revised SID: Study on 6G Scenarios and requirements”,	 CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, RAN#108

[3]	NGMN RPAF Paper: https://www.ngmn.org/publications/ngmn-radio-performance-assessment-framework.html