3GPP TSG-RAN WG2 Meeting #130                                                                R2-2504832

St. Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0879 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Inter-RAT Cell Selection for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2025-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inter-RAT cell selection for NB-IoT was discussed and introduced in Rel-16 NB-IoT enhancement topic. Corresponding descriptions of the feature can be found in TS36.300 and TS36.331. #### – SystemInformationBlockType27 The IE SystemInformationBlockType27 contains information relevant only for inter-RAT cell selection i.e. assistance information about NB-IoT frequencies for cell selection.. #### – SystemInformationBlockType27-NB The IE SystemInformationBlockType27-NB contains information relevant only for inter-RAT cell selection i.e. assistance information about E-UTRA frequencies and/ or GERAN frequencies for cell selection.. ### 10.2.6 Idle mode Inter-RAT Cell Selection to/from NB-IoT NB-IoT may provide assistance information for inter-RAT cell selection to E-UTRAN/GERAN and E-UTRAN may provide assistance information for inter-RAT cell selection to NB-IoT. A UE may use the assistance information provided by the network for cell selection to/from NB-IoT.. However, TS36.304 (the general description of functionality for NB-IoT in IDLE mode) says that the Inter-RAT cell selection is not applicable for NB-IoT. There is an inconsistency between TS36.300/TS36.331 and TS36.304. |
| Summary of change: |  | In 4.4 NB-IoT functionality in Idle Mode Clarify that the inter-RAT cell selection is applicable to NB-IoT. Impact analysis Inter-operability: 1. If the network supports the change and the UE does not, there is no inter-operability issue foreseen. 2. If the UE supports the change and the network does not, there is no inter-operability issue foreseen. Impacted functionality: Cell selection for NB-IoT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is an inconsistency between TS36.300/TS36.331 and TS36.304, which makes it ambiguous whether the inter-RAT cell selection for NB-IoT is supported or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 4.4	NB-IoT functionality in Idle Mode

This specification is applicable to NB-IoT, except for the following functionality which is not applicable to NB-IoT:

-	Acceptable cell

-	Accessibility measurements

-	Access Control based on ACDC categories

-	Camped on Any cell state

-	CSG, including support for manual CSG selection and CSG or Hybrid cell related functionality in PLMN selection, or HNB name (SIB9), Cell selection and Cell reselection.

-	Emergency call

-	E-UTRAN Inter-frequency Redistribution procedure

-	Inter-RAT Cell Reselection including measurements in other RATs

-	Logged measurements

-	Mobility History Information

-	Mobility states of a UE

-	Priority based reselection

-	Public warning system including CMAS, ETWS, PWS.

-	RAN-assisted WLAN interworking

-	RRC_INACTIVE state

-	Sidelink operation

End of change