3GPP TSG-RAN WG2 Meeting # 130	R2-2504928

St Julian’s, Malta, May. 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2076 | rev | 1 | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on power ramping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Samsung, MediaTek Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2025-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to section 7.1.1 of TS 38.213 as quote below, the transmission power of Msg3 PUSCH is based on the “total power ramp-up”. According the MAC specification as quoted below, the UE can switch from 2-step RACH to 4-step RACH, and “POWER_OFFSET_2STEP_RA” added in section 5.1.1a and 5.1.3 is used to compensate the power ramping difference between 2-step RACH and 4-step RACH, as the value of “PREAMBLE_POWER_RAMPING_STEP” (i.e. powerRampingStep) for 4-step RACH can be different from “MSGA_PREAMBLE_POWER_RAMPING_STEP” (i.e. msgA-PreamblePowerRampingStep) for 2-step RACH. The total power ramping amount for the 4-step RACH is calculated in 5.1.3 as follows: (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA However, in section 5.1.4 of TS 38.321, the example formula used for indicating the power ramping value to PHY was not changed since Rel-15, which does not include POWER_OFFSET_2STEP_RA. This causes the misalignment between TS 38.213 and TS 38.321. TS 38.213: ## 7.1 Physical uplink shared channel … 7.1.1 UE behaviour … - If the UE receives a random access response message in response to a PRACH transmission or a MsgA transmission on active UL BWP b of carrier f of serving cell c as described in clause 8 - fb,f,c0,l=∆Prampup,b,f,c+δmsg2,b,f,c, where l=0 and - δmsg2,b,f,c is a TPC command value indicated in a random access response grant of the random access response message corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant of the random access response message corresponding to a MsgA transmission according to Type-2 random access procedure with RAR message(s) for fallbackRAR, on active UL BWP b of carrier f of serving cell c, and - and ∆Prampup_requested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers [11, TS 38.321] for carrier f in the serving cell c, MRB,b,f,cPUSCH(0) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP b of carrier f of serving cell c, and ∆TF,b,f,c0 is the power adjustment of first PUSCH transmission on active UL BWP b of carrier f of serving cell c. TS 38.321: ### 5.1.1a Initialization of variables specific to Random Access type … 2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure: 3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). ### 5.1.3 Random Access Preamble transmission … 1> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA; |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.1.4, it is clarified that the amount of power ramping applied to the latest Random Access Preamble transmission includes POWER_OFFSET_2STEP_RA. Impact analysis Impacted 5G architecture options: Standalone, NR-DC, NE-DC, and (NG)EN-DC Impacted functionality: 2-step RACH Inter-operability: If the network implements the CR and the UE does not, there is no interoperability issue. If the UE implements the CR and the network does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There could be some misunderstanding that when the UE switches from 2-step RACH to 4-step RACH, the power ramping of 4-step RACH does not include POWER_OFFSET_2STEP_RA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission on the search space indicated by recoverySearchSpaceId is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes a MAC subPDU with RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see clause 5.2);

5>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA);

5>	if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	if the Random Access procedure was initiated for SpCell beam failure recovery and spCell-BFR-CBRA with value true is configured:

7>	indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

NOTE:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of contention-based Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined.

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request:

5>	consider the Random Access procedure unsuccessfully completed.

3>	else if the Random Access Preamble is transmitted on an SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in clause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2);

3>	else if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

4>	delay the subsequent Random Access transmission until the Random Access Procedure is triggered by a PDCCH order with the same ra-PreambleIndex, ra-ssb-OccasionMaskIndex, and UL/SUL indicator TS 38.212 [9].

3>	else:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2) after the backoff time.

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response reception.

END OF CHANGE