3GPP TSG-RAN WG2 Meeting #130	R2-2504438

# Malta, MT, May 19th – May 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2091 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TX carrier (re-)selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The carrier selection procedure (5.22.1.11) for the IUC Information Request MAC CE transmission below reuses the operation of 5.22.1.1's NOTE 2A. - 5.22.1.11 TX carrier (re-) selection: “The MAC entity shall select the carrier where the IUC resource set(s) are request as the carrier for transmission of a Sidelink Inter-UE Coordination Information Request MAC CE.” - 5.22.1.1’s NOTE 2A: For the transmission of Sidelink Inter-UE Coordination Request MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is required. For the transmission of Sidelink Inter-UE Coordination Information MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is located. Therefore, "request" in the sentence should be changed to "required". 2. Sidelink Inter-UE Coordination Information Request MAC CE is incorrect term. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. In section 5.22.1.11, “request” wording was modified to “required”. 2. In section 5.22.1.11, “Sidelink Inter-UE Coordination Information Request MAC CE” was modified to “Sidelink Inter-UE Coordination Request MAC CE”. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: NR sidelink CA Interoperability: For change 1, if the network implements this CR but not the UE, there is no interoperability issue. For change 1 if the UE implements this CR but not the network, there is no interoperability issue. For change 1 if one UE implements this CR but not the other UE, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. There is an incorrectly described text in the sentence of carrier selection procedure for MAC CE transmission in the specification. 2. There is incorrect term for Sidelink Inter-UE Coordination Request MAC CE in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.22.1.11	TX carrier (re-)selection

The MAC entity shall consider a CBR of a carrier to be one measured by lower layers according to TS 38.215 [24] if CBR measurement results are available, or the corresponding sl-DefaultTxConfigIndex configured by upper layers if CBR measurement results are not available.

If the TX carrier (re-)selection is triggered for a Sidelink process according to clause 5.22.1.1, 5.22.1.2 or 5.22.1.3.3, the MAC entity shall:

1>	if there is no selected sidelink grant on any carrier allowed for the sidelink logical channel where data is available as indicated by upper layers (TS 38.331 [5] and TS 23.287 [19]):

2>	for each carrier configured by upper layers associated with the concerned sidelink logical channel:

3>	if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel:

NOTE 1:	In the case of multiple resource pools configured on a carrier, which specific resource pool is used to determine the CBR of this carrier is up to UE implementation, taking into account of sl-HARQ-FeedbackEnabled for the sidelink logical channel.

4>	consider the carrier as a candidate carrier for TX carrier (re-)selection for the concerned sidelink logical channel.

1>	else:

2>	for each sidelink logical channel, if any, where data is available and allowed on the carrier(s) for which Tx carrier (re-)selection is triggered according to clause 5.22.1.1, if the CBR of the carrier(s) is below sl-threshCBR-FreqKeeping associated with priority of the sidelink logical channel:

3>	select the carrier(s) and the associated pool of resources.

2>	else:

3>	for each carrier configured by upper layers on which the sidelink logical channel is allowed, if the CBR of the carrier is below sl-threshCBR-FreqReselection associated with the priority of the sidelink logical channel:

4>	consider the carrier as a candidate carrier for TX carrier (re-)selection, for each carrier configured by upper layers on which the sidelink logical channel is allowed.

NOTE 1a:	For the carriers configured in SIB12 and for which SL-PRS transmission is allowed, the UE selects one carrier for SL-PRS among the selected carriers per above procedure, and which one the UE selects is up to UE implementation.

The MAC entity shall select the carrier on which the SL-CSI Request was received as the carrier for transmission of a Sidelink CSI Reporting MAC CE. The MAC entity shall for the Sidelink CSI Reporting MAC CE select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

The MAC entity shall select the carrier on which a Sidelink Inter-UE Coordination Request was received as the carrier for transmission of a Sidelink Inter-UE Coordination Information MAC CE.

The MAC entity shall select the carrier on which IUC resource set(s) are located as the carrier for transmission of a Sidelink Inter-UE Coordination Information MAC CE triggered by condition.

The MAC entity shall select the carrier where the IUC resource set(s) are required as the carrier for transmission of a Sidelink Inter-UE Coordination Request MAC CE.

The MAC entity shall for the Sidelink Inter-UE Coordination Information MAC CE or Sidelink Inter-UE Coordination Request MAC CE select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

The MAC entity shall perform the TX carrier (re-)selection such as TX carrier (re-)selection for the logical channel when TX carrier (re-)selection for a Sidelink Inter-UE Coordination Request MAC CE, and/or a Sidelink DRX Command MAC CE is triggered.

The MAC entity shall:

1>	if one or more carriers are considered as the candidate carriers for TX carrier (re-)selection:

2>	if Tx carrier (re-)selection is triggered, for each sidelink logical channel allowed on the carrier where data is available:

3>	select one or more carrier(s) among the candidate carriers with increasing order of CBR from the lowest CBR, and select the associated pool(s) of resources:

4>	if sl-HARQ-FeedbackEnabled is set to enabled for the sidelink logical channel:

5>	select one pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

4>	else:

5>	select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or Dedicated SL-PRS resource pool, if configured.

2>	else if Tx carrier (re-)selection is triggered, for a Sidelink DRX Command MAC CE or a Sidelink Inter-UE Coordination Request MAC CE:

3>	select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X, if configured or SL-PRS dedicated resource pool, if configured.

NOTE 2:	It is left to UE implementation how many carriers to select based on UE capability.

NOTE 3:	It is left to UE implementation to determine the sidelink logical channels among the sidelink logical channels where data is available and that are allowed on the carrier for which Tx carrier (re-) selection is triggered.

NOTE 4:	It is left to UE implementation to determine whether the resource pool for CBR measurement is reused as the resource pool for SL grant creation.

NOTE 5:	For the transmission of Sidelink Inter-UE Coordination Request MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is required. For the transmission of Sidelink Inter-UE Coordination Information MAC CE, the MAC entity selects the TX pool of resource where the IUC resource set is located.

NOTE 6:	If RRC indicates the MAC layer to use the legacy carrier in Sidelink CA PDCP duplication for backwards-compatibility, it is up to UE/Network implementation to ensure legacy carrier is used.

END of CHANGE