3GPP TSG RAN Meeting #108					        RP-251704

Prague, Czech Republic, June 9-13, 2025

Source:	AT&T, BT, Ericsson, Spark NZ, Telstra, T-Mobile USA

Title:	A New Testing and Certification Paradigm for 6G

Agenda Item:	16.1

Document for: 	Discussion/Decision

# 1. Introduction

At the TSG RAN #107 meeting in Incheon, Korea a revised SID was approved to study 6G scenarios and requirements [1]. One of the objectives is to “ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting” [1].

Over 20 companies across the ecosystem and geographic regions, spanning mobile network operators, infrastructure and chipset vendors additionally submitted their position on “Lean and Streamlined 6G Standards” to TSG RAN #107 demanding the following [2]:

During the course of 5G and 5G-Advanced standardization we have observed an ever-increasing trend of excessive 3GPP specifications, e.g., adopting multiple options to the same functionality. While there may be various factors contributing to the above, it is clear that the excessive specifications in 3GPP not only make the standards unnecessarily complex to define and maintain, it also affects, crucially, commercialization, causing high implementation complexity, slow time-to-market, poor feature adoption and uptake as well as cumbersome interoperability. As 6G standardization is now starting, the 3GPP community has a great opportunity to reverse this trend and strongly commit to producing lean and streamlined standards. The contributing companies propose to endorse an overall principle in 3GPP to create lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified. We propose that 3GPP, through its Individual Members, TSGs and WGs shall adhere to this principle in their daily work, in accordance with 3GPP Working Procedures while ensuring that this principle is not misused e.g. to stall the work progress.

Lastly, the same theme was captured as key 6G standardization principle in [3], supported by over ten companies across the ecosystem and geographic regions, again spanning mobile network operators, infrastructure and chipset vendors alike [3]:

Ensure a globally aligned meaningful minimum set of capabilities and normatively specified testing and certification procedure for 6G RAT.

In this contribution, we are providing additional key aspects of this 6G priority of great importance that we believe are of fundamental relevance to many other 6G priorities, such as a software-based 6G with needs-based hardware refresh [4] and a modular and scalable 6G design [5]. We also provide a text proposal for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements” [6] for 3GPP to agree the proposed procedures.

# 2. Discussion

The justification of the 6G study item on scenarios and requirements states that “the lessons learnt from the development, deployment, and operation of 5G networks around the world [include] system complexity and large number of migration options” [1]. Reference [2], supported by over 20 companies, equally observes that “during the course of 5G and 5G-Advanced standardization we have observed an ever-increasing trend of excessive 3GPP specifications, e.g., adopting multiple options to the same functionality” and that “excessive specifications in 3GPP not only make the standards unnecessarily complex to define and maintain, it also affects, crucially, commercialization, causing high implementation complexity, slow time-to-market, poor feature adoption and uptake as well as cumbersome interoperability.” They come to the conclusion that “the 3GPP community has a great opportunity to reverse this trend and strongly commit to producing lean and streamlined standards.”

AT&T fully and strongly supports this postulate. However, we believe there is an additional, complementary aspect beyond “too many options”, namely, the need for a “globally aligned meaningful minimum set of capabilities and normatively specified testing and certification procedure for 6G RAT” [3]. We believe these are two different manifestations of the same development in 5G and 5G-A that is proving to becoming detrimental to 3GPP, as an SDO, from a project and time management perspective, but even more importantly, to the success of 3GPP technologies in the marketplace and in deployments worldwide. In fact, the 6G SID [1] perfectly captures both these two artifacts of the same, or related problem in 5G/5G-A:

- Minimize the adoption of multiple options for the same functionality

- Avoid excessive configurations, excessive UE capabilities, and UE capabilities reporting

While both are included in the SID, our concern is that too much priority is given to the former one, which, in reality, is just a “way-we-work” improvement internally to 3GPP. Of course, adoption of multiple options for the same functionality presents a clear and present danger for fragmentation in the marketplace and in deployments. But only, if the multiple options all get deployment. If neither option ever gets implemented or deployed, minimizing the adoption of multiple options for the same functionality is not observable outside 3GPP.

That doesn’t mean, one should not strive, in fact, one should avoid such thing, but in the end, it amounts to a 3GPP internal improvement of time and project management. While important, for reasons beyond efficiency, to the long-term viability of 3GPP, we feel the second aspect is of much greater importance to actual deployments and the success of 6G: avoidance of excessive configurations, excessive UE capabilities, and UE capabilities reporting . It is thus crucial for 6G to define a globally aligned meaningful minimum set of capabilities and normatively specified testing and certification procedure for 6G RAT [3].

For 6G, for each work item, basic feature groups should be defined to ensure implementation, certification, and deployment of a large number of features as components of a single feature group, rather than specifying a myriad of individual feature groups for a small number, and often a single component. As one prominent example, Release 18 alone defines 165 NR UE features for MIMO enhancements and Release 19 has already agreed over 80 additional ones.

A large number of optional feature groups comprising individual, elementary components is often agreed and specified by 3GPP to allow the UE to signal a given feature has been IODT’ed. This is a consequence of 3GPP only mandating user equipment to implement mandatory features. MBSFN subframes are a formidable example for how continuous innovation and continuous disruption is stifled today through the myriads of features and an ecosystem where even mandatory features cannot be relied on. Case in point: LTE-NR coexistence had to rely on inefficient, in fact undesired CRS rate matching for 4G/5G MRSS because in practical deployments MBSFN subframes could not be relied on. MBSFN subframes were a mandatory, “native” 4G feature whose whole purpose was forward compatibility! By the same token, whilst the first release of 5G exhibited a tremendous emphasis of forward compatibility—forward compatibility may very well be the most often mentioned phrase in the outcome of the 3GPP 5G NR study item—it is doubtful how many of these “native” but optional features were ultimately deployed to guarantee any degree of forward compatibility.

Hence, a globally aligned and normatively specified testing and certification procedure is equally crucial. Features should not just be declared as mandatory, but deployment of these (and other features) should be tracked in 3GPP with a binding certification procedure for both the network and the user equipment as detailed in Figure 1.

Figure 1: New paradigm for testing and certification

# 3. Text Proposal

The following text is proposed for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements” [6].

# 5.x Testing and Certification Paradigm for 6G

A globally aligned and normatively specified testing and certification procedure should ensure features are not just declared as mandatory, but deployment of these (and other features) is tracked in 3GPP with a binding certification procedure for both the network and the user equipment.

For each 3GPP 6G work item, basic feature groups should be defined to ensure implementation, certification, and deployment of large numbers of features as components of a single feature group, rather than specifying a myriad of individual feature groups for a small number, and often a single component. These basic feature groups together define a meaningful, minimum set of features that are globally aligned among operators.

# 4. Conclusion

In this document, we provided our views and vision for a new testing and certification paradigm in 6G. A text proposal for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements” is proposed for 3GPP to agree said new testing and certification procedure.

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC/Verizon/NTT DOCOMO, INC./Deutsche Telekom

- RP-250766, Lean and Streamlined 6G Standards, AT&T/Apple/BT/CMCC/DT/Ericsson/Huawei/Intel/KT/MediaTek/Nokia/NTT Docomo/Qualcomm/Reliance Jio/Samsung/Spark NZ/T-Mobile USA/Telstra/Verizon/Vodafone/ZTE

- 6GWS-250234, 6G Standardization – Principles, T-Mobile USA Inc./AT&T/BT/Ericsson/KT/MediaTek/Nokia/NTT DOCOMO/Qualcomm/Rakuten/Softbank/Telstra

- RP-251702, Software-based 6G with Needs-based Hardware Refresh, AT&T

- RP-251703, On 3GPP Support of LPWA in 6G, AT&T/Ericsson/FirstNet/KT Corp./Rakuten/Spark NZ/T-Mobile USA/Verizon

- 3GPP TR 38.914, Study on 6G Scenarios and requirements, Release 20 (draft)