3GPP TSG RAN Meeting #108					                  RP-251706

Prague, Czech Republic, June 9-13, 2025

Source:	AT&T

Title:	AT&T Views on New RAN WG SI for 6G

Agenda Item:	16.2

Document for: 	Discussion/Decision

# Introduction

During the TSG RAN #108 meeting, 3GPP is expected to agree a new RAN WG SI for 6G. In this contribution, we provide our views on the drafting and scope of a working group level study for 6G.

# Discussion

## Backward Compatibility

Our detailed views can be found in our companion contribution in [1] and are summarized as follows:

3GPP TSG RAN working groups should discuss how to meaningfully differentiate between software vs. hardware to assess the impact of non-backward compatible changes such that 6G deployment does not require complete overhaul of hardware to realize benefits. 3GPP should facilitate software-driven 6G deployment with existing open radios in legacy bands leveraging efficient 5G-6G MRSS, where and when needed. 3GPP RAN working groups should study the full capability of 6G when radios are refreshed in existing or new greenfield bands as necessitated by actual needs. Specifically, the UE shall always assume a base station deploys new 6G hardware. A UE shall be completely oblivious as to whether a given base station uses “capable 5G RAN infrastructure” or a new 6G radio. After network attach, through RRC (re)configuration, a vast number of novel waveform designs, modulations, and coding schemes could be considered, as long as they can coexist with the OFDM time-frequency grid as specified in 5G NR. By guaranteeing that the UE always assumes a base station deploys new hardware the pitfalls similar to 5G NSA are avoided where a UE was aware of whether 5G was rolled out as NSA or SA. This requirement is thus key to ensuring the principle of a single branch of 6G standards development. The vision of a software-based 6G rollout with needs-based hardware refresh is consequently perfectly congruent with the desire for 3GPP RAN working groups to study the full capability of 6G when radios are refreshed (waveform design, modulation, coding schemes …). Software-based 6G with needs-based hardware refresh is not a conservative approach to 6G. It is the very key to unleashing innovation through new radios and reflects operators’ desire of doing things fundamentally differently in 6G, by not requiring a complete, nationwide overhaul of hardware to realize the benefits of a new G.

## System Simplification

Our detailed views can be found in our companion contribution in [2] and are summarized as follows:

A globally aligned and normatively specified testing and certification procedure should ensure features are not just declared as mandatory, but deployment of these (and other features) is tracked in 3GPP with a binding certification procedure for both the network and the user equipment. For each 3GPP 6G work item, basic feature groups should be defined to ensure implementation, certification, and deployment of large numbers of features as components of a single feature group, rather than specifying a myriad of individual feature groups for a small number, and often a single component. These basic feature groups together define a meaningful, minimum set of features that are globally aligned among operators.

## Practical User Experience

Our detailed views can be found in our companion contribution in [3].

## Supported Frequency Ranges

6G shall support FR1, FR2-1, and the frequency range in between (“FR3”). If the new 6G Radio does not support operation in unlicensed spectrum, support beyond 52.6 GHz (viz., FR2-2) is not needed.

## Stand-alone Operation, 5G-6G Multi-RAT Spectrum Sharing (MRSS), and Migration from 5G NR to 6GR

3GPP shall standardize one standalone architecture with 6G-6G carrier aggregation and 5G-6G Multi-RAT Spectrum Sharing (MRSS) for migration. Additional migration architecture options from 5G NR to 6G should be studied, but at most one additional architecture, if any, should be specified. The study should consider UE impact, RAN impact, and overall specification impact. Mobility between 5G NR and 6G is supported.

## Network Energy Savings and UE Power Savings

Network energy savings (NES) and UE power savings (PS) should not be treated as two separate, and at times competing objectives as in 5G. The 6G Radio shall maximize NES and PS using one common design and both NES and PS should be prioritized from the onset, unlike 4G and 5G where NES followed PS.

## Coverage

6G Radio design should focus on cell-edge performance and UL coverage. Re-use of the existing 5G mid-band (~3.5GHz) site grid for 6G deployments at least around 7-8 GHz with comparable coverage to 5G mid-band is crucial.

While 3GPP channel model parameters are used to model equivalent deployment scenarios in various cities for different operators, often the urban nature of different cities differs across countries and across continents. There is, for example, a big difference in the terrain, urban planning, and consequently deployment scenarios between San Francisco, Dallas or New York City, as well as a big difference between San Francisco, Prague or Paris. In the United States, a typical inter-site distance in suburban environments is on average around 1700m. The UMa channel model for urban Macro environments assumes a 500m ISD.

A typical city in the United States, outside of the downtown area, which can be modeled as a sub-urban or sometimes urban environment, is characterized by big residential areas of one or two-story buildings, high vegetation, sometimes rising above 25m, and commercial buildings that are 4 floors in height. These types of deployment scenarios are not well captured by the urban macro UMa scenario, used in the ITU-2020 evaluations, nor the rural macro RMa scenario, used to evaluate eMBB in ITU-2020; yet they constitute more than 70% of the deployment configurations in north America.

In Rel. 19, the study item on channel modelling enhancements for 7-24GHz for NR introduced a new channel model for suburban deployments called SMa – suburban macro scenario. The suburban macro scenario captures all the deployment characteristics for a suburban macro scenario, including the inter-site distance, the base station height and the nature of the clutter in these environments. The details of the SMa channel model were finalized in Rel. 19 as part of the study item and can be readily used for evaluations in 6G SIs and WIs as needed. It is thus very important to include this model in the evaluations of capabilities for IMT-2030, for all the considered frequencies, and all relevant usage scenarios.

## Forward Compatible Design for Diverse Device Types

Our detailed views can be found in our companion contribution in [4] and are summarized as follows:

The 6G RAN and 6G Radio should be designed bottom-up, meaning a single RAT is scaled from a minimum set of KPIs, and resources are allocated upwards to the most demanding ones. The 6G RAN and 6G Radio should enable a platform comprising one set of specifications that support all use cases with minimum customization and large degrees of economy of scale. Because crucial design questions can most efficiently be decided in the first release of 6G, 6G LPWA must be specified in the first release of 6G. 6G LPWA here refers to the evolution of 5G eMTC as specified in 3GPP, and KPIs should meaningfully differ from (e)RedCap. Solutions should be designed to last across technology generations in the core network, the RAN, and in overlapping spectrum with significantly improved efficiency compared to DSS between 5G and eMTC.

## Waveform, Modulation, Frame Structure, and Channel Coding

Our detailed views can be found in our companion contribution in [1] and are summarized as follows:

3GPP TSG RAN working groups should discuss how to meaningfully differentiate between software vs. hardware to assess the impact of non-backward compatible changes such that 6G deployment does not require complete overhaul of hardware to realize benefits. 3GPP should facilitate software-driven 6G deployment with existing open radios in legacy bands leveraging efficient 5G-6G MRSS, where and when needed. 3GPP RAN working groups should study the full capability of 6G when radios are refreshed in existing or new greenfield bands as necessitated by actual needs. Specifically, the UE shall always assume a base station deploys new 6G hardware. A UE shall be completely oblivious as to whether a given base station uses “capable 5G RAN infrastructure” or a new 6G radio. After network attach, through RRC (re)configuration, a vast number of novel waveform designs, modulations, and coding schemes could be considered, as long as they can coexist with the OFDM time-frequency grid as specified in 5G NR. By guaranteeing that the UE always assumes a base station deploys new hardware the pitfalls similar to 5G NSA are avoided where a UE was aware of whether 5G was rolled out as NSA or SA. This requirement is thus key to ensuring the principle of a single branch of 6G standards development. The vision of a software-based 6G rollout with needs-based hardware refresh is consequently perfectly congruent with the desire for 3GPP RAN working groups to study the full capability of 6G when radios are refreshed (waveform design, modulation, coding schemes …). Software-based 6G with needs-based hardware refresh is not a conservative approach to 6G. It is the very key to unleashing innovation through new radios and reflects operators’ desire of doing things fundamentally differently in 6G, by not requiring a complete, nationwide overhaul of hardware to realize the benefits of a new G.

## Testing and Certification

Our detailed views can be found in our companion contribution in [2] and are summarized as follows:

A globally aligned and normatively specified testing and certification procedure should ensure features are not just declared as mandatory, but deployment of these (and other features) is tracked in 3GPP with a binding certification procedure for both the network and the user equipment. For each 3GPP 6G work item, basic feature groups should be defined to ensure implementation, certification, and deployment of large numbers of features as components of a single feature group, rather than specifying a myriad of individual feature groups for a small number, and often a single component. These basic feature groups together define a meaningful, minimum set of features that are globally aligned among operators.

## Spectrum Aggregation

The 6G Radio shall support 6G-6G CA. The 6G CA design shall be significantly simplified as current CA implementations are becoming too complex and are not sustainable. Specifically, 6G shall optimize CA band combination configurations and capabilities.

## AI/ML Framework

Our views on the 6G AI/ML framework can be found in our companion contribution in [5] and are summarized as follows:

6G RAN and 6G Radio shall define an integrated framework for AI/ML data management based on the following design principles: - MNO visibility and control of AI/ML based data traffic (model transfer/delivery, data collection, model monitoring and management) - Universal framework to avoid fragmentation and per-use case specifications and ensure scalability to new AI/ML use cases - Support multiple termination points for AI/ML data within the network with MNO visibility - Differentiate AI/ML data traffic from user plane traffic and control plane traffic - Ensure data traffic security and privacy A unified flexible LCM framework is to be defined for model management, model transfer, model training, and model testing.

## Sensing

Integrated sensing and communication is a crucial feature for 6G radio, suitable for various object detection and tracking applications such as UAV tracking, smart transportation, and public safety. Several sensing topologies and modes can be considered for integrated sensing and communication.

Network-based sensing modes, including TRP-TRP bistatic, UE-TRP bistatic, and TRP-TRP monostatic sensing when feasible, should be included in the ISAC study in Rel. 20.

6G radio must address sensing performance for all sensing use cases by considering factors like angular, radial, and doppler resolution, link budget, interference sources, and environment and channel characteristics, while optimizing the sensing signal design for integrated sensing and communication.

# Conclusion

During the TSG RAN #108 meeting, 3GPP is expected to agree a new RAN WG SI for 6G. In this contribution, we provided our views on the drafting and scope of a working group level study for 6G.

# References

- RP-251702, Software-based 6G with Needs-based Hardware Refresh, AT&T

- RP-251704, A New Testing and Certification Paradigm for 6G, AT&T

- RP-251701, AT&T Views on minimum TPRs for IMT-2030, AT&T

- RP-251703, On 3GPP Support of LPWA in 6G, AT&T/Ericsson/FirstNet/KT Corp./T-Mobile USA/Verizon

- RP-251705, Views on 6G AI/ML Framework, AT&T