3GPP TSG RAN Meeting #108							RP-251709

Prague, Czech Republic, June 09, 2025

Source:	Qualcomm Incorporated

Title:	Views on Ambient IoT WI in Rel-20

Agenda Item:	15.1.3

Document for: 	Discussion and Decision

# Background

In RAN#106, WID of Rel-19 Ambient IoT was approved [1] with following general scope.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only.

In RAN#106, regarding the Rel-20 Ambient IoT scope, we agreed following WF in [2].

Workplan for Rel-20: - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase. For both Rel-19 and Rel-20, co-ordination with SA2/3 is expected

The above way forward was based on the following conclusion of Rel-19 Ambient IoT SI captured in the TR38.769 [3].

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR. (1) The following techniques are recommended in the first normative phase: - PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R - Modulations of OOK-1 and OOK-4 in R2D - Multiplexing in time domain for R2D - Convolutional FEC in D2R and no FEC in R2D - R2D and D2R signal(s) (2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release: - Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture) - D1T1 and/or D2T2 - Scenario A1 and/or A2 and/or B and/or C - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R - Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D - OOK and/or BPSK and/or MSK modulation for D2R - Carrier-wave waveform 1 and/or 2, and transmission case(s) - Device (un)availability direction 1 and/or 2 - Proximity determination solution 1 and/or 2

In this contribution, we provide our views on Ambient IoT work item for Rel-20.

# Work item scope for Rel-20 A-IoT

Rel-20 A-IoT can include aspects identified as beneficial or attractive during the Rel-19 study, to develop a new 3GPP-standardized A-IoT ecosystem. Unlike Rel-19 A-IoT work item which focused on T1 and Device 1, Rel-20 could have broadened and enriched the scope to further build up 3GPP A-IoT ecosystem addressing diverse use cases. Here are our views on the scope for Rel-20 A-IoT.

## 2.1	Use-cases and traffic types

In Release 19, the traffic types under consideration included device-oriented by device-terminated trigger (DO-DTT) and device-terminated (DT), with a focus on indoor inventory and indoor command use cases. These areas have been studied and should also be supported in the Rel-20 work item. In addition, the device-originated autonomous (DO-A) traffic for sensor use case should also be considered in the Rel-20 work item.

## 2.2	Topologies and deployment scenarios

In Rel-19, both D1T1 (deployment scenario 1 with connectivity topology 1) and D2T2 (deployment scenario 2 with connectivity topology 2) were studied. D2T2 presents a feasible and useful deployment/topology for indoor A-IoT scenarios. For instance, a customer can use a handheld terminal or smartphone to scan and inventory A-IoT devices in environments such as indoor smart homes, factories, and warehouses. Intermediate UE as a reader is also useful for a certain local area outdoor deployment where A-IoT devices are distributed outdoors, and a handheld/smartphone reader comes to scan/read the devices nearby. In these scenarios, the devices can still rely on RF energy harvesting and can use RF envelope detector-based receiver in addition to others, since the distance between the reader and the devices is similar to, or slightly larger than indoors. Accordingly, the requirements could be similar.

D2T2 requires specifying the Uu interface/signaling and relevant procedures for intermediate UE. However, these are transparent to A-IoT devices.

## 2.3	Device type(s)

In Rel-19, three device types have been studied and documented in TR38.769. For Rel-20, we recommend supporting device type 2b with advanced features to achieve larger coverage, improved energy efficiency, better spectral efficiency, and much better spectrum coexistence and interference filtering capability etc.

We do not support further studying or specifying device type 2a. Given that RF energy harvesting will be the default operating mode, the range for all device types will be limited by that capable of supporting energy harvesting, so range improvement of device type 2a over device type 1 would not be significant. At the same time, device type 2a will not improve upon spectrum coexistence and interference filtering capability like device type 2b could.

We do not support studying device types that assume power being supplied by installed batteries (such as ‘device type C’). The defining feature of Ambient IoT devices has been understood to be battery-less operation.

## 2.4	Essential features for Rel-20

Based on the RAN1 conclusion captured in TR38.769, we share our views on the essential features to support device 2b in D1T1 and D2T2.

RAN1 objectives

- The following basic techniques may be reused for Rel-20 work item with minimal modifications

- PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

- Modulations of OOK-4 in R2D

- Manchester line code for R2D

- Multiplexing in time domain for R2D

- Convolutional FEC in D2R

- R2D and D2R signal(s)

- At least the following aspects should be considered for inclusion in Rel-20 work item:

- Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture)

  - Synchronization of device 2b clock/LO via R2D transmission from a reader

- Deployment scenarios

  - D1T1-C

  - D2T2-C

  - D2T2-A2

- FEC/repetition with multiple coding rates for R2D and D2R

- Scrambling in R2D and D2R

- D2R waveform/modulation/coding and spectrum shaping

  - No line code

  - OOK and/or BPSK and/or MSK

  - Double sideband and/or Single sideband

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R; potential FDMA for device 2b (IF-ED/ZIF receiver) in R2D

- Better clock accuracy target for Device 2 (based on TR38.769)

- Solution(s) for Device (un)availability due to energy harvesting direction 2

  - Support at least a solution controlling device states (ON, OFF, SLEEP) (e.g., duty-cycle monitoring, any necessary periodic signaling, etc.)

- Proximity determination solution 2

  - D2R power measurement at reader side

RAN2 objectives

- Specify the A-IoT radio resource management to support intermediate UE as Reader

  - Uu signaling for A-IoT resource request and resource configuration

- Specify the signaling and procedure of UE Reader performing A-IoT operation in all RRC states

- Specify the signaling and procedure of UE Reader performing A-IoT operation upon temporary out of connection

- Study and if found feasible, specify the UE Reader performing A-IoT operation in out-of-coverage scenario.

- Multi reader scenario support: device under coverage of multiple readers

- Enhancements to support sensor use case (DO-A) for both T1 and T2

RAN3 objectives

- Specify RAN architecture and protocols to support A-IoT operations with intermediate UE as Reader using user-plane transport between the UE Reader and the AIOTF

- Xn signaling to support UE Reader mobility and multi reader scenario

- Reader-CN signaling to support other A-IoT use cases (e.g., sensors, tracking, outdoor use cases)

- Finer granular techniques for locating the A-IoT device (e.g., using multiple readers)

RAN4 objectives

- Specify a frequency band and the system parameters for Ambient IoT system

  - Example band(s) is needed

- Specify RF requirements for Ambient IoT devices and reader (gNB and intermediate UE)

  - Rx and Tx requirements

    - Conduct the coexistence analysis based on simulation assumptions and methodology in TR 38.769 (e.g., ACLR, ACS for readers and unwanted emission for devices)

    - Study and specify the guard-band between NR and Ambient IoT system if necessary

  - The feasibility of CW interference cancelation and its impact on performance

  - RF energy harvesting shall be assumed and taken into account.

- Study and define the test methodology for devices RF requirements verification

- Demodulation performance requirements

- List new specification documents if so decided

NOTE: Coordination with other RAN WGs and SA WGs on the above objectives as needed.

# Timeline

Supporting device 2b may need substantial enhancements compared to the Rel-19 A-IoT. It is reasonable to allocate the entire Rel-20 timeline (i.e., 18 months) for the Rel-20 A-IoT work item.

# Conclusion

In this contribution, we provided our views on Ambient IoT work item for Rel-20. The proposal is summarized as follows.

Proposal:

- RAN to approve a Rel-20 WI for Ambient IoT

  - Support device 2b without backscatter operation

  - Support both D1T1-C, D2T2-C for device 2b and D2T2-A2 for device 1

    - Specify necessary Uu signalling/procedure for intermediate UE as a reader

  - Support additional multiplexing schemes to improve spectrum efficiency in R2D and D2R

  - Support solution(s) for device (un)availability due to energy harvesting Direction 2

  - Support Proximity determination solution 2

  - Specify RAN architecture and protocols to support A-IoT operations with intermediate UE as Reader using user-plane transport between the UE Reader and the AIOTF

  - Support UE Reader performing A-IoT operation in all RRC states, temporarily out-of-connection and in out-of-coverage

# Reference

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- RP-243280 Way forward for Ambient IoT for REL-19 and REL-20

- 3GPP TR38.769 Study on solutions for Ambient IoT (Internet of Things) in NR