3GPP TSG RAN Meeting #108	                                                RP-251724

Prague, Czech Republic, June 9-13, 2025

Title: 	Views on further network energy saving enhancement in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.1.6

Document for:	Discussion and decision

Introduction

In R19, on demand SSB SCell operation for UE in connected mode configured with CA, on demand SIB1 for UE in idle/inactive mode as well as SSB adaption, PRACH adaption in time domain and paging adaption are specified. For R20, we still see room for further energy saving at network side and would like to share our thoughts in this paper.

On-demand SSB for UEs in idle/inactive mode

The energy consumption caused by periodic SSB transmission is a non-negligible part of that of the gNB, especially in the case of empty/low load. Reducing or even eliminating SSB transmission can bring considerable energy saving gains. In order to reduce the SSB transmission, SSB-less SCell for inter-band CA, on-demand SSB SCell for connected mode UE, and SSB adaptation in time domain are specified in Rel-18 and Rel-19. However, due to the limited application scenarios, these energy saving technologies cannot achieve the maximum energy saving gain. For example, SSB-less SCell is limited to FR1 and co-located scenarios, on-demand SSB only used for SCell with NCD-SSB out of sync-raster, and the SSB adaptation also needs to minimize the legacy UE impacts of initial access.

The applicable scenarios of SSB transmission reduction specified in Rel-18 and Rel-19 are limited.

For UEs in idle/inactive mode, to guarantee the initial access of the UE, the maximum SSB transmission cycle is 20ms, which lead to significant network energy consumption. Therefore, reduce the SSB transmission in idle/inactive mode is crucial for network energy saving. In Rel-20, on-demand SSB transmission for idle/inactive mode UE should be considered.

Compared with the on-demand SSB transmission introduced in Rel-19, which is only used for SCell with NCD-SSB out of sync-raster, the on-demand SSB transmission for idle/inactive mode UE can effectively extend the on-demand SSB application scenarios, and achieve the network energy saving gain by reducing the SSB transmission.

Introduce on-demand SSB transmission for idle/inactive mode UE in Rel-20 can extend the on-demand SSB application scenarios, and achieve the network energy saving gain by reducing the SSB transmission.

For the on-demand SSB transmission for idle/inactive mode UE, a wake-up signal (WUS) can be considered to trigger the transmission, similar to the on-demand SIB1 introduced in Rel-19. In addition, the NES cell supporting on-demand SSB can function as a capacity cell and operate in heterogeneous manner with a coverage cell. The idle/inactive mode UEs can acquire the synchronization and WUS configuration of the NES cell through the coverage cell before the SSB is triggered.

The idle/inactive mode UEs can acquire the synchronization and WUS configuration of the NES cell through the coverage cell before the SSB is triggered.

What’s more, considering the impact on the legacy UE, the on-demand SSB transmission for UEs in idle/inactive mode can be further considered in the new or re-farming spectrum without commercialized UEs, such as 6GHz, etc. In addition, leaner common signals/channels design is also beneficial for better spectrum sharing between 5G-A and 6G.

The on-demand SSB transmission can be further enhanced considering the new or re-farming spectrum without commercialized UEs, and leaner common signals/channels design can be expected to provide better spectrum sharing between 5G-A and 6G.

Above all, on-demand SSB transmission for idle/inactive mode UEs should be supported in Rel-20.

Further enhancement on SSB transmission reduction, such as on-demand SSB for idle/inactive mode UEs, can be specified in 5G-Advanced Rel-20.

Conclusion

Based on the discussion about Rel-20 NES techniques above, we have the following observations and proposals.

Observation 1: The applicable scenarios of SSB transmission reduction specified in Rel-18 and Rel-19 are limited.

Observation 2: Introduce on-demand SSB transmission for idle/inactive mode UE in Rel-20 can extend the on-demand SSB application scenarios, and achieve the network energy saving gain by reducing the SSB transmission.

Observation 3: The idle/inactive mode UEs can acquire the synchronization and WUS configuration of the NES cell through the coverage cell before the SSB is triggered.

Observation 4: The on-demand SSB transmission can be further enhanced considering the new or re-farming spectrum without commercialized UEs, and leaner common signals/channels design can be expected to provide better spectrum sharing between 5G-A and 6G.

Proposal 1: Further enhancement on SSB transmission reduction, such as on-demand SSB for idle/inactive mode UEs, can be specified in 5G-Advanced Rel-20.

References

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR