3GPP TSG RAN meeting #108                                                 RP-251730

Prague, Czech Republic, June 9-13, 2025

Title: 	Views on NR-enabled UAV in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.2.5

Document for:	Discussion and decision

Introduction

As the additional RAN2-led topic, the following consensus has been reached at RAN#107[1]:

- As a starting point we consider: - Specify a UAV/UAM specific frequency/cell/beam prioritization mechanism for IDLE/INACTIVE mobility. TBD next meeting whether we have a separate WI or TEI - FFS CHO for RAN#108 - If this is a WI, the work should be completed in 2 Quarters.

In this contribution, our views on this item are elaborated.

Discussion on the scope of NR-UAV

As highlighted in [2], to enable the “low altitude economy”, the celluar-based UAV connection is one of critical aspects. On top of the specification works in Rel-18, the following consideration is made in Rel-19.

Conclusions - Large operator support for UAV as one of the most important Rel-19 topics (within the items in this offline) from a commercial and mission critical perspective. - Architectural changes will not be part of the UAV work (i.e. any architectural enhancements can be considered under NTN) - Starting point of an acceptable objective: - Mobility enhancements - Specifying new CHO triggering events - Studying/specifying LTM enhancements for UAV - No transmit zone (if SA2 will also do work on this) - Enhancements for RRC_IDLE/RRC_INACTIVE

It can be found that in addition to enhancement for RRC_IDLE/RRC_INACTIVE, the CHO is also highlighted. The reason behind is that:

- For the UAV network, to ensure the seamless service along with the continuous monitoring (e.g., flight path reporting, CC signalling) due to the regulatary needs, once the UAV device taken off, it’s almost always in RRC-connected instead of the RRC_IDLE/INACTIVE. From this perspective, supporting the connected state mobility is more important.

- As captured in TR 36.777, the number of detectable cells and the range of the detected cells increases as UAV altitude increases. Besides, UAV UE moves faster compared to ground UEs. Thus, more frequent handover shall be triggered for UAV UE. Improving mobility robustness and reducing mobility latency are key points to be considered for UAV mobility enhancements.

In Rel-16, CHO is introduced to improve mobility robustness. Considering the characteristics of UAV, there could be more candidate cells available for CHO, and different candidate cells in different altitude ranges. Thus, the height information and/or flight path information reported by the UE can be considered for CHO candidate cell preparation and CHO triggering.

Then, for the 5G-A Rel-20 UAV, the CHO is still preferred to be supported.

Additionally, regarding the organization of corresponding works, given the potential load, either the Rel-20 TEI or dedicated WI is fine.

Proposla 1: For the NR-enabled UAV in 5G-A Rel-20, the CHO with following objective can also be considered:

- Specifying new CHO triggering events, e.g., height-based and/or flight-path based CHO triggering events.

Conclusion

Based on the discussion about NR-enabled UAV in 5G-A Rel-20, the following proposal is made:

Proposla 1: For the NR-enabled UAV in 5G-A Rel-20, the CHO with following objective can also be considered:

- Specifying new CHO triggering events, e.g., height-based and/or flight-path based CHO triggering events.

References

- RP-250812 Summary for RAN Release 20 5G-Adv

- RP-251730	Views on NR-enabled UAV in 5G-A Rel-20