3GPP TSG RAN WG meeting #108	                                                                            RP-251733

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.3.3

Source:	ZTE Corporation, Sanechips

Title:	Views on SON/MDT in 5G-A Rel-20

Document for:		Discussion and Decision

1. Introduction

The data collection for SON&MDT support for NG-RAN has been specified from Rel-16 to Rel-19[1][2][3][4]. In this contribution, the motivation and scope of the proposed new work item towards Rel-20 is elaborated.

- Discussion

During RAN#107 meeting, the scope of Rel-20 SON/MDT in 5G-A has been discussed and way forward of this topic has been captured in RP-250812, the detail is excepted as below:

The following to be endorsed as WF: The objectives of SON/MDT in Rel-20 including and to be down-selected on the following (1 or 2 use cases): MRO enhancement for R19 LTM (the detail scope should be limited and needs to be further checked based on the progress in R19 LTM). [RAN3, RAN2] Support of SON/MDT enhancements for UAV or LP-WUS/WUR [RAN3, RAN2] Note: Coordination with other working groups when needed. 18 months for the Rel-20 SON/MDT WI.

2.1 MRO enhancement for R19 LTM

- MRO for inter-CU LTM

For inter-CU LTM, the normative work primarily focuses on coordination between CUs. Therefore, in Rel-20 MRO, the Xn interface should be considered to enhance mobility robustness, particularly in scenarios involving  L1/L2-triggered mobility events . Both cases  (e.g., radio link failure) and  near-failure cases  (e.g., SHR) should be addressed in this release. Based on information provided by the UE or derived from the network, the  receiving node should also consider implementing a  failure message forwarding mechanism  in Rel-20. Additionally, mechanisms established in Rel-19 MRO should be taken into account for backward compatibility and consistency. The possible enhancements for this features including new parameters in RLF report and SHR reports and forwarding signalling in XnAP.

Conditional LTM (C-LTM) is one of the key objectives under the Mobility Enhancements topic. It aims to improve mobility procedures by introducing conditional LTM updates, which allows the network to instruct the UE to perform  flexible L1- or L3-triggered mobility in Rel-19. Therefore, C-LTM should also be selected as a  candidate objective in Rel-20 MRO for LTM. Both  failure and near-failure cases should be considered in the context of C-LTM as well. Unlike the CHO mechanism, C-LTM introduces new parameters such as  TAT (Timing Advance Timer)  to evaluate TA values. Consequently, corresponding  MRO mechanisms for optimizing these new parameters.  The possible enhancements in this feature including new parameters in RLF report and SHR reports and optimization of TAT configuration.

Proposal 1: MRO optimization for inter-CU LTM and conditional LTM optimization should be considered for normative work in Rel-20.

- Leftover issues from Rel-19

Due to time constraints in Rel-19, several features within the original scope were deferred and not supported in that release. For MRO for LTM , RAN3 prioritized  MCG LTM  over  SCG LTM ; as a result,  MRO for SCG LTM  was not addressed in Rel-19, leading to an incomplete MRO mechanism. The possible enhancements in this feature including new parameters in SPR report.

Proposal 2: MRO for SCG LTM can be considered as leftover issues for normative work in Rel-20.

2.2 Support of SON/MDT enhancements for UAV or LP-WUS/WUR

To develop  Rel-20 5G-Advanced SON/MDT functions with a focus on real commercial priorities,  mobility robustness  remains one of the most critical features within SON. The new mobility enhancement features introduced in Rel-19, such as  inter-CU LTM  and  conditional LTM , should be further refined in Rel-20 to improve MRO performance.

As for features like  UAV mobility  and  LP-WUS/WUR , considering the shorter development timeline compared to Rel-19, and the fact that their commercial deployments are not yet sufficiently mature, we propose that enhancements related to these features be postponed to a future release.

Proposal 3: SON/MDT enhancements for UAV or LP-WUS/WUR can be considered in R20 if time allows.

3. Conclusion

In this document, the motivation and the potential topics for data collection for SON&MDT in NR and MR-DC towards Rel-20 are described and the proposals as follows:

Proposal 1: MRO optimization for inter-CU LTM and conditional LTM optimization should be considered for normative work in Rel-20.

Proposal 2: MRO for SCG LTM can be considered as leftover issues for normative work in Rel-20.

Proposal 3: SON/MDT enhancements for UAV or LP-WUS/WUR can be considered in R20 if time allows.

Proposal 4: RAN plenary to consider the following objectives for SON/MDT in 5G-A Rel-20.

Objectives:

- MRO enhancement for R19 mobility mechanisms, including Inter-CU LTM and Conditional LTM [RAN3, RAN2]:

  - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

  - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

- Support of the leftovers in Rel-19 SON/MDT, including:

  - MRO for SCG LTM [RAN3, RAN2]

If needed, co-operate with RAN1, SA2, SA5 when needed.

4. Reference

[1]  RP-250812, WID on SON/MDT support for NR

[1]  RP-192610, WID on SON/MDT support for NR

[2]  RP-201281, WID on enhancement of data collection for SON/MDT in NR and EN-DC

[3]  RP-221825, WID on further enhancement of data collection for SON/MDT in NR and MR-DC

[4] RP-234038, Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[5] RP-210460 SID on XR Evaluations for NR

[6] TR 38.838 Study on XR (Extended Reality) Evaluations for NR

[7] RP-230786 R18 WID on XR Enhancements for NR

[8] RP-241771 R19 WID on XR (eXtended Reality) for NR Phase 3