3GPP TSG RAN meeting #108                                                 RP-251737

Prague, Czech Republic, June 9-13, 2025

Title: 	ZTE's views on candidate topics Impacting both RF/OTA and Demod/RRM for Rel-20

Source: 	ZTE Corporation,Sanechips

Agenda item:	15.4.4

Document for:	Discussion

Introduction

In the RAN#107 meeting, there was some initial discussion on Rel-20 RAN4 package and the discussion outcome is summarized in moderator’s summary [1]. In this contribution, we would like to share our view from NTN and cross-area perspective.

Discussion

- 2.1. NTN enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ proposals on Rel-20 NTN enhancement so far are summarized in the following table.

| Numbering | Objective | ZTE’s Preference |
| --- | --- | --- |
| 1 | NTN HPUE | High priority To add more details for this objective For IoT NTN, both PC1.5 and PC1 for handheld UE is feasible by reusing the existing GSM PA and without duplexer implementation challenge. For NR NTN, PC1.5 for handheld UE needs more feasibility study in RAN4. |
| 2 | NTN CA | Neutral for FR1 Intra-band and inter-band UL/DL CA for NR-NTN |
| 3 | Ku band NTN | Positive and propose to add the HD-FDD for Ka-band if HD-FDD for Ku is agreeable. |
| 4 | NTN mobile VSAT | Neutral |
| 6 | NTN power domain enhancements | Positive |
| 7 | MSR for in-band IoT-NTN in NR | Neutral |
| 8 | Irregular CBW 7MHz for NR-NTN | Neutral, this is up to the operator’s spectrum and also request. |
| 9 | Mobility between TN and NR-NTN | Middle priority. This depends on RAN2 agreement |
| 10 | Repeater for NTN | High priority |

- 2.1.2 Justification

Objective 10:

In Rel-17, RF repeater is introduced to improve the coverage of terrestrial network. In Rel-18, NCR is introduced to further improve the coverage in more intelligent way. In Rel-19, lots of HPUE related enhancements are approved to further improve the NTN UL coverage. The coverage issue for NTN scenario is expected to be even worse than the terrestrial network, how to further improve the coverage from network deployment perspective is also critical. Rel-17 RF repeater/Rel-18 NCR could be well leveraged for coverage enhancement for NTN scenario. In Rel-20,it is proposed to specify the RF repeater for NTN scenario at least.

- 2.1.3 WID objective

- To define the RF requirement for Ka-band/Ku-band NTN VSAT with HD-FDD

To define the RRM requirement for Ka-band/Ku-band NTN VSAT with HD-FDD

- To define the RF requirement for the terrestrial repeater for FR1 NTN deployment [RAN4].

  - TN repeater 38.106 should be treated as starting point.

- 2.1.4 Proposed TU budget

NOTE: the proposed TU budget for NTN enhancement in Rel-20 is based on the other 1/2 objectives to be agreed at the end.

- 2.2. Cross areas

- 2.2.1 ZTE’s view

The high level views on the companies’ proposals on other enhancement so far are summarized in the following table.

| Numbering | Objective | ZTE’s Preference |
| --- | --- | --- |
| 1 | NR FR1 DL fragmented carrier | Positive |
| 2 | Extension of low-low band CA via switching | Neutral and scope should be well limited to minimize the workload in other WGs. |
| 3 | Irregular channel bandwidth (flexible channel bandwidth) | Low priority, It could be covered by the existing network implementation. |
| 4 | 3MHz channel bandwidth for eRedcap devices | Neutral |
| 6 | Intra-band non-collocated EN-DC/NR-CA deployment | Neutral |
| 7 | AI/ML assisted receiver | Have interest and treat it in 6G phase |
| 8 | ATG evolution | Low priority |
| 9 | UAV enhancement | High priority NR based UAV is introduced into RAN4 specification in Rel-18, however UAV power class is limited to PC3, considering the potential coverage and use case of UAV service, it is proposed to consider the higher power for UAV operation. |

Conclusions

In this contribution, we shared some further views on Rel-20 RAN4 package based on RAN#107 moderator’s summary and the outcome of adhoc discussion.

References

- Moderator's summary for RAN4 led REL-20 topics [RP-250817]