3GPP TSG RAN Meeting #108																		RP-251739

Prague, Czech Republic, June 9-13, 2025

Source:	ZTE Corporation, Sanechips

Title:	High level views on 6G RAN general study

Agenda Item:	16.1

Document for:   Discussion and Decision

- Introduction

In RAN#107 meeting, the new 6G RAN SID was updated, where new services and the corresponding scenarios and requirements should be a crucial part of study. The agreed objective is listed below [1].

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

In this contribution, we provide our views on the general RAN study for 6G, and our suggestion for the skeleton of the TR for this study.

- Discussion

For 6G, while considering that we need to have specific outcome for ITU IMT-2030 technical performance requirement, the objectives/aspects of RAN level study are generally divided into two steps.

- Step-1: Study on key performance indicator(s), which refer to the outcome of IMT-2030 Technical Performance Requirements (TPRs). The details can be found in our companion contribution [2].

- Step-2: Study on 6G general issue(s) which can be similar to the 5G SID as in TR 38.913[3], while being based on the outcome of Step-1. First of all, we have the objective of this study as described in the Section 1 agreed in RAN#107. Then, as a final outcome, we will have a new/specific TR38.914 [4] on 6G-SID. As usual, in order to well move forward this new TR discussion, we believe that we should have an investigation on the work plan and then develop a draft table of content for the new TR for 6G, i.e. a skeleton of the TR. The details on 6G deployment scenarios, ISAC, AI/ML, 6G UE device types, RIS and NTN can be found in our companion contributions [5-10], respectively.

Regarding Step-1, as IMT-2030 TPRs developed by the ITU-R will include new items compared to IMT-2020 TPRs, it is anticipated that 6G RAN KPIs will also contain new items, which is a major reason to update the skeleton of 6G RAN TR. As reflected by the current discussion progress, we are likely to see new ITU TPRs for IMT-2030 on top of the IMT-2020 TPRs. AI related capability, composite requirement, and positioning are likely to become new TPRs, with the first one most likely a qualitative TPR. The new TR for 6G RAN can be updated accordingly to add these new KPIs. For positioning, in TR 38.913 [3], positioning is included under section for supplementary service related requirements. As it is going to become an independent TPR, it can be moved to the section for KPIs.

Regarding Step-2, besides for inheriting the existing successful services/scenarios (e.g., eMBB) as described in 5G TR 38.913, more importantly, we should identify the new services/scenarios/requirements to ensure the long-term prosperity of IMT/3GPP.

- As an emerging technique/generational leap for 6G, Integrated Sensing and Communication (ISAC) should be well studied. A dedicated clause in TR is deserved at least for ISAC use case study. Particularly for sensing, 3GPP SA1 has started to study sensing use cases from 5G-A, and already identified four agreed use cases, namely, UAV detection and tracking, Pedestrian detection and tracking, Vehicle detection and tracking, AGV detection and tracking in TS 22.137 [11]. For 6G, SA1 also agreed to support ship detection and tracking in TR 22.870 [12]. All these use cases would demand new KPIs and new deployment scenarios to properly demonstrate their performance. As in our companion contribution [6], we elaborate our views on 6G ISAC scenarios and requirements .

- For 6G deployment scenarios, some more new scenarios, such as indoor factory, nearshore waters and possibly others, for the need to serve communication users in a variety of cases and the need for sensing services. All deployment scenarios for both communication and sensing can be listed together.  In addition, it’s important in 6G to identify more practical and energy-efficient manners to achieve enhanced spectral efficiency and coverage. This potentially can be achieved by deploying more sustainable network nodes including passive/semi-passive nodes, very lower power nodes without PA, nodes with energy harvesting capability. Hence, it is proposed to have a dedicated sub-section under the section of deployment scenarios of TR 38.914[3] to capture all the potential energy-efficient network deployment scenarios in 6G [9].

- Without doubt, AI/ML should be considered as another technical breakthrough for 6G. Consequently, for AI part, like ISAC, we believe a separate clause should be included, and, in this clause, we can further discuss the detailed issues, such as general principles of AI/MI network and unified AL/ML LCM and data collection. For sake of presentation, our detailed thoughts for AI/ML can be found in our companion contribution [7].

- Some operational requirements including TN and NTN integration[10], scalable and forward compatible design for diverse device types, etc. should also be included. The detailed views on scalable and forward compatible design for diverse device types can be found in our companion contribution [8].

Based on these reasons, we propose a 6G RAN TR skeleton on the following aspects.

Proposal 1: The 6G RAN TR skeleton can be based on the NR TR 38.913 with the revision on the following aspects:

- For KPIs, refers to the outcome of ITU-TPR, and add new items such as for sensing, AI/ML, positioning, composite requirements

- For deployment scenarios, add new scenarios such as indoor factory and nearshore waters, and have a dedicated sub-section to capture all the potential energy-efficient network deployment scenarios.

- Add new separate clauses for 6G ISAC and AI/ML, in order to facilitate the comprehensive studies on emerging 6G techniques.

- Add new operational requirements such as TN and NTN integration, scalable and forward compatible design for diverse device types, etc.

Note: a skeleton for the TR 38.914 is provided in section 5.

As to the time plan to steer work for the RAN WGs as below, we propose to check WG progress for those items in June 2026 considering the balance between 6G progress and ensuring time for technique study, analysis, and evaluation. For example, some aspects (e.g. on waveform/modulation/channel coding) may need detailed studies to make decision and such detailed studies may last the whole SI timeframe depending on the technical directions.  From our perspective, we should avoid frequent checkpoints as more checkpoints can imply more technical discussion in RAN plenary.   The progress should be kept track in each WG under a proper work plan instead (which is business as usual). If needed, we can setup only one checkpoint (in June 2026) to check the WG progress and determine whether we need to refine the RAN WG SI scope in RAN plenary.

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface and RAN internal interfaces (including CU-DU split, CP-UP split)

Proposal 2: In June 2026, RAN check the WG progress on fundamental 6G radio design aspects including waveform, numerology, and channel coding, on migration from 5G to 6G, on RAN architecture and interfaces, and on coordination of 6G AI/ML framework.

- Conclusion

In this contribution, we elaborate our views on the general RAN study for 6G, and also provide our suggestion for the skeleton of the TR and the time plan.

Proposal 1: The 6G RAN TR skeleton can be based on the NR TR 38.913 with the revision on the following aspects:

- For KPIs, refers to the outcome of ITU-TPR, and add new items such as for sensing, AI/ML, positioning, composite requirements

- For deployment scenarios, add new scenarios such as indoor factory and nearshore waters, and have a dedicated sub-section to capture all the potential energy-efficient network deployment scenarios.

- Add new separate clauses for 6G ISAC and AI/ML, in order to facilitate the comprehensive studies on emerging 6G techniques.

- Add new operational requirements such as TN and NTN integration, scalable and forward compatible design for diverse device types, etc.

Note: a skeleton for the TR 38.914 is provided in section 5.

Proposal 2: In June 2026, RAN check the WG progress on fundamental 6G radio design aspects including waveform, numerology, and channel coding, on migration from 5G to 6G, on RAN architecture and interfaces, and on coordination of 6G AI/ML framework.

- References

- RP-250810 “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-251738 “Views on IMT-2030 TPRs”, ZTE, Sanechips

- 3GPP RAN, TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

- RP-250809, “Proposal for structure for 6G RAN SI skeleton TR”, CMCC

- RP-251740, “General view on 6G deployment scenarios”, ZTE, Sanechips

- RP-251742 “Study on 6G scenarios and requirements - ISAC”, ZTE, Sanechips

- RP-251741, “Study on 6G scenarios and requirements - AI/ML part” ZTE Corporation, China Unicom, Pengcheng Laboratory, Sanechips

- RP-251743, “Views on requirements for 6G diverse device types”, ZTE, Sanechips

- RP-251745, “Energy-efficient network deployment scenarios in 6G”, ZTE Corporation, China Telecom, China Unicom, KT Corp., Sanechips, Shanghai Jiao Tong University, Queens University Belfast

- RP-251744, “Views on 6G NTN scenarios”, ZTE, Sanechips

- 3GPP SA1, TS 22.137, “Service requirements for Integrated Sensing and Communication”

- 3GPP SA1, TR 22.870, “Study on 6G Use Cases and Service Requirements”

- Skeleton for TR of 6G RAN study

Below we provide a draft skeleton for the new TR for RAN 6G scenarios and requirements,

1 Scope 2 Reference 3 Definitions, symbols and abbreviations 4 Introduction 5 Objectives 6 Deployment Scenarios 6.0 General 6.1 Deployment scenarios [NOTE: Compared with NR TR 38.913, at least some new deployment scenarios should be added including, indoor factory, nearshore waters, etc., see section 6.1.13, and 6.1.14. NOTE: A dedicated sub-section should be added to capture all the potential energy-efficient network deployment scenarios, see section 6.1.15.] 6.1.1 Indoor hotspot 6.1.2 UMi 6.1.3 RMa 6.1.4 UMa 6.1.5 High speed 6.1.6 Extreme long distance coverage in low density areas 6.1.7 Urban coverage for massive connection 6.1.8 Highway Scenario 6.1.9 Urban Grid 6.1.10 Commercial Air to Ground scenario 6.1.11 Light aircraft scenario 6.1.12 Indoor factory 6.1.13 Nearshore waters 6.1.14 Energy-efficient network deployment scenarios 7 Requirements and key technical principles [NOTE: The described KPIs can refer to the outcome of ITU TPRs. Compared with NR TR 38.913, new KPIs including sensing, positioning, composite requirements, AI/ML, etc. should be added.] 7.1 Key performance indicators 7.2 Requirements for architecture and migration [NOTE: Including System simplification: This can include reducing configuration complexity, enabling more efficient Cell/UE management, etc, also ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting] 7.4 AI and communication Including general principles and LCM coordination. 7.5 ISAC Including use cases study. 7.6 Other supplementary-service related requirements …. 7.7 Operational requirements 7.7.1 TN and NTN integration 7.7.2 Scalable design for diverse device types 7.7.3 Duplexing flexibility …… 7.7 Testing and conformance requirements