3GPP TSG RAN Meeting #108	RP-251740

Prague, Czech Republic, June 9-13, 2025

Source:	ZTE Corporation, Sanechips

Title:	General views on 6G deployment scenarios

Agenda Item:	16.1

Document for:   Discussion and Decision

- Introduction

In RAN#107 meeting, the new 6G RAN SID was updated, where new services and the corresponding scenarios and requirements should be a crucial part of study. The agreed objective is listed below [1].

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

In this contribution, we elaborate our general views on 6G deployment scenarios, especially based on the analysis from coverage and energy efficiency perspective.

- Discussion

As shown in the SID in above box of section 1, the goal of 6G includes the following two bullets:

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

As the frequency goes up, e.g. from 3.5GHz to 7 GHz, both propagation loss and penetration loss increase significantly. It results in an estimated 12 dB coverage gap between 3.5GHz and 7 GHz. Compensating for this gap is crucial to ensure successful 6G deployment. In order to enhance overall cell-edge performance, and meanwhile to ensure energy efficiency and energy saving for both network and UE side, the following directions having impacts on the deployment scenario and requirements should be considered.

- Spatial domain, such as increasing the number of antennas/RF chains at the BS and/or UE, and introduce coherent joint transmission multi-TRP (CJT-mTRP) and new architecture of DL-TRP and UL-TRP decoupling, i.e., asymmetric DL sTRP/UL mTRP.

- Multi-carrier operation, allowing UEs to flexibly operate across both 7 GHz and lower frequency bands with minimal UE complexity. Especially, for UL, a UE configuring with both low frequency and high frequency carriers can solve the coverage issue while keep good system capacity.

In addition to the above consideration, our views of 6G deployment scenarios and requirements on ISAC, AI/ML, NTN, energy-efficient network are provided in our companion contributions [3-6].

2.1 Deployment scenarios with ultra-massive MIMO

To address the issue of coverage and/or higher downlink spectrum efficiency, larger antenna arrays for single TRP transmissions with an increased number of antennas, multi-TRP operation (coherent joint transmission, CJT, and non-coherent joint transmission, NCJT) and new architecture of DL-TRP and UL-TRP decoupling have an increased interest in the industry, partly due to new available frequency bands. Then, in this section, we elaborate the 6G deployment scenarios with multi-antenna schemes.

### 2.1.1 Single TPR scenarios

While enabling U6G/around 7GHz communications, to compensate link-budget loss due to higher frequency and to maximize the spatial multiplexing/diversity gains, the significant link budget loss over 3.5GHz should be compensated by beam-forming gain with having much more antenna elements.

- For instance, compared with 3.5GHz, U6G has to experience an extra path loss of -5.8 dB and an extra penetration loss of -0.9 ~ -4.4 dB (depending on low/high penetration loss), i.e., a total of -6.7 ~ -10.2dB loss. Also, this issue become more severe for FR3, e.g., for 13 GHz.

Figure 1 Typical antenna array for 3.5GHz (64T192A) and U6G (128T1024A)

While benefiting by shorter wave-length due to higher centric frequency, we can consider to deploy ultra-massive MIMO (e.g., 128T1024A or even more) to handle this total of -6.7 ~ - 10.2dB link-budget loss, compared with 3.5G typical 32T96A. One example for candidate antenna array for U6G is shown in the Figure 1. No doubt that the cell-edge UE throughput can be improved significantly as well. In such case, the property of near-field communications may be observed.

- Through comprehensive SLS evaluation, as shown in Figure 2, it can be observed that: if still using the same antenna arrays as 3.5GHz, there is a dominant performance loss (-40.25% and -71.70% loss for average and cell-edge cases, respectively); then, ultra-massive MIMO (e.g., 128T1024R) can bring significant gains and outperform the typical 3.5GHz. It should be noticed that there is still an extra performance gains if further extending ultra-massive MIMO to 256T1024R or 512T2048R. Note that, in such case, DL 4-RX UE as legacy is considered.

Figure 2 Evaluation results for enabling ultra-massive MIMO for U6G (i.e., f = 7GHz)

- Then, with the increase of frequency band, deploying more DL Rx antennas in the hand-held terminal becomes possible as another critical enhancement. While taking DL 8-RX antenna into account, as shown in Figure 3, it can be observed that the increase of DL RX from 4Rx to 8Rx bring significantly additional gain in terms of both average and cell-edge UPT. While observing the NW deployment from 32T96A to 256T1024A, we can observe that there are 60%~80% average UPT gain and up to 300% cell-edge gain. Furthermore, if having some more Tx/Rx in UE sides, like 16T/16R, some more performance gain can be observed.

Figure 3 Evaluation results for additional gain via introducing DL 8-RX UE over legacy 4-RX MIMO for U6G (i.e., f = 7GHz). Left: Average UPT, Right: Cell-edge UPT

Finally, similar to DL transmission, we have the similar observation for UL transmission/coverage enhancement upon the support of ultra-massive MIMO. The SLS results can be found in the Figure 4. It can be observed that, for increase of the number of Tx in a UE, UL coherent transmission and corresponding high-resolution UL codebook become critical for boosting up UL transmission, especially for cell-edge UE (50~100% UPT gain). That means that, compared with 5G, UL coherent transmission should be mandatorily supported by 6G.

Figure 4 Evaluation results for additional gain via introducing ultra-massive MIMO for U6G while considering different UE device types (noncoherent/coherent Tx) (i.e., f = 7GHz). Left: Average UPT, Right: Cell-edge UPT

Based on the above evaluation, we should introduce the cases of gNB/UE antenna element of around 7GHz under the at least following deployment scenarios as mentioned in TR 38.913, large antenna array with more than 1024 NW antenna elements should be considered, e.g. 1536 or 2048 antenna elements, should be supported.

- Indoor hotspot

- Dense urban

- Rural

- Urban Macro

Proposal 1: To ensure overall coverage, focus on cell-edge performance and UL coverage (especially for co-site deployment of around 7GHz), large antenna array with more than 1024 BS antenna elements (e.g. up to 1536 or 2048 antenna elements) and 8 or more Tx and Rx UE antenna elements (e.g., up to 16) should be considered.

- Indoor hotspot

- Dense urban

- Rural

- Urban Macro

For instance, we have the following modification in red on the top of indoor hotspot as in TR 38.913:

| BS antenna elements NOTE4 | Around 30GHz or Around 70GHz: Up to 256 Tx and Rx antenna elements Around 7GHz: Up to [1536 or 2048] antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements |
| --- | --- |
| UE antenna elements NOTE4 | round 30GHz or Around 70GHz: Up to 32 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements |

### 2.1.2 Multi-TRP scenarios including asymmetric DL sTRP/UL mTRP

Besides for the increase of antenna elements in a single transmission point, distributed MIMO has become another emerging feature for maximizing multi-antenna gains while balancing implementation complexity and throughput performance. In our views, coherent joint transmission (CJT) improves coverage and average throughput in commercial deployments with high-performance backhaul and synchronization, targeting FR1, can be beneficial in expanding the utility of multi-TRP deployments; non-coherent joint transmission (NC-JT) can be considered as a starting point for FR2 deployment to accommodate the challenge of unpredicted blockage. The deployment scenarios of multi-TRP operation (CJT/NCJT) can be divided into the following three categories: a) intra-site, b) inter-site, and c) intra-&inter-site as shown in Figure 5.

Figure 5 Deployment scenarios for coherent JT: (a) intra-site, (b) inter-site, (c) intra-&inter-site

In this subsection, for 3.5GHz, we have the following evaluation for coherent joint transmission to show its potential gains in terms of ensuring overall coverage and cell-edge performance, through improving spatial multiplexing gain and mitigating inter-UE interference in MU-MIMO, besides having the above-mentioned ultra-massive MIMO. One example can be found in the Figure 6.

- In Rel-18 CJT-CSI evaluation, per sTRP TypeII vs CJT-TypeII CSI, it can be observed that there is ~15% average UPT gain and ~40% cell-edge gain, as shown in the Figure 3, and then we can further observe some more performance gain (e.g., ~100% gains for cell-edge UE), if being based on TDD channel reciprocity.

Figure 6 UPT performance comparison based on single TRP operation and multi-TRP CJT operation

Currently, techniques applied to MIMO systems are basically based on the assumption that uplink (UL) and downlink (DL) services are provided by the same physical serving site (e.g., base station, network node, access point, etc.), however challenges appear if such DL and UL serving site bundling architecture is deployed in the D-MIMO system.

- Firstly, from the perspective of deployment cost of serving sites, due to cost of downlink transmission module of a site is normally higher than that of the uplink reception module, deploying a site supporting both uplink and downlink in case of only uplink capacity is restricted definitely leads to a notable waste on the deployment cost.

- Secondly, in Heterogeneous Network (HetNet) scenario where macro base stations and micro nodes are deployed, the best serving site providing downlink services to a UE can be different from that providing uplink services when the optimal site selection scheme is applied to downlink and uplink individually. Still using DL and UL serving site bundling architecture in such case may lead to performance degradation.

- Finally, besides DL transmissions of control and data messages specific to UEs, network site needs to transmit downlink reference signals (DL-RS) periodically by broadcast, hence network energy consumption is mainly contributed by downlink transmission modules. For the sake of network energy saving or dynamic network operation optimization, downlink transmission function can be reduced, even fully turned off, for some network sites when downlink load in the whole network is not heavy. Nevertheless, the traditional DL and UL serving site bundling architecture has to shut down the uplink reception function for these network sites as well, in other words, these sites are totally shut down. In such way, the number of available spatial resources for transmissions are reduced, the uplink coverage insufficiency issue arises again, and the network performance is degraded.

Then, we have the following deployment scenarios for UL-TRP (DL and UL site-decoupling) as shown in the Figure 7. In short, DL reception can be cut down or even turned off in micro nodes from UE’s perspective, owing to the limited DL coverage and network energy reducing in micro nodes.

Figure 7	Asymmetric DL sTRP/ UL mTRP scenarios

The evaluation results can be found in Figure 8. It can be observed that, compared with legacy site selection (i.e., based on DL-RSRP), introducing DL/UL-site decoupling in HetNet (i.e., using UL TRP scheme(s)) can bring +31% cell-edge and +21.9% average UPT gain, through identifying optimal UL serving sites and balancing UL traffic payload among macro/micro serving sites. Besides, some other evaluation for introducing UL TRP as an individual deployment can be found in our previous plenary contribution [2].

Figure 8	UL performance of asymmetric DL sTRP/UL mTRP scenario in HetNet: (left) 5% and average UPT comparison; (right) the number of serving UL-UEs for each of macro/micro nodes. Where ISD=200m, UE {4T, 23dBm}, Macro {32R, 44dBm for DL-Tx}, Micro {8R*3 site, 33dBm for DL Tx}, P0-80, FTP-1 RU~30%, and Δ_Macro=6 dB

Proposal 2: To improve transmission performance, especially for cell-edge performance, besides for legacy single-TRP operation, multi-TRP operation should be considered for 6G deployment. To reduce deployment cost, ensure UL coverage and save NW energy cost, the new architecture of DL-TRP and UL-TRP decoupling, i.e., asymmetric DL sTRP/ UL mTRP

- Case #1: On the deployment scenarios of indoor hotspot, dense urban, rural, urban macro, regarding TRP operation, a mixed mode of {DL+UL TRP, UL-only TRP} should be supported.

- Case #2: On the deployment scenarios of dense urban (two-layer), micro layer: UL-only TRP should be additionally provided, besides for DL+UL-TRP.

For instance, based on above analysis on multi-TRP operation, we have the following modification in red on the top of indoor hotspot/dense urban, rural, urban macro as in TR 38.913:

| TRP operation mode | Around 4GHz and 7GHz: single TRP operation or coherent joint transmission Around 30GHz or Around 70GHz: single TRP operation or non-coherent joint transmission Note: TRP(s) can be all DL+UL TRP, or randomly selected from {DL+UL TRP, UL-only TRP}. |
| --- | --- |

Then, for dense urban (two-layer), we have the following recommendation:

| TRP operation mode | Macro layer: DL+UL TRP Micro layer: UL-only TRP Around 4GHz and 7GHz: single TRP operation or coherent joint transmission Around 30GHz or Around 70GHz: single TRP operation or non-coherent joint transmission |
| --- | --- |

2.2 Multi-carrier based deployment scenario

In some scenarios, compensating for the significant coverage gap between 3.5 GHz and 7 GHz via single-carrier operation can be highly challenging, especially for UL because of limited UE transmission power. Moreover, some single-carrier-based enhancements may remain optional, meaning 6G deployments in Day 1 cannot rely on these non-mandatory features. For operators with access to multiple spectrum bands, a practical solution is to leverage flexible spectrum utilization—combining high-frequency bands for capacity for cell-center UEs and low-frequency bands for coverage for cell-edge UEs.

In Figure 9, it presents a comparison of ‘UE distance -UL throughput’ among different frequency bands. It can be observed that the UE at lower frequency band typically have better UL throughput when the UE distance to base station is larger than 100 meters.

Figure 9 Comparison of ‘UE distance - UL throughput’ in different frequency bands

In 5G, carrier aggregation (CA) is supported to aggregate different frequency carriers for a UE. In TR 38.913, multi-carrier based deployment scenario is supported, e.g., in dense urban scenario.

Table 6.1.2-1: Attributes for dense urban Attributes; Values or assumptions. Carrier Frequency NOTE1; Around 4GHz + Around 30GHz (two layers). Aggregated system bandwidth NOTE2; Around 30GHz: Up to1GHz (DL+UL) Around 4GHz: Up to 200MHz (DL+UL). Layout; Two layers: Het - Macro layer: Hex. Grid - Micro layer: Random drop Step 1 NOTE3: Around 4GHz in Macro layer Step 2 NOTE3: Both Around 4GHz & Around 30GHz may be available in Macro & Micro layers (including 1 macro layer, macro cell only). …; ….

In 5G, CA is an optional feature requiring a UE to simultaneously receive and transmit among the multiple aggregated carriers. This leads to CA is not widely commercialized in early 5G deployment. In 6G, more flexible spectrum utilization meanwhile with minimal UE complexity should be considered, e.g., carrier-switch based operation. Therefore, multi-carrier based deployment scenario should be supported for 6G with taking flexible spectrum utilization and minimal UE complexity into account.

Proposal 3: To achieve comparable UL coverage and better capacity in 6G deployment scenarios, especially around 7GHz, support multi-carrier based deployment scenario for 6G with taking flexible spectrum utilization and minimal UE complexity into account.

- Conclusion

In this contribution, we elaborate our general views on 6G deployment scenarios, based on the analysis from coverage and cell-edge performance perspective. We have the following proposals:

Proposal 1: To ensure overall coverage, focus on cell-edge performance and UL coverage (especially for co-site deployment of around 7GHz), large antenna array with more than 1024 BS antenna elements (e.g. up to 1536 or 2048 antenna elements) and 8 or more Tx and Rx UE antenna elements (e.g., up to 16) should be considered.

- Indoor hotspot

- Dense urban

- Rural

- Urban Macro

Proposal 2: To improve transmission performance, especially for cell-edge performance, besides for legacy single-TRP operation, multi-TRP operation with coherent join transmission  should be considered for 6G deployment.  To reduce deployment cost, ensure UL coverage and save NW energy cost, the new architecture of DL-TRP and UL-TRP decoupling, i.e., asymmetric DL sTRP/ UL mTRP

- Case #1: On the deployment scenarios of indoor hotspot, dense urban, rural, urban macro, regarding TRP operation, a mixed mode of {DL+UL TRP, UL-only TRP} should be supported.

- Case #2: On the deployment scenarios of dense urban (two-layer), ‘macro layer: DL+UL TRP; micro layer: UL-only TRP” should be additionally provided, besides for DL+UL-TRP.

Proposal 3: To achieve comparable UL coverage and better capacity in 6G deployment scenarios, especially around 7GHz, support multi-carrier based deployment scenario for 6G with taking flexible spectrum utilization and minimal UE complexity into account.

- References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements CMCC, Verizon, NTT DOCOMO, INC., 	Deutsche Telekom

- RP-232152, Views on Rel19 MIMO evolution, ZTE

- RP-251742 “Study on 6G scenarios and requirements - ISAC”, ZTE, Sanechips

- RP-251741, “Study on 6G scenarios and requirements - AI/ML part” ZTE Corporation, China Unicom, Pengcheng Laboratory, Sanechips

- RP-251745, “Energy-efficient network deployment scenarios in 6G”, ZTE Corporation, China Telecom, China Unicom, KT Corp., Sanechips, Shanghai Jiao Tong University, Queens University Belfast

- RP-251744, “Views on 6G NTN scenarios”, ZTE, Sanechips

- Appendix

Simulation assumption for multi-antenna schemes can be found in the following Table 5-1/2/3.

Table 5-1 SLS evaluation assumption for ultra-massive MIMO at 3.5 GHz and 7 GHz co-site deployment

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5GHz and 7GHz |
| Channel Model | According to the TR 38.901 3D Umi, ISD=200m |
| Antenna setup and port layouts at gNB | - 32 ports: (M, N, P, Mg, Ng, Mp, Np) =(12,4,2,1,1,4,4) - 128 ports: (M, N, P, Mg, Ng, Mp, Np) =(32,16,2,1,1,4,16) - 256 ports: (M, N, P, Mg, Ng, Mp, Np) =(32,16,2,1,1,8,16) (dH, dV) = (0.5, 0.8) |
| Antenna setup and port layouts at UE | - 4 Rx/Tx: (M, N, P, Mg, Ng, Mp, Np) =(1,2,2,1,1,1,2) - 8 Rx/Tx: (M, N, P, Mg, Ng, Mp, Np) =(1,2,2,1,1,1,2) (dH, dV) = (0.5, 0.5) |
| Modulation | Up to 256QAM |
| gNB Tx power | 44 dBm |
| UE Tx power | 23dBm |
| gNB antenna height | 25 m |
| gNB receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| Bandwidth | 20MHz, 52RBs, 8RBs/subband |
| UE receiver | MMSE-IRC |
| Network Layout | 72 UEs per sector (in a total of 21 sectors) |
| PMI/CQI feedback | SVD codebook Subband feedback |
| UE distribution | 20% outdoor (30km/h) |
| Traffic model | FTP |
| CSI feedback delay | 5ms |
| MIMO scheme | 4MU-MIMO with rank adaptation Maximum rank = 4 per UE |
| Performance metrics | 5%-ile, 95%-ile and Average UPT |

Table 5-2 SLS evaluation assumption for CJT-mTRP deployment scenario

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2GHz |
| Channel Model | According to the TR 38.901 3D UMa |
| Scenario | Dense urban |
| Antenna setup and port layouts at gNB | - 16 ports: (M, N, P, Mg, Ng, Mp, Np) =(6,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (M, N, P, Mg, Ng, Mp, Np) =(12,4,2,1,1,4,4) (dH, dV) = (0.5, 0.8) λ |
| Antenna setup and port layouts at UE | (M, N, P, Mg, Ng, Mp, Np) =(1,2,2,1,1,1,2) (dH, dV) = (0.5, 0.5) |
| Modulation | Up to 256QAM |
| BS Tx power | 46 dBm |
| UE Tx power | 23dBm |
| BS antenna height | 25 m |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Numerology | 14 OFDM symbol slot, 30kHz SCS |
| Bandwidth | 20MHz |
| UE receiver | MMSE-IRC |
| Network Layout | 20 UEs per cell (in a total of 21 cells) |
| PMI/CQI feedback | Subband |
| Traffic model | FTP-1 |
| MIMO scheme | SU/MU-MIMO with rank adaptation Maximum rank = 4 per UE SU = 30%, MU = 50~70% |
| Performance metrics | Average UPT and cell-edge/95%-ile UPT |

Table 5-3 	System-level simulation parameters for asymmetric DL sTRP/UL mTRP scenario

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4GHz |
| Bandwidth | 20MHz |
| SCS | 30kHz |
| Network Layout | Outdoor FWA (TR 38.901): UMa, 2-layer (HetNet layout) |
| Macro BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1; 4,4). (dH,dV) = (0.5, 0.8)λ |
| Micro BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (2,2,2,1,1; 2,2). (dH,dV) = (0.5, 0.8)λ |
| UE antenna configuration | 8Tx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2). (dH,dV) = (0.5, 0.5)λ |
| Scheduling | Subband PF |
| Traffic model | fullbuffer |
| ISD | 500 m |
| UE layout | 100% Outdoor |
| UE speed | 3km/h |
| Number of average UE per macro cell | 20 |
| Max UE power | 23 dBm |
| UL power control | Open loop TPC |
| Power control parameter | P0 = -60, Alpha = 0.8 |
| Modulation | Up to 64QAM |
| Layer | MCS and RANK (1-to-4) adaptation |