3GPP TSG RAN Meeting #108                                                          	RP-251745

Prague, Czech Republic, June 9-13, 2025

Source:	ZTE Corporation, China Telecom, China Unicom, KT Corp., Sanechips, Shanghai Jiao Tong University, Queens University Belfast

Title:	Energy-efficient network deployment scenarios in 6G

Agenda Item:	16.1

Document for:   Discussion and Decision

- Introduction

In RAN#107 meeting, the new 6G RAN SID was updated with the detailed objective [1], where the identification of “typical and practical deployment scenarios” is one of the critical aspects. In addition, sustainability is one of the overarching aspects considered in the ITU-R IMT-2030 Framework[2].

In this contribution, the views on energy-efficient network deployment scenarios are provided to address the need of building more sustainable networks in 6G. We particularly focus on the scenarios with passive nodes which provide benefits on both coverage and energy efficiency.

- Discussion on energy-efficient network deployment scenarios

As highlighted in the SID[1], the following aspects are considered as part of the important requirements to introduce “substantial gains” for 6G:

- Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage

This motivates the development of a network that can serve all UEs in various conditions (e.g., channel condition, frequency) with satisfied performance and energy efficiency. Densification (e.g., deploying small cells) with multi-layer heterogeneous networks is proven to provide enhanced spectral efficiency and coverage. However, there are practical factors (e.g. site acquisition with power supply, cost including power cost and deployment cost) to consider for realizing network densification. It’s important in 6G to identify more practical and energy-efficient manners to achieve enhanced spectral efficiency and coverage. This potentially can be achieved by deploying more sustainable network nodes including passive/semi-passive nodes, very lower power nodes without PA, nodes with energy harvesting capability. Hence, it is proposed to have a dedicated sub-section under the section of deployment scenarios of TR 38.914[3] to capture all the potential energy-efficient network deployment scenarios in 6G.

Proposal 1: Introduce a dedicated sub-section under the section of deployment scenarios in TR 38.914 to capture all the potential energy-efficient network deployment scenarios in 6G.

In the subsequent part of this contribution, we focus on the multi-layer heterogeneous network deployment scenarios with passive/semi-passive nodes.

2.1 Multi-layer heterogeneous network deployment scenarios with energy-efficient nodes

Among all the energy-efficient nodes, reconfigurable intelligent surfaces(RIS) and network-controlled repeaters(NCR) are considered as potential types of nodes for the future trend of multi-layer heterogeneous network deployment. Especially, in the past years, RIS is considered as the promising enabler to construct the smart propagation with decent progress in R&D, e.g., IMT-2030(6G)[4], RISTA alliance[5] and ETSI RIS ISG[6]. As an assisting node, RIS is considered to tune the incident wireless signals through various mechanism, e.g., reflection, refraction, absorption, with either controllable (i.e., the adjustable response of RIS on the incident wave based on the received signalling) or fixed operation in the deployment. Additionally, the study and specification works in 5G-A Rel-18 for NCR can also be considered as the stepping stone to enable RIS deployment in 6G [7].

Based on the all the progress, as the typical example shown in Figure 1, the deployment with RIS can be flexibly (e.g., on-demand) integrated in the traditional scenarios (e.g., UMa, UMi, InH, etc) to improve the service quality for all services.

Figure 1 Illustration of the outdoor scenarios with RIS

2.2 Preliminary discussion on the performance of networks with passive nodes

2.2.1 Pre-commercial trial results

The performance of the RIS-assisted deployment has been verified with a lot of prototyping and trials including the real-field scenarios [5][9][10][11]. For example,

- For the outdoor scenario, as shown in Figure 2, after deployment of RIS, the RSRP (Reference Signal Received Power) is improved by 24 dB. The downlink and uplink peak rate are increased from 119 Mbps to 1.89 Gbps, and 2.2 Mbps to 248 Mbps, respectively.

Figure 2 Illustration of the outdoor scenario verification (URL)

In another outdoor case (URL), with introduction of the controllable RIS, the maximum RSRP is increased by 23 dB, and the user rate is increased by five times. Additionally, the scenarios with both single and multi-RISs for multi-UEs cases are also verified in URL and URL.With coordination of multiple RISs, the significant improvement of the RSRP and throughput is observed.

- For the indoor scenario, as shown in Figure 3, the RSRP and SINR can be increased by about 20dB, and the cell edge throughput can be increased by 121%~280% after RIS is deployed. In a Sub-6GHz network, the RSRP can be increased by about 6 dB after RIS is deployed.

Figure 3 Illustration of the indoor scenario verification (URL)

Additionally, for another indoor case, as shown in the Figure 4, when RIS is deployed, the RSRP, SINR, and uplink and downlink throughput of the six points have been greatly improved, with the RSRP at the far point being increased by up to 22 dB, and the near point also having a gain of nearly 10 dB. In one specific beam, the user uplink and downlink rates are increased by about 130% and 1050%, respectively.

Figure 4 Test environment in the Hangzhou Asian Games velodrome

Observation 1: Network performance, e.g., overall coverage, cell-edge UE performance, can be significantly improved with the deployment of the passive node, e.g., RIS.

2.2.2 Simulation results for energy efficiency

In addition to the improvement of performance, comparing with other approaches, e.g., repeater, relay, due to negligible power consumption, the deployment of RIS, especially multi-RISs, can be considered as the efficient way to boost the benefits of MIMO. To assess the energy efficiency gains of the multi-layer deployment, the simulation based on the assumption listed in [12] is conducted with the results shown in Figure 5, i.e., performance of both systems in terms of power usage and energy efficiency with respect to achievable spectral efficiency. It can be found that with deployment of RIS, overall energy efficiency is improved.

- Power consumption                       b) Energy efficiency

Figure 5 Comparison of Power consumption and Energy efficiency for the cases with/without RIS

The simulations are repeated with different BS antenna configurations, different BS transmit power as well as different distribution of RIS, to show that the increase in energy efficiency introduced by passive nodes can be achieved in a wide range of deployment scenarios.

Observation 2: Network energy efficiency can be improved with deployment of passive nodes e.g., RIS.

Proposal 2: The multi-layer deployment scenarios with passive nodes e.g., RIS, should be captured as one of the typical and practical energy-efficient network deployment scenarios in TR 38.914.

- Conclusion

In this contribution, we provide our views on energy-efficient network deployment scenarios in 6G. Particularly, we focus on the multi-layer deployment scenarios with performance analysis. The following observations and proposals are made:

Observation 1: Network performance, e.g., overall coverage, cell-edge UE performance, can be significantly improved with the deployment of the passive node, e.g., RIS.

Observation 2: Network energy efficiency can be improved with deployment of passive nodes e.g., RIS.

Proposal 1: Introduce a dedicated sub-section under the section of deployment scenarios in TR 38.914 to capture all the potential energy-efficient network deployment scenarios in 6G.

Proposal 2: The multi-layer deployment scenarios with passive nodes e.g., RIS, should be captured as one of the typical and practical energy-efficient network deployment scenarios in TR 38.914.

- References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements CMCC, Verizon, NTT DOCOMO, INC., 	Deutsche Telekom.

- ITU-R M.2160 Framework and overall objectives of the future development of IMT for 2030 and beyond, URL: https://www.itu.int/rec/R-REC-M.2160/en.

- TR 38.914 Study on 6G Scenarios and requirements.

- Technology report of IMT 2030(6G) promotion group on RIS, URL: https://www.imt2030.org.cn/html/default/en/Publications/Report/index.html?index=2.

- White paper for RIS, URL: https://www.risalliance.com/en/white-paper/.

- RP-250033 LS on ETSI ISG RIS GRs, ETSI ISG RIS.

- RP-232884 WI summary for NR network-controlled repeaters.

- RP-250809 Proposal for structure for 6G RAN SI skeleton TR, CMCC.

- R. Liu, J. Dou, P. Li, J. Wu, and Y. Cui, “Simulation and field trial results of reconfigurable intelligent surfaces in 5G networks,” IEEE Access, vol. 10, pp. 122 786–122 795, 2022.

- RWS-230292 Views on NCR and RIS in Rel-19.

- RP‑232276 Demonstration of RIS and ISAC techniques in Hangzhou Asian Games, China Telecom, ZTE.

- NETWORK ENERGY EFFICIENCY PHASE 2, NGMN Alliance, URL: https://www.ngmn.org/wp-content/uploads/NGMN_Network_Engergy_Efficiency_Phase2.pdf.