3GPP TSG RAN WG meeting #108                                          RP-251747

Prague, Czech Republic, June 9-13, 2025

# Title: 	6G requirements related to higher layer aspects

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	16.1

# Document for:	Discussion and decision

# Introduction

In RAN#107, the 6G RAN-level SID has been updated in RP-250810[1] with the addition of objectives related to 3GPP requirements, deployment scenarios as shown below:

This study item aims to … - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - … - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - … - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

In this paper, we discuss the general requirements for RAN design for higher layer protocol aspects.

# Control plane aspects

## General requirements for RAN protocol architecture

The following general design principles and targets shall be kept in mind for 6G protocol design:

- The overall protocol and RAN architecture shall enable C-plane / U-plane separation and shall allow independent evolution of CN and RAN

- The RAN level study should consider network architecture both at RAN and CN taking into account the possibility for split architecture, distributed vs centralized RAN and CN including service-based interfaces and their impacts to RRC state machine and UE context management

- The overall protocol design should enable power saving and other CAPEX/OPEX reduction related features for networks such that these features are supported from day1 both on UE and network side

- In addition to the traditional service classes defined in 5G (e.g. interactive, background, streaming and conversational), the 6G RAN protocol design shall allow support service classes for immersive services and AI related traffic

Based on the above considerations, we propose the following:

Proposal 1: The following high-level design principles and targets shall be captured in the RAN-level TR38.914 for 6G study:

- The overall protocol and RAN architecture shall enable C-plane / U-plane separation and shall allow independent evolution of CN and RAN

- The RAN level study should consider network architecture both at RAN and CN taking into account the possibility for split architecture, distributed vs centralized RAN and CN including service-based interfaces and their impacts to RRC state machine and UE context management

- The overall protocol design should enable power saving and other CAPEX/OPEX reduction related features for networks such that these features are supported from day1 both on UE and network side

- In addition to the traditional service classes defined in 5G (e.g. interactive, background, streaming and conversational), the 6G RAN protocol design shall support service classes for immersive services and AI related traffic

## Radio resource control and CP latency

6G should support a battery efficient state for UEs and an active state. The RRC state machine and the overall control plane design should target to satisfy the CP latency requirements in in ITU keeping in mind the latency vs power saving benefits both for devices and the network. The requirements should encompass both TN and NTN networks for the control plane.

Proposal 2: The 6G control plane design should satisfy the ITU requirements for transition from a battery efficient state to an active state keeping in mind, the requirements for both TN and NTN networks and should keep in mind the latency vs power saving benefits for both networks and devices

## Mobility interruption time

In general, basic mobility comes in two flavours: UE controlled and network controlled. On top, a lot of mobility enhancement related work was done in successive releases in 5G and this resulted in a host of features such as LTM, CLTM, CHO, etc. In 6G the mobility framework should consider all these features and main focus should be on a unified and simplified framework for mobility. The decision for this depends partly also on the RAN level security framework and also RAN architecture.

Furthermore, one of the key considerations for the mobility framework is the overall design for spectrum aggregation (including support for CA and/or DC etc) and the general design to minimize the throughput fluctuation during the handover in case CA/DC is configured. Furthermore, mobility decisions should consider both TN and NTN systems. The target of 0 ms interruption time should be achievable in a simplified realistic mobility framework for 6G for both TN and NTN.

Proposal 3: The mobility interruption time close to 0 ms in 6G should be targeted in typical scenarios, keeping in mind a unified mobility framework for network-controlled mobility and should take into account both TN and NTN.

# User plane aspects

The overall UP protocol stack has largely remained unchanged since 3G. The protocol architecture in 5G was designed to support a variety of applications and verticals. To gain a better understanding of the overall requirements for 6G UP, the functional requirements and design targets for UP should be first identified. The UP-latency targets for 6G should take into account support of various verticals including smart manufacturing, XR. applications including remote/telepresence scenarios, AI applications, TN and NTN networks.

Furthermore, the reliability targets for user plane should encompass applications to support HRLLC use cases including autonomous driving, remote surgery and immersive communications

Proposal 4: The UP latency and reliability targets for 6G should take into account support for various verticals and HRLLC applications including autonomous driving and should consider both TN and NTN use cases.  The design should be hardware friendly, should minimize functional redundancy and overhead across protocol layers and ensure forward compatibility

# Security requirements

Overall security framework at RAN level strongly depends again on SA study and the outcome of this study. At RAN level, security for both UP and CP needs to be considered and lower layer security is important for 6G. Compared to 5G, it is likely that the main differences in security framework for 6G may come from the requirement to support lower layer security and also to support secure system information. These aspects need to be considered along with SA decisions for overall security framework and should be studied in conjunction with overall RAN/Core network architecture.

Proposal 5: RAN security requirements for 6G should include lower layer security and secure system information broadcast taking into consideration the overall network architecture in RAN and core

# Design targets for UE capabilities

In 5G, the design of UE capabilities offers excessive flexibility, leading to considerable complexity in both standardization and implementation. For instance, almost all features introduced after Release 15 are defined as optional, and the framework based on band combinations and feature set combinations results in significant signaling overhead and further complexity.

In 6G, it is crucial to strike a balance between flexibility and complexity in the design of the UE capability framework. Additionally, since IoT is expected to be supported from day one, a UE category-based framework can be considered to define the minimal capability requirements for different types of devices (e.g., smartphones, wearable devices, IoT, etc.).

Proposal 6: The RAN level study should capture a requirement for a balance between flexibility and complexity for the design of the UE capability framework, for instance, using a UE category-based framework to establish the minimal capability requirements for various types of devices (e.g., smartphones, wearable devices, IoT, etc.)

# Conclusion

In this contribution, we propose some design requirements/principles related higher layer aspects which can be considered for TR38.914 e.g. under the section of requirements for architecture.  The following proposals are made in this contribution:

Proposal 1: The following high-level design principles and targets shall be captured in the RAN TR38.914 for 6G study:

- The overall protocol and RAN architecture shall enable C-plane / U-plane separation and shall allow independent evolution of CN and RAN

- The RAN level study should consider network architecture both at RAN and CN taking into account the possibility for split architecture, distributed vs centralized RAN and CN including service-based interfaces and their impacts to RRC state machine and UE context management

- The overall protocol design should enable power saving and other CAPEX/OPEX reduction related features for networks such that these features are supported from day1 both on UE and network side

- In addition to the service classes defined in 5G (e.g. interactive, background, streaming and conversational), the 6G RAN protocol design shall support service classes for immersive services and AI related traffic

Proposal 2: The 6G control plane design should satisfy the ITU requirements for transition from a battery efficient state to an active state keeping in mind, the requirements for both TN and NTN networks and should keep in mind the latency vs power saving benefits for both networks and devices

Proposal 3: The mobility interruption time close to 0 ms in 6G should be targeted in typical scenarios, keeping in mind a unified mobility framework for network-controlled mobility and should take into account both TN and NTN.

Proposal 4: The UP latency and reliability targets for 6G should take into account support for various verticals and HRLLC applications including autonomous driving and should consider both TN and NTN use cases

Proposal 5: RAN security requirements for 6G should include lower layer security and secure system information broadcast taking into consideration the overall network architecture in RAN and core

Proposal 6: The RAN level study should capture a requirement for a balance between flexibility and complexity for the design of the UE capability framework, for instance, using a UE category-based framework to establish the minimal capability requirements for various types of devices (e.g., smartphones, wearable devices, IoT, etc.)

# References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements CMCC, Verizon, NTT DOCOMO, INC., 	Deutsche Telekom