3GPP TSG-RAN WG Meeting #108	                                          RP-251768

Prague, Czech Republic, 9 – 12, Jun, 2025

Source: 	Xiaomi

Title:  	Views on R20 AI/ML for Mobility

Agenda Item:	15.2.1

Document for:	Discussion and Decision

Introduction

AI/ML for mobility is studied by RAN2 in R19. In this contribution, we discuss the scope of R20 AI/ML for mobility.

Discussion

In RAN#107, following scope was endorsed for R20 AI mobility,

Only the following objectives are considered as baseline: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction and Frequency domain prediction. FFS Spatial domain pending RAN2 Study progress. FFS Cell level vs beam level prediction NOTE: network sided models can be concluded in RAN#108 after RAN2 discussions - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for measurement event prediction - For both RRM measurement prediction and measurement event prediction [RAN2]: - Specify signalling and protocol to facilitate functionality-based LCM procedures - To be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible - For both RRM measurement prediction and measurement event prediction: [RAN4] - Specify core requirements identified in study item phase - Specify LCM-related requirements, if any

It’s FFS whether spatial domain prediction is supported. In RAN2#129, RAN2 has agreed that the temporal and frequency prediction will be considered as higher priority as following.

- For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

In RAN2#130, Although RAN2 agreed to study the spec impact of spatial domain prediction, the scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G as following,

- The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). - The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

RAN1 only discussed and concluded intra cell beam prediction. Therefore, it could be possible to extend it for intra-cell RRM spatial domain prediction. But there is no any discussion about the feasibility of inter-cell RRM spatial domain prediction. Considering RAN2 has agreed not consider spatial domain prediction for inter-cell scenario, and intra cell RRM spatial domain prediction can be further studied, therefore, in Rel-20 scope, RRM prediction on spatial domain shall be limited to intra-cell prediction.

Proposal 1: RRM prediction on spatial domain and L3 beam prediction is limited to intra-cell prediction.

Regarding the Note Network sided models can be concluded in RAN#108 after RAN2 discussions, there is no consensus in RAN2.

For RRM prediction of temporal domain case B, the goal is to reduce measurement by skipped measurement in temporal domain. We understand that only UE sided model is useful in this scenario. Because UE may still need the predicted measurement result on the skipped time instance. Some companies commented NW sided model can be used to reduce the RS signalling. However, we believe this is not feasible. Unlike BM, RS for RRM is cell specific rather than UE specific. Only if all the UEs in the serving cell and neighbour cells are capable to predict the cell level measurement results in the same pattern, the RS signalling can be reduced. If there is one UE cannot support AI/ML or not support the same prediction pattern, cell-specific RRM RS can’t be reduced. So, we believe it’s impossible for NW to reduce RS signalling by NW side model in temporal domain case B.

Observation 1: NW-side model in temporal domain case B can neither reduce measurement nor reduce RS signalling.

Proposal 2: NW sided model is not supported in temporal domain case B.

Depending on mode input, three sub use cases are supported for RRM prediction according to TR [1] as following,

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The inference input of sub use case 1, and 3 are L1 beam measurement results. In RAN2#130, RAN2 discussed the issue, but it’s still FFS on whether to support sub-case 1 and 3.

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

Note the L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, which is different from the beam for beam management. Current CSI report framework doesn’t support reporting L1 beam measurement result for RRM purpose. Furthermore, RRM prediction may require neighbor cell’s L1 beam measurement result as model input. But current CSI report framework only support reporting serving cell’s L1 beam measurement result.

It’s up to UE how to measure the RRM L1 beam within the required latency, i.e. by what periodicity filtering. NW side model may require L1 beam measurement with fixed periodicity as input. It’s new requirement for UE measurement, which is impossible to apply from UE point of view. Furthermore, it’s up to UE how to perform the L1 filtering. UEs may apply different L1 filtering method. NW is not aware which L1 filtering is applied by UE. The inference accuracy may be poor without such information.

As discussed above, new report signalling is required to report L1 beam measurement result. New requirement on UE measurement is needed to align with the model input interval. UE may also need to report how L1 filtering is done. The spec impact is huge to support L1 beam as model input, i.e. sub-case 1 or 3, for NW sided model.

Observation 2: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1 and 3, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Therefore, we propose to only support sub-case 2, i.e. L3 cell level measurement results as model input, which is used for NW sided model in RRM prediction.

Proposal 3: L1 beam measurement result as model input is not supported for NW sided model in RRM prediction and L3 beam prediction.

Due to time constraints, RAN4 has not reached a conclusion on testability and interoperability aspects in this study item, with several issues remaining unresolved, including but not limited to prediction consistency in the time domain, generalization capability, LCM-related testability concerns, and test goal definition. These aspects require further investigation by RAN4. Additionally, the previously endorsed agreement lacked performance requirements and corresponding test cases. To address these critical gaps, it is recommended to refine the RAN4 study objectives accordingly.

Proposal 4: The objectives for RAN4 can be updated as following:

- Core requirements for both RRM measurement prediction and measurement event prediction [RAN4]:

  - Evaluate RAN4 testability and interoperability for AI/ML based mobility case

  - Specify core requirements identified in study item phase

  - Specify LCM-related requirements, if any

- Performance requirements and test cases for both RRM measurement prediction and measurement event prediction [RAN4]

Conclusions

According to the discussion above, we have the following proposals:

Proposal 1: RRM prediction on spatial domain and L3 beam prediction is limited to intra-cell prediction.

Observation 1: NW-side model in temporal domain case B can neither reduce measurement nor reduce RS signalling.

Proposal 2: NW sided model is not supported in temporal domain case B.

Observation 2: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1, 3, 4 and 6, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Proposal 3: L1 beam measurement result as model input is not supported for NW sided model in RRM prediction and L3 beam prediction.

Proposal 4: The objectives for RAN4 can be updated as following:

- Core requirements for both RRM measurement prediction and measurement event prediction [RAN4]:

  - Evaluate RAN4 testability and interoperability for AI/ML based mobility case

  - Specify core requirements identified in study item phase

  - Specify LCM-related requirements, if any

- Performance requirements and test cases for both RRM measurement prediction and measurement event prediction [RAN4]

Reference

[1] TR 38.744