3GPP TSG RAN Meeting #108				RP-251769

Prague, Czech, June 9-13, 2025

Source:	OPPO

Title:	Discussion on 6G RAN WG Study

Agenda Item:	16.2

Document for:	Discussion

Introduction

Targeted as a new generation of mobile communication and information system, 6G needs to meet the requirements of the evolving social trends and needs of end users. A few examples of general design goals are listed:

Goal 1: to provide substantial value to users by fostering new value-adding services:

- to create new distinguished intelligent services utilizing new capabilities like AI, data, sensing, etc.

- to grant service participants access to new capabilities through service interfaces and APIs

Goal 2: to reduce total cost of ownership (TCO) of both network and UE by adopting high-efficient, sustainable and extensible technologies to:

- substantially reduce the cost and complexity of migration from earlier generations to with minimum number of options

- adaptively support diverse service requirements with minimized cost & complexity of both network and UE

Goal 3: to support IMT-2030 and other advanced usage scenarios in a future-proof manner

- essential scenarios are addressed with Day One features, while other features/functionalities may be added in later phases according to market demands, without introducing complexities due to backward/forward compatibility and inter-dependencies

The Radio Access Network (6G RAN) will continue to play a fundamental role in the 6G era. High-level capabilities of 6G radio technologies and suggestions on 6G RAN WG level study objectives are discussed in later parts of this document.

Capabilities of 6G radio technologies

Support various services with high flexibility and low complexity

6G is expected to support a large variety of services ranging from LPWA to the extremely high data-rate and low-latency mobile broadband required by e.g. virtual reality applications. Thus, a system framework supporting high flexibility while at the same time minimizing incurred complexity will be the key for a multi-purpose 6G.

Different services may have diverse requirements in e.g. traffic patterns, propagation conditions and coverage goals. Additionally, life cycles of different services may also mismatch greatly, for example some verticals ask for 20 years of service period, which is longer than the time span of a single generation. Thus a “one for all” system design accommodating all services will introduce unnecessary interdependencies and make the system design very complex. And this problem will become more outstanding with time when more demands come and more features are introduced in the specifications in the later releases. A prohibitively complex set of specifications, where only a small portion of which are actually implemented here and there, will greatly increase the cost and difficulty of system deployment and operation for the whole industry.

Considering the abovementioned scenario, expandable sets of functionalities targeting different aspects of 6G requirements may be a good choice. In the example illustrated below, the common base of the 6G Radio (6GR) is the basic function set, which provides the basic functions of the 6GR for all 6G usage scenarios, such as wide area coverage to support IDLE mode mobility, basic access and control channels, low data rate transmission, and basic QoS (basic latency, reliability performance). At the same time, this basic feature set should be able to serve as an "energy-saving mode" for Immersive Communication (which can be seen as the 6G version of eMBB). Finally, this basic feature set can independently fulfil all requirements of Massive Communication (Low end IoT) usage scenario.

In addition to Massive Communication, the air interfaces for other usage scenarios are modularly scaled up from the basic functionality set to form their own full functionality sets. As the most important usage scenario in 6G, the full functionality set of Immersive Communication (IC) is scaled up from the basic feature set [6].

Figure 1. Example of scalable design for 6G

In sum, the ability to respond to diverse service requirements with “just enough” resources and minimum interdependency between service “tracks” may be the key for a cost effective and profitable 6G system.

High energy efficiency for both network and UEs with high availability

Energy cost has been a very large part of the system OPEX for years. Battery life of smart phones is also essential to end user experience. However, during 5G timeframe, the energy saving mechanisms of the network and UE e.g. NES and LP-WUS were not specified in the first release but are standardized independently from each other in later releases, which leads to the lack of coordination between the network and UE and thus difficulty to achieve jointly optimized results. Late introduction of these features also makes network deployments and UE implementation difficult and fragmented.

6G should develop a holistic, end-to-end energy saving framework from Day One, which means the energy efficiency for both the network and the UE are jointly optimized without sacrificing either side. Thus, a well-defined mechanism of cooperative energy saving between the network and the UE will be essential to address different deployment scenarios varying from hot spots to cell edge to low-covered areas. And this may need to be done at an early phase of the 6G WG level study since many important system components e.g. initial access and mobility procedures are heavily dependent on the energy/power saving schemes.

Built-in support for new intelligent services

New intelligent services are essential to the increase of revenues of operators. One example of new intelligent services is a broad-sense mobile Metaverse, where interactions between the cyber world and the physical world happen on different levels. Another example is mobile AI applications, where AI helps to improve communication performance and enables “endogenous” intelligent services of 6G. Low-altitude economy and autonomous self-driving cars are also hot topics today.

For these new services, higher peak data rate and spectrum efficiency may not always be the deciding factors. New capabilities like compute, sensing and data life cycle management (Data LCM) may be more relevant. The capability to provide rightful capabilities like data and compute in the right place and at the right time would be essential for user experience, which gives an edge to a “distributed” or “layered” system architecture where the acquisition, transportation, storage, processing and consumption of data are somehow “decoupled” and thus could be independently optimized.

One important new aspect emerges together with 6G new services is the security and privacy. In the context of RAN, the lower layers (L1 and L2) may become more relevant where attackers may leverage un-protected radio data to gain access to private information of users e.g. health status and user location/trajectory [5]. This kind of risk does not exist in 5G. Therefore, integrated lower-layer (L1/L2) security and privacy mechanisms may also be desired in 6G.

Smooth migration from earlier generations

Well-deployed 4G and 5G infrastructures and the prevalence of 4G/5G capable smart devices have laid a good foundation for current mobile Internet services. And the smooth and simple migration from these earlier generations will be a key to ensure the quality of existing service while new services are being introduced.

Technologies like advanced spectrum sharing are anticipated to play an important role, and the minimization of migration options, as well as the availability of low complexity UEs, will be critical for the successful large-scale deployment of 6G.

Reduce unnecessary inter-layer action

We noticed that quite a lot of MAC CEs are introduced for PHY layer features e.g. MIMO features. It was believed by RAN1 that MAC CE bears less signaling and processing delay compared to RRC while it is more reliable and scalable compared to DCI. The main problem is that MAC CE is a white box for RAN2 i.e. RAN2 need to design all the signaling details based on requirements from RAN1. Because of this, a stable MAC CE design usually demands several handshakes between RAN1 and RAN2, not mentioning the maintenance job afterwards. In many cases, RAN2 has to design a new MAC CE when one RAN1 feature is updated instead of extending existing MAC CE. Compared to RRC ASN.1, “non-critical extension” is much more difficult for MAC CE design because all the R bits need be reserved from very beginning and it is not always sufficient for new functionality of RAN1 features. For DCI, reliability should not be a concern because otherwise the whole PHY layer doesn’t work well. In addition, some solutions are already on the table e.g. HARQ feedback for DCI. In addition, some of the MAC CE e.g. “SRS Pathloss Reference RS Update” is used to update RRC parameter directly, which causes mess for RRC also. Having said that, we think it is the right time to open the discussion in order to reduce such inter-WG workload.

Suggestions on 6G RAN WG Level Study Objectives

According to the discussion of Section 2, we have the following suggestions on 6G RAN WG level study objectives:

- the study should provide scalability and modularity to support various services with high flexibility and low complexity, and to address different life cycles among services, as well as minimization of interdependencies between Day1 and later features

- the study should aim to provide high energy efficiency for both network and UEs, and cooperative energy/power saving mechanisms are highly desired in the early phase of the study to facilitate related features e.g. initial access and mobility procedures

- the study should provide built-in support for new services beyond connectivity, and to integrate lower-level (L1/L2) security mechanisms to ensure end-to-end security and privacy protection

- the study should support smooth migration from earlier generations and minimization of migration options

- the study should aim at minimization of unnecessary inter-layer actions e.g. using excessive number of MAC CEs for L1 signalling

Conclusion

6G should be able to support a variety of services with high efficiency and low complexity from Day 1, and by introducing an extensible system architecture, 6G will be able to introduce new capabilities in the later phases to respond to emerging market needs, while at the same time by avoiding unnecessary interdependencies, does not interfere with existing services. Cost-effectiveness could also be achieved since only the necessary technologies will be applied according to emerging service demands.

Our proposals for 6G RAN WG level study are listed below:

Proposal 1: the study should provide scalability and modularity to support various services with high flexibility and low complexity, and to address different life cycles among services, as well as minimization of interdependencies between Day1 and later features

Proposal 2:  the study should aim to provide high energy efficiency for both network and UEs, and cooperative energy/power saving mechanisms are highly desired in the early phase of the study to facilitate related features e.g. initial access and mobility procedures

Proposal 3: the study should provide built-in support for new services beyond connectivity, and to integrate lower-level (L1/L2) security mechanisms to ensure end-to-end security and privacy protection

Proposal 4: the study should support smooth migration from earlier generations and minimization of migration options

Proposal 5: the study should aim at minimization of unnecessary inter-layer actions e.g. using excessive number of MAC CEs for L1 signalling

References

- Recommendation ITU-R M.2160-0 (11/2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

- RP-250810, Revised SID: Study on 6G scenarios/use cases and requirements, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

- RP-243276, RAN Chair’s Guidance for 6G Study: ITU Focused, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17), V1.0.0 (2021-12)

- RP-251255, Discussion on security on low layer in 6G RAN, 3GPP TSG RAN Meeting #108, Prague, Czech, June 9-13, 2025

- RP-250974, Discussion on 6G Scenarios and Requirements, 3GPP TSG RAN Meeting #108, Prague, Czech, June 9-13, 2025