3GPP TSG RAN Meeting #108                                                                               RP-251786

Prague, Czech Republic, June 9-13, 2025

Source:	Qualcomm (Moderator)

Title:	Moderator's summary for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

Agenda Item:	15.1.1

Document for:	Discussion and Decision

# Introduction

This document provides a summary of proposals made for RAN#108 on scope for a project on AI/ML for NR air interface enhancements.

The central-theme is the main purpose of the Rel-19 study in FS_NR_AIML_air, namely, on two-sided CSI compression:

- Temporal domain sub use case: Case 0 (recommended), Case 2, Case 3

  - Case 0: current slot CSI, spatial/frequency domain compression without temporal component

  - Case 2: current slot CSI, spatial/frequency domain compression with temporal component at UE and NW

  - Case 3: future slot prediction, spatial/frequency domain compression with temporal component at UE

- Inter-vendor training collaboration with following definitions:

  - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

    - Sub option 4-1: standardized dataset format + dataset exchange

    - Sub option 3a-1: standardized model structure + parameter exchange with or without target CSI sharing

  - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1)

    - Note: This is the Direction that RAN4 has been focusing on for testing

At Chairs’ Release 20 5G-Adv RAN1/RAN2/RAN3/RAN4 Package in RP-250859, the following is indicated for AI/ML NR Air Interface project:

AI/ML Air Interface: Work item - Focus on CSI compression for two-sided model - Further discuss how to handle inter-vendor collaboration - Further discuss and down-select which CSI compression cases to support (e.g. with/without temporal aspects, with prediction, …) - Further discuss how to handle data collection - No new use cases in Release 20, no further work on Rel-19 positioning 2nd priority cases in Release 20

# Background

In this section we summarize a number of relevant RAN1 agreement important for the scoping of the Rel-20 project.

## 2.1	RAN1 recommendations:

Based on the study, RAN1 recommends the following with potential RAN1 specification impacts common to all inter-vendor collaboration options. - Recommend Case 0 - Case 2 and 3 can be considered for further enhancement built upon the specification of case 0. - Recommend following specification impacts for the two-sided CSI compression use case - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI / RI determination, payload determination, quantization configuration / codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note: The framework defined in BM and CSI prediction use cases could be reused. - Specify performance monitoring, if needed

## 2.2	Agreements around inter-vendor collaboration Directions.

For inter-vendor collaboration, RAN1 concludes that both Direction A and Direction C are feasible. For Direction A, RAN1 concludes that the feasible sub-options are sub-option 4-1 and sub-option 3a-1, and in case of sub-option 3a-1, with and without target CSI sharing from NW side. - Performance: Direction C with standardized reference model (structure + parameters) ensures a minimum performance requirement, while Direction A with standardized reference model structure + parameters (sub-option 3a-1), or standardized dataset exchange (sub-option 4-1) from NW-side to UE-side may achieve better performance. - Direction C addresses inter-vendor collaboration complexity: From RAN1 perspective, Direction C (a.k.a., option 1) eliminates the inter-vendor collaboration complexity if it is feasible to standardize the model. The fully standardized reference model (structure + parameters) should be trained using synthetic data. The model structure and parameters are assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. RAN1 concludes that the scalable model structure specification and model parameters specification is feasible, but the conclusion on interoperability and RAN4 testing feasibility is a separate study in RAN4. - For Direction A with sub-option 4-1 and 3a-1, RAN1 concludes that standardized signaling, if feasible and specified, can be used for model parameter / dataset exchange to alleviate / resolve the inter-vendor collaboration complexity. The standardized signaling, if needed, may be over-the-air, or other approaches. For OTA and non-OTA based standardized signaling, as well as the feasibility of standardized signaling, related status have been provided in [R2-2503169]. For sub-option 3a-1, the specified encoder (a.k.a., CSI generation part) structure should be determined using synthetic data, and the scalable encoder structure specification is feasible. For NW-side performance monitoring, RAN1 concludes that target CSI reporting via legacy codebooks can be used. Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side performance monitoring with additional cost of complexity and overhead at UE side. For UE-side side monitoring, RAN1 concludes that it is feasible, but some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports. NW side data collection for training is studied including data format, configuration of rank / layer, number of subbands and mechanisms for ground-truth reporting, but not all aspects are concluded. UE side data collection for training is studied including NW configuration or UE request, configuration for temporal aspects, but not all aspects are concluded. Aspects that were not concluded can be discussed in the normative phase.

Further agreements on inter-vendor collaboration Directions understanding how the specification work could be mutually leveraged:

Based on the study, for addressing inter-vendor collaboration complexity, RAN1 identifies the following specification impacts for supporting sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C - Reference model / structure specification (for sub-option 3a-1 and Direction C) - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. - Note: The model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 can be same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder. - To reduce the workload of the potential normative work scope, the model structure(s) for the reference encoder in inter-vendor collaboration sub-option 3a-1 is(are) same as/equivalent to the structure of the RAN4 defined/specified (if defined/specified for testability) reference encoder. - Note: RAN1 has studied and agreed on a scalable model structure for feasibility study during Rel-19 study, and considers that can serve as a starting point for the model structure specification for the potential normative work. - Format and contents of parameter/dataset/information exchange: dataset (for 4-1 and 3a-1 with target CSI)/ encoder parameter (for 3a-1) / additional information (for 3a-1 and 4-1). - The workload can be reduced by identifying the common part among different aspects.

# Discussion

RAN1 made a number of Recommendations which are taken into account for the drafting of the proposed Rel-20 objectives. Those recommendations relate to the CSI compression sub-use cases to provide normative support on, as well as, recommendations on specification impacts for the two-sided CSI compression use case.

Much of the discussions in RAN1#121 revolved around the Directions facilitating inter-vendor collaboration and how the relate to each other. It is worth noting that  Direction A, sub-option 3a-1 and Direction C share much of the specification work. Direction C is equivalent to RAN4 test models and is meant to offer a minimum performance for the AI/ML model implementation.

It was observed in contributions that AI/ML technology evolves very quickly and with standardized model structure by Option 3a-1 or Direction C, a lot of spec efforts may be required to update the model structure. Also, it appears that more time in SA WGs is needed to study the feasibility of non OTA approach to support Direction A (option 3a-1 and option 4-1).

As a result, further coordination is required with SA WGs and a check point in RAN#109 could be set to confirm feasibility of Direction A (option 3a-1 and option 4-1) inter-vendor collaboration.

# Proposed Objectives

In this section we provide a tentative set of objectives for the Rel-20 of NR air interface.

Provide specification support for the following aspects:

AI/ML general framework for two sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air_Ph2 and NR_AIML_Air projects for CSI feedback enhancement [RAN2]:

- Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model selection, activation, deactivation, switching, fallback

  - ID related signalling is part of the above objective

- Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring,.

CSI feedback enhancement, encompassing (two-sided model) [RAN1/2]:

- CSI spatial/frequency compression without temporal aspects (“Case 0”),

  - Consider specification of Case 2 and Case 3 based on Case 0 completion

- Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including

  - Model pairing procedure including ID and applicability reporting

  - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports

  - NW and UE side data collection for training,

    - Target CSI format

    - Note The framework defined in BM and CSI prediction use cases could be reused

  - Specify performance monitoring if needed

Inter-vendor training collaboration for two-sided AI/ML models [down-selection possible?]:

- Necessary signalling and protocol aspects [RAN2/RAN3]

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized model structure plus parameter exchange (“Direction A, sub-option 3a-1”) levering defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

    - Necessity of with and/or without target CSI sharing needs to be assessed aiming at specifying just one of the two

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Aiming at unifying specification handling for all the Directions

Data collection [RAN2 in close collaboration with SA WGs]:

- [Further study and] specify if deemed necessary, standards-based UE data collection at least for UE-side model training including partial visibility of collected data in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 upon SA WG feedback

Model Transfer [RAN2 in close collaboration with SA WGs]

- Necessary signalling for the transfer and delivery of one-sided and two-sided dataset/model taking into account the principles agreed during the Rel-19 work item and including the possibility to further agree on additional principles deemed necessary.

Interoperability and RRM requirement [RAN4]:

- Specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration option Direction C.

- Necessary RRM requirement for specified LCM procedures.

- NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air .

Performance requirement [RAN 4]:

- Demodulation requirement for CSI compression Case 0.

- RRM performance test cases for the corresponding RRM requirement for LCM procedures.

## 4.1	Other possible objectives which require further discussion

The following potential objectives have been brought up by companies which would require further discussion for inclusion into the Rel-20 scope:

- APU/CPU counting and timeline reporting

- RAN2 left-overs from Rel-19 after RAN2#131, e.g.  enhanced procedures for the configuration and activation of UE- and NW-side data collection, etc.)

- RAN3 involvement in Rel-20 for the work on NG-RAN node assisted positioning with a gNB-side model.

- New use cases in Release 20 for 5G-A, e.g. take one of the following two options:

  - Option 1: Study and specify a limited set of new use cases for AI/ML air interface for 5G-Advanced in Rel-20 5G-A WI, e.g. overlaid DMRS and data with one-sided AI/ML channel estimation

  - Option 2: Study new use cases for AI/ML air interface in Rel-20 6G SI, and specify the applicable use cases for 5G-A in Rel-21 depending on the outcome of the study in Rel-20

# Conclusions

In this contribution we have presented a set of proposed objectives for a Rel-20 work item on AI/ML for NR air interface. While some areas have been explicitly recommended, some areas may require discussion and confirmation. Note that the moderator has tried to come up with a set of objectives that would be relevant for 5G-Advanced itself and also could be leveraged upon towards AI/ML in 6G.

Note that a number of areas require close collaboration with SA WGs:

- Views on inter-vendor collaboration Direction A: check-point suggested for RAN#110

- Continued collaboration to enable standards-based UE data collection at least for UE-side model training: throughout Rel-20

# References

RP‑250864, AI/ML for air interface Rel-20 WI scope, Futurewei

RP‑250875, Discussion on Rel-20 AIML for PHY, Ericsson

RP‑250892, Discussion on AI/ML for air interface in 5G-Advanced Rel-20, Fujitsu Limited

RP‑250968, Qualcomm views on AI/ML for Air Interface WI in Rel-20, Qualcomm Atheros, Inc.

RP‑250987, Views on Rel-20 AI/ML Enhancement, Google

RP‑251021, Views on scope of AI/ML for Air interface, Samsung

RP‑251041, AIML for NR air interface enhancements, NEC

RP‑251194, Discussion on Rel-20 AI/ML air-interface, LG Electronics

RP‑251198, Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Tejas Networks Limited

RP‑251203, Scope of AI/ML based physical layer enhancements in 5G-A Rel-20, vivo

RP‑251220, Views on AIML air interface for Rel-20 5G-Advanced, Spreadtrum, UNISOC

RP‑251242, Discussion on 5G-A AI/ML-based CSI compression in Rel-20, Intel Corporation

RP‑251252, Discussion on AI/ML for NR air interface enhancements work scope in Rel-20, OPPO

RP‑251281, Scope of AI/ML air interface in 5G-A Rel-20, Xiaomi

RP‑251295, Rel-20 AI/ML Radio scope, MediaTek Inc.

RP‑251381, Views on Rel-20 AIML for Air Interface, CMCC

RP‑251403, Scope discussion on CSI compression in AI/ML Air Interface, Panasonic

RP‑251411, Views on Rel-20 5G-A AI/ML for air interface enhancements, NTT DOCOMO, INC.

RP‑251425, Views on AI/ML for NR air interface enhancements,	Lenovo

RP‑251436, Discussions on AIML for NR Air Interface enhancements, CAICT

RP‑251518, Left over items on AIML for positioning accuracy enhancement case 3a, CEWiT

RP‑251621, Views on 5G-A Rel-20 AI/ML for air interface, CATT, CBN, China Broadnet

RP‑251646, Rel-20 AI/ML for NR Air Interface, Huawei, HiSilicon

RP‑251653, Scope for AI/ML Air Interface for Rel-20 5G-Advanced, InterDigital, Inc.

RP‑251659, Views on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Apple

RP‑251700, AT&T views on AI/ML for air interface enhancements, AT&T

RP‑251719, AIML for Air Interface for Rel-20 5G-A, ZTE Corporation, Sanechips

RP‑251749, 5G requirements for Model Transfer and UE sided data collection, T-Mobile USA Inc.

RP‑250874, Discussion on Rel-20 NR MIMO Phase 6, Ericsson