3GPP TSG RAN Meeting #108						RP-251789

Prague, Czech Republic, June 9-13, 2025

NR coverage enhancements Phase 3- MODERATOR SUMMARY AND PROPOSAL

On the basis of submitted documents under this agenda item, and of the following WF (43 supporters, whose 14 operators):

RP-251467 China Telecom, China Unicom, Charter Communications, CHTTL, Deutsche Telekom, KDDI, KT Corp., LG Uplus, NTT DOCOMO, Orange, SK Telecom, SoftBank, Telecom Italia, Telstra, Verizon, Apple, Baicells, CAICT, CATT, ETRI, Ericsson, Fujitsu, Futurewei,Huawei, HiSilicon, HONOR, Intel Corporation, Lenovo, LGE, NEC, New H3C, Nokia, OPPO, Panasonic, Ruijie Networks, Sharp, Spreadtrum, Tencent, TCL, vivo, Xiaomi, ZTE  WF on NR coverage enhancements for Rel-20

After offline, It is proposed by moderator to agree on:

Proposal 1: Include coverage enhancement as one of the RAN1-led item in Rel-20.

Proposal 2: The detailed objectives of Rel-20 coverage enhancements are as follows:

    - Specify following PRACH coverage enhancements (RAN1, RAN2)

    - Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure.

- UL beam information sent to UE, following the reception of MSG1, to assist the UE decision on Msg3 beam selection.

      - Note 1: RAN1 to define “different Tx beams”

      - Note 2: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable.

      - Note 3: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable.

      - Note 4: The multiple PRACH are transmitted over ROs associated with the same SSB.

      - Note 5: The procedure for repetitions of a PRACH transmission is as in Rel-18

    - Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI, (RAN1, RAN2)

    - Specify enhancements to improve PUSCH coverage for high uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables (RAN1)

    - Extend pi/2-BPSK feature and corresponding capability for powerBoosting-pi/2-BPSK to n104 [RAN4]