3GPP TSG-RAN Meeting #108                                              	RP-251794

Prague, Czech Republic, June 9-13, 2025

Source:	CATT (Moderator)

Title:	Moderator's summary for NR mobility enhancements Phase 5

Agenda Item:	15.2.3

Document for:	Discussion

# Introduction

This document is to capture the offline discussions for the WID of NR mobility enhancements Phase 5.

# Discussion

## 2.1 Background

The following are part of the summary RP-250812, which were already endorsed in RAN#107:

The following objectives are considered as baseline for Rel-20 - Specify LTM SCell improvements for further reducing interruption delays - SCell activation - Specify the necessary configuration and UE procedures for NW triggering of LTM SCell activation with LTM cell switch [RAN2, RAN3]. - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4]. NOTE: Reuse the candidate cell LTM measurement and reporting procedures where possible for the SCell improvements NOTE: This objective is NOT applicable to non-co-located CA and is NOT applicable to C-LTM NOTE: This objective is applicable to MCG and SCG - Dynamic L1 measurement and reporting configuration change - Specify the necessary configuration and procedures for MAC CE based configuration selection among prior provided RRC configurations of UE L1 measurement and reporting LTM configuration [RAN2] - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4]. ???? NOTE 1: This sub-objective is applicable to C-LTM as well. NOTE 2: For this objective, we strive to avoid RAN1 impact (including reporting) NOTE 3: RAN1/RAN3/RAN4(?) involvement is based on LS if necessary - No further RAN2 objectives are considered for Rel-20

In RAN#108, contributions [1]-[12] are submitted on this topic, and the plan is to discuss and finalize the corresponding work item description. The moderator also provides draft WID (see the draft folder) as a starting point of the discussion.

## 2.2 Discussions

### 2.2.1 On the scope of the discussion

The moderator suggests to focus this discussion on the following objectives as per the endorsed summary (see section 2.1), i.e.,

- 1st objective - LTM SCell activation improvements for further reducing interruption delays, and

- 2nd objective - Dynamic L1 measurement and reporting configuration change

The main reason is that even if there are some company proposals (see e.g., in [1], [8], and [11]), the previous RAN endorsed conclusion needs to be respected. Therefore the moderator suggests that companies refrain from proposing scope expansion or additional objectives.

### 2.2.2 On RAN4 work for the 2nd objective

As can be seen in 2.2.1, one outstanding open issue is whether RAN4 work is needed for the 2nd objective ‘Dynamic L1 measurement and reporting configuration change’? From the contributions there seems to be the following options:

- Option 1 RAN4 work is NOT needed for the corresponding objective (e.g., see [3], [12])

- Option 2 RAN4 work is needed, therefore should capture in the WID (e.g., see [2], [6])

- Option 3 RAN work may or may not be needed,  and this issue it can be addressed with a note such as ‘RAN4 involvement is based on LS if necessary’ (e.g., see [5])

Companies are invited to provide their views in the following Table.

| Company Name | Which Option do you prefer (1, 2 or 3)? | Further comments if any |
| --- | --- | --- |
| Ericsson | Option 3 | Activation and deactivation of configurations provided by RRC do not seem to have any impact on RAN4. In fact, this does not modify the way how the UE performs the measurements and report them. We don’t need to list explicit NOTE about sending an LS – this should be business as usual in case we find something that impacts RAN4. |
| OPPO | 2 | R4 work is on the definition of the new MAC-CE’s processing delay. |
| Apple | Option3 | UE’s processing delay for MAC CE for LTM is already discussed in RAN4. The question is whether there needs to be additional requirements for the new R20 MAC CE (when introduced). We do not need an explicit objective to RAN4 for this, RAN2 can do technical discussions and based on the agreements made for the new MAC CE, RAN4 impact can be assessed and if needed, an LS can be sent. |
| Samsung | Option 1 or 2 | It depends on the actual scope of RAN4 work. That is, if the scope is only about the processing delay, then it may not be needed (Option 1). However, if the scope is to define the entire switching delay requirements (similar to SCell activation delay requirements), this may need to be specified in RAN4 (Option 2). Maybe the intention of the proponents is about the latter case? If so, we are fine to go with Option 2. |
| vivo | Option 3 | By now, we donot see any RAN4 impact. But for safety, we could add a note that RAN4 could be involved by LS if necessary. |
| MediaTek | Option 1 or 3 | If this objective is included, there should be no RAN4 work. They delay and accuracy requirement should be the same no matter the L1 measurement is configured statically or dynamically. We are also fine to have a NOTE saying something like “RAN4 involvement is based on LS if necessary”. This should be business as usual. With or without this NOTE, RAN2 could still send LS to RAN4. |
| ZTE | Option 3 | We don’t foresee much RAN4 work on this, there might be some update needed about the additional MAC CE processing delay, but it’s minor and can be triggered by RAN2 LS. |
| LGE | Option3 | We do not see clear RAN4 impacts for now. We can identify potential RAN4 impacts, if any, after some progress in RAN2. |
| Futurewei | Option 3 (assuming “RAN” meant to be “RAN4”) | Also agree with Ericsson that the Note doesn’t have to be explicitly captured. |
| NEC | Option 2 or 3 | If companies have common understanding that the RAN4 work, where necessary, can be triggered via LS, it is fine. |
| CATT | Option 3 | We're OK to trigger RAN4 discussion via LS as needed. Then the bullet with "[RAN4]" can be removed. |
| Qualcomm | Option 2 | RAN4 needs to be added and they can discuss RAN4 impacts for dynamic L1 updates in co-ordination with RAN2 discusisons. Atleast we should be put note and to co-ordinate with RAN4. |
| Ofinno | Option 3 | We don’t foresee much RAN4 work impact but think we can send LS if necessary. |

### 2.2.3 Further improvements for the endorsed objectives

Apart from the previous discussion point, there are also other proposals regarding the two endorsed objectives. These proposals are listed in the following.

RP-251303	Rel-20 Mobility Scope	MediaTek Inc.

Proposal 2: “dynamic L1 measurement and reporting configuration” objective preferably not pursued

RP-251045 View on NR Mobility enhancement	NEC

Proposal 1. Dynamic LTM SCell activation is supported for following scenarios:

	Intra-CU Intra-DU MCG/SCG LTM cell switch

	Intra-CU Inter-DU MCG/SCG LTM cell switch

	Inter-CU MCG/SCG LTM cell switch

Proposal 2: For dynamic LTM SCell activation, both L1 and L3 measurement associated with the LTM SCells should be taken into consideration.

RP-251210	Scope of NR mobility enhancements phase 5 in 5G-A Rel-20	vivo

Proposal1 RAN to discuss and clarify the scope for LTM SCell activation with LTM cell switch includes the

below cases

⁃LTM Cell Switch to change the PCell PSCell with SCell based on the LTM candidate configuration and activate

the target SCell ( indicated in the LTM Cell Switch MAC CE

⁃LTM Cell Switch to change a Serving SCell and activate the target SCell directly

Proposal2 RAN to discuss and clarify the SCell measurement for LTM SCell activation with LTM cell switch

should be performed before LTM Cell switch, on more than one LTM candidates

RP-251225	Views on NR mobility enhancements Phase 5	Spreadtrum, UNISOC

Proposal Added ‘Network triggering or UE triggering can be considered.’

RP-251432	Consideration on Rel-20 Mobility	Lenovo

Proposal 1: The objective for SCell activation is applicable to Intra-CU/Inter-CU MCG and Intra-CU/Inter-CU SCG.

Proposal 2: It is suggested to support that network triggering of SCG SCell activation with LTM MCG cell switch.

RP-251045 View on NR Mobility enhancement	NEC

Proposal 3: At least dynamic LTM L1 measurement report configuration change is supported in this WI, and whether to support dynamic LTM L1 measurement configuration change can be discussed during the WI phase.

Proposal 4: Dynamic LTM L1 measurement and reporting configuration change is applicable to both MCG and SCG.

RP-251275	Views on Rel-20 Mobility Enhancements	Qualcomm Incorporated

Proposal 4: Add RAN3 as secondary WG to the WI objective of Dynamic L1 measurement and reporting configuration change

The moderator fails to find a change that gains wide support among all the company contributions. Therefore, companies are invited to share their views regarding whether/how to further improve the two endorsed objectives. As indicated, please refrain from proposing any scope expansion on top of the endorsed objectives.

| Company Name | Any further changes needed to the 1st objective and the 2nd objective as endorsed by RAN (no scope expansion please)? |
| --- | --- |
| Fujitsu | For “Dynamic L1 measurement and reporting configuration change”, does this intend to include both L1-RSRP measurement and CSI measurement before CSC? If RAN1 impact needs to be completely avoided, CSI measurement should be explicitly excluded. |
| Ericsson | Current objectives are enough, and we should not expand the scope further. For the first objective, we can further clarify that the SCell(s) to be activated with the LTM cell switch MAC CE are only those which are within the same cell group from which the LTM cell switch MAC CE is received (i.e., the MAC CE used for LTM cell switch within MCG cannot activate SCell which are at the SCG, and vice versa) |
| Lenovo | Regarding 1st objective for SCell activation, SCell can be activated during LTM cell switch. Currently, we support Intra-CU LTM and Inter-CU LTM in Rel-18 and Rel-19. Therefore, we should clarify in WID that both Intra-CU LTM and Inter-CU LTM are involved for the first objective. |
| OPPO | We do not see the need to largely revise WID (we also foresee the possibility of R3 impact for Obj2, so OK to involve R3) For the first objective, we are OK to cover both intra-CU and inter-CU, but do not see a need to update the objective. We hold similar view as Ericsson that cross-MAC-entity control via MAC-CE should not be pursued. |
| Apple | Some views on the proposals from multiple companies: NEC : the scenarios from these should be allowed with the current wording? We only preclude C-LTM and non-co-located cases. Vivo: on P1, our intention is NOT to have an LTM cell switch command to switch the SCells (something like this can be done with legacy SCell MAC CEs..?). Our intention is to allow for SCell activation as part of an LTM cell switch that does an LTM operation to a PCell or PSCell. On P2, the Note “Reuse the candidate cell LTM measurement and reporting procedures” would imply this? Spreadtrum/UNISCOC: not critical for us, prefer to not add more (per our guidelines from RAN-107) Lenovo: P1 should be implied from obj1 wording (i.e, not precluded). On P2, same view as Ericsson. We do not want to introduce cross CG from MAC, MACs are per-CG and this will add new complexity without a practical usecase. Qualcomm: RAN3 would likely be impacted for obj2 as well. Agree with this. General comments: RAN3 would be the secondary WG for this. And R4 involvement would be only for obj 1. |
| Samsung | Fine with further clarification from Ericsson about the first objective. Fine to add RAN3, and we could add ‘if necessary’. |
| Vivo | On 2nd bullet, considering the latest conclusion in RAN2 as below, we think the motivation discussed in last RAN meeting has been already addressed. In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to). If companies really want this bullet, we suggest to make the below clarification: Note: This objective includes dynamic change on beam level configuration and cell level configuration, which allows the RRC configuration exceed the UE capability. Besides, the justification should be updated accordingly. |
| MediaTek | We fully agree with moderate that we should NOT further extend the scope of this WID. We already have enough objectives. In fact, we think current objectives could be further reduced. We should remove the 2nd objective as this already covered by the latest R19 functionality (agreed in R2 meeting last Q). R2 TU is also another concern. We have too few TU for 6G and we should try to down-scope 5G-A objectives. |
| ZTE | For the 1st objective, we also think both intra-CU and inter-CU are considered, and the SCell activation is triggered together with LTM cell switch command, and there is no intention to support cross-CG SCell activation. It is ok to add more clarification to the 1st objective. For the 2nd objective (which should be kept as it is not really fully covered by the recent R2 conclusion mentioned by Vivo), the intention is to only change the L1-RSRP measurements (it does not impact the early CSI measurement), and we understand this is already clear with the current wording. Regarding the RAN3 impact, so far it is unclear to us about the potential work in RAN3, further clarification is needed. But in any case, RAN3 work (if needed) can be triggered by LS. Regarding the clarification of 2nd objective from Vivo, we share the same understanding, but we also think that what is proposed is not precluded by the current wording, e.g. whether it is beam-level or cell-level or both will be discussed in normative phase. |
| LGE | We see the two objectives are valuable. So, the two objectives need to remain. Objective1 - We understand that early CSI acquisition for SCell before cell switch is not the scope. - Ericsson’s clarification for objective1 seems useful. - On vivo’s comments on the scope, we understand the intention of the objective is to support direct SCell activation upon PCell/PSCell change, nothing else. Objctive2 - we are not convinced if RAN3 involvement is absolutely necessary. We can trigger necessary RAN3 work based on RAN2 discussion. |
| Futurewei | We agree with the moderator and prefer not to further expand the scope. |
| NEC | On our proposals; P1,P2, P4 is to confirm the scopes. If we can confirm they are common understanding, it is sufficient (no change is needed). Clarification in P3 may depend on possible RAN4 involvement (in 2.2.2 above), i.e. as long as some possibility of RAN4 involvement is expected, it is fine to keep the objective 2 as it is. On QC proposal of adding RAN3 for the objective 2, we are fine to trigger RAN3 work via LS, where necessary. |
| CATT | We can add a NOTE(s) clarifying the applicable scenarios of both intra-/inter-CU cases, and clarifying that cross-MAC-entity operation is not within the scope. |
| Qualcomm | Objective 1 : Scell activation is within given Cell Group only (either in MCG or SCG). But it is applicable for Intra-CU (Intra-DU and Inter-DU) and Inter-CU as well. Objective 2: There will be RAN3 impacts as well and must be added as secondary WG. When a specific cell L1 measurements are activated/deactivated, after subsequent LTM HO, source node has to inform target node about the status. New target will make further decision about which cells to be activated/deactivated based on current status of cells and new measurements received from UE or based on gNB implemenatation. So, RAN3 should be added as secondary WG . |
| Ofinno | For the 1st objective, we are fine with the clarification from Ericsson. For the 2nd objective, we can send LS to RAN3 if necessary. So, we are fine with the NOTE. |

### 2.2.4 On other parts of the draft WID

The previous sections already address the issues for the objective part. In this section, company can provide other comments on other parts of the draft WID.

| Company Name | Any other comments |
| --- | --- |
| Fujitsu | If TU (especially RAN4) is available, we would like to consider additional scope in Rel-20 LTM. RAN1 impact is very small (0.5 for 1~2 quarter) and no RAN2 impact is foreseen. RAN4 part can be further discussed. - Specify support for L1-SINR measurement quantity for L1 measurements for LTM [RAN1, RAN4] - L1-SINR measurements to be included gNB scheduled reporting - Filtering of measurement results is up to UE implementation - Support of inter-frequency scenario - Support of SSB (SS-SINR) |
| DOCOMO | Support Fujitsu’s proposal |
| Samsung | It seems Fujitsu’s additional proposal is being discussed in the other thread led by RAN2 chair, so we do not have to collect the view in this thread. |

# Conclusion

Based on the discussions, the following objectives are proposed for Rel-20 NR mobility enhancements Phase 5.

### Core part

The objectives are as the following

- LTM SCell activation improvements for further reducing interruption delays

  - Specify the necessary configuration and UE procedures for NW triggering of LTM SCell activation as part of the SpCell LTM cell switch [RAN2, RAN3]

  - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4]

NOTE 1: Reuse the candidate cell LTM measurement and reporting procedures where possible for the SCell activation improvements

NOTE 2: Early CSI acquisition procedure for SCell is not in the scope

NOTE 3: This objective is not applicable to non-co-located CA and is not applicable to C-LTM

NOTE 4: This objective is applicable to MCG and SCG, and is applicable to both intra-CU and inter-CU cases

NOTE 5: The objective is only applicable to the case of SCell activation within the same cell group from which the LTM cell switch command MAC CE is received

- Dynamic L1 measurement and reporting configuration change

  - Specify the necessary configuration and procedures for MAC CE based configuration selection among prior provided RRC configurations of UE L1 measurement and reporting LTM configuration [RAN2]

NOTE 6: This sub-objective is applicable to C-LTM as well

NOTE 7: This objective is limited to the configuration related to L1 measurement and reporting configuration and strives to avoid RAN1 impact (including reporting)

NOTE 8: RAN1/RAN3/RAN4 involvement is based on LS if necessary

### Performance part

The objectives are as follows [RAN4]:

- Evaluate and specify as necessary the needed RAN4 requirements for LTM SCell activation improvements

# Reference

- RP-250979	Proposal on additional scope for Rel-20 mobility	Fujitsu Limited

- RP-251045	View on NR Mobility enhancement	NEC

- RP-251188	Finalizing objectives for Rel-20 Mobility Enhancements	Apple Inc, ZTE Corporation

- RP-251210	Scope of NR mobility enhancements phase 5 in 5G-A Rel-20	vivo

- RP-251225	Views on NR mobility enhancements Phase 5	Spreadtrum, UNISOC

- RP-251275	Views on Rel-20 Mobility Enhancements	Qualcomm Incorporated

- RP-251303	Rel-20 Mobility Scope	MediaTek Inc.

- RP-251340	On Rel-20 Mobility Enhancements – Phase 5	Charter Communications

- RP-251432	Consideration on Rel-20 Mobility	Lenovo

- RP-251522	View of the WID scope for NR Mobility Enhancements Phase 5	CEWiT

- RP-251606	Views on Rel-20 Mobility Enhancements	Ofinno, LLC

- RP-251727	Views on further mobility enhancement in 5G-A Rel-20	ZTE Corporation, Sanechips