3GPP TSG RAN meeting #108	RP-251802

Prague, Czech Republic, June 09-13, 2025	revision of RP-251240

## Status Report to TSG

Agenda item:			9.2.1

| WI / SI Name | Study on channel modelling enhancements for 7-24GHz for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_7_24GHz_CHmod |  |  |  |  |
| Unique ID | 1020081 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234018 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2025 | Core part: n/a | Performance part: n/a | Testing part: n/a |  |
| Overall Completion level | Study Item: 100 % | Core part: n/a | Performance Part: n/a | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (Primary) | Name | LEE Daewon |
|  | Company | Intel Corporation |
|  | Email | daewon.lee@intel.com |
| Rapporteur | Name | ZHANG Nan |
|  | Company | ZTE |
|  | Email | zhang.nan152@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

##### 2.1.1.1	Conclusions/Observations/Agreements from RAN1 #120-bis

Agreement

- Endorse R1-2503060 to be captured as reference for TR38.901.

- R1-2503060 contains the latest update of measurement data sources for channel model as part of the Rel-19 channel modeling enhancements for 7-24 GHz SI.

Agreement

Draft CR R1-2503129 to TR38.901 is endorsed in principle.

Conclusion

- Based on measurement data provided, RAN1 concludes that following delay spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are needed.

  - InH LOS and NLOS

Agreement

- Update the UMi LOS and NLOS delay spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| Delay spread (DS) lgDS=log10(DS/1s) | µlgDS | -0.24 log10(1+ fc) - 7.14 | -0.24 log10(1+ fc) - 6.83 | -0.18 log10(1+ f) - 7.28 | -0.22 log10(1+ f) – 6.87 |
|  | σlgDS | 0.38 | 0.16 log10(1+ fc) + 0.28 | 0.39 | 0.19 * log10(1+ f) + 0.22 |

Agreement

- Update the UMa LOS and NLOS delay spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| Delay spread (DS) lgDS=log10(DS/1s) | µlgDS | -6.955 - 0.0963 log10(fc) | -6.28 - 0.204 log10(fc) | -0.0794 * log10(f) – 7.067 | -0.134 * log10(f) – 6.47 |
|  | σlgDS | 0.66 | 0.39 | 0.026 * log10(f) + 0.57 | 0.39 |

Agreement

- Update the UMi LOS and NLOS AOA spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOA spread (ASA) lgASA=log10(ASA/1) | µlgASA | -0.08 log10(1+ fc) + 1.73 | -0.08 log10(1+ fc) + 1.81 | -0.07 * log10(1+ f) + 1.66 | -0.07 * log10(1+ f) + 1.76 |
|  | σlgASA | 0.014 log10(1+ fc) + 0.28 | 0.05 log10(1+ fc) + 0.3 | 0.021 * log10(1+ f) + 0.26 | 0.05 * log10(1+ f) + 0.27 |

Agreement

- Update the UMa LOS and NLOS AOA spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOA spread (ASA) lgASA=log10(ASA/1) | µlgASA | 1.81 | 2.08 - 0.27 log10(fc) | 1.76 | -0.25 * log10(f) + 2.04 |
|  | σlgASA | 0.20 | 0.11 | 0.19 | -0.03 * log10(f) + 0.17 |

Working Assumption

- Adopt the following absolute delay parameters for RMa scenarios.

| Scenarios |  | RMa |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -8.33 |
|  | σlgΔτ | 0.26 |
| Correlation distance in the horizontal plane [m] |  | 50 |

Agreement

- Defines upper bound of absolute delay in indoor office scenario as 2L/c where L is the largest dimension of the office hall.

- No upper bound of absolute delay in UMi, UMa, and RMa scenario.

Conclusion

- For the following scenarios, there is no consensus to update pathloss models due to lack of consistent and significant observed difference between model and measurements.

  - UMa LOS/NLOS

Agreement

- Update the UMa LOS and NLOS AOD spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit without frequency dependency. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 1.06 + 0.1114 log10(fc) | 1.5 - 0.1144 log10(fc) | 0.95 | 1.12 |
|  | σlgASD | 0.28 | 0.28 | 0.31 | 0.42 |

Agreement

- For UE antenna modeling, support the following direction antenna radiation pattern for calibration purposes.

| For directional radiation pattern |  |  |
| --- | --- | --- |
| Vertical Radiation Pattern | Horizontal | Max Gain |
| θ3dB=125° SLAV= 22.5 dB | ϕ3dB=125° Am= 22.5 dB | 5.3 dBi |

Agreement

- Reference UE orientation vector of the handheld UE is perpendicular to the plane of the flat UE handheld device, and reference point for near field phase calculation of the UE is assumed to be the center of the plane of the UE handheld.

Agreement

- For cases when a candidate antenna placement location is used for two distinct antenna polarization field pattern:

  - Reference radiation pattern of the UE antenna model is

    - For first antenna field pattern: Fθ″″θ″,ϕ″=A″θ″,ϕ″and Fϕ″″θ″,ϕ″=0.

    - For second antenna field pattern: Fθ″″θ″,ϕ″=0 and Fϕ″″θ″,ϕ″=A″θ″,ϕ″.

Agreement

Update and agree to the following the SMa LOS pathloss working assumption:

| LOS | For 10m ≤ d < dBP, PL1=20log1040πdfc3+min0.03h1.72,10log10d-min0.044h1.72, 14.77+0.002log10hd For dBP ≤ d < 5000m, PL2=PL1dBP+40log10ddBP dBP=2πhBShUTfc/c, fc is the center frequency in Hz, c=3×108m/s | For 10m ≤ d < dBP, σSF=4 For dBP ≤ d < 5000m, σSF=6 |
| --- | --- | --- |

Agreement

Update and agree to the following the SMa NLOS pathloss:

| NLOS | PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc-3.2log1011.75hUT2-4.97 | σSF=8 | 0.8 ≤fc≤ 37 GHz 10 m < dBP < 5000 m 1 m < hUT < 14 m h = 10 m, W = 10 m |
| --- | --- | --- | --- |

Agreement

For suburban scenario, adopt the LOS probability

PLOS=1, d2D≤10exp-d2Dkcommercialexp-d2Dkresidentialexp-d2Dkvegetation, d2D>10

kcommercial=-dcommercialln1-rcommericial∙hBS-hUThcommercial-hUT

kresidential=-dresidentialln1-rresidential∙hBS-hUTmax⁡(hresidential-hUT ,0)

kvegetation=-dvegetationln1-rvegetation∙hBS-hUThvegetation-hUT

dcommercial,residential,vegetation=clutter bin size

rcommercial,residential,vegetation=clutter density

hcommercial,residential,vegetation=clutter height

| Parameter |  |
| --- | --- |
| dcommercial,residential,vegetation | 30 m |
| hcommercial | 20 m |
| hresidential | 8 m |
| hvegetation | 15 m |
| rcommercial | 2% |
| rresidential | 18% |
| rvegetation | 0% (no vegetation), 10% (sparse vegetation), or 20% (dense vegetation) |

Agreement

- Update the UMa NLOS ZOA spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | 0.95 | -0.3236 log10(fc) + 1.512 |  | -0.2856 log10(fc) + 1.445 |
|  | σlgZSA | 0.16 | 0.16 |  | 0.17 |

Agreement

- Adopt the following absolute delay parameters for InH scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -8.6 |
|  | σlgΔτ | 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Agreement

Adopt the following correlation distance for SMa spatial consistency

Table 7.6.3.1-2 Correlation distance for spatial consistency

| Correlation distance in [m] | SMa |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS | O2I |
| Cluster and ray specific random variables | 40 | 50 | 15 |
| LOS/NLOS state | 50 |  |  |
| Indoor/outdoor state | 50 |  |  |

Agreement

Update SMa description as follows:

- In suburban macro-cells base stations are located above the rooftops and clutter (e.g. vegetation) to allow wide area coverage, and mobile stations are outdoors at street level. Buildings are typically low residential detached houses with one or two floors, or blocks of apartments/condos or commercial buildings with a few floors. Occasional open areas such as parks or playgrounds between the houses make the environment rather open. Streets do not form urban-like regular strict grid structure. In suburban areas, vegetation is more prevalent than in urban areas with a high variability of foliage density across different suburban areas.

Agreement

For suburban scenario, adopt the following assumptions for calibration purposes:

- ISD = 1299m and 1732m

- Note: For evaluation in study items/work items, if needed, admission control policies may be considered in conjunction with the ISD choice to address out of coverage UEs.

Agreement:

- For standard multi-panel glass penetration loss model, clarify that reference thickness is 3 cm. no changes to the model.

- For IRR glass penetration loss model, update the model equation as follows:

  - LIRRglass=25.4+ 0.11f

  - Single coating for IRR glass is assumed.

- For concrete penetration loss model, clarify that reference thickness is 23 cm. no changes to the model.

- For wood penetration loss model, clarify that reference thickness is 3.3cm. no changes to the model.

- For plywood penetration loss model:

  - Lplywood=1.03 + 0.17f

  - Reference thickness is 1.27 cm

  - Note: plywood penetration loss model is intended to be used for SMa penetration loss modeling.

Agreement

To potentially resolve the UMi/UMa pathloss convergence beyond breakpoint distance, RAN1 to further discuss following options:

- Option 1) Set hE to 0 m

- Option 2) Applying only PL1 when performing link analysis

- Option 3) add note that provide information on pathloss convergence phenomena beyond the breakpoint distance across different frequencies.

- Other options are not precluded.

Working assumption

Proposed Working assumption

- Update the UMa LOS, NLOS, and O2I Cluster AOD spread as follows:

  - Note: the update parameter for LOS and NLOS was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.  Each group is given equal weightage.

  - Note: the update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz.

| Scenarios | TR 38.901 UMa |  |  | Updated UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O2I | LOS | NLOS | O2I |
| Cluster ASD (CASD) in [deg] | 5 | 2 | 5 | 3.74 | 1.82 | 1.82 |

Working assumption

Proposed Working assumption

- Update the UMa O2I AOD spread as follows:

  - Note: the update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz.

  - Note: UMa O2I ASD values of Rel-14 TR38.901 used values from UMa O2I ASD values of TR36.873. UMa O2I ASD values of TR36.873 used UMi O2I ASD values from ITU-RM.2135-1. IMT-2020 modeling used the same values from TR36.873. TR25.996 does not contain UMa O2I values. The UMi O2I ASD parameters were derived from Winner II report.

| Scenarios |  | TR 38.901 UMa | Updated UMa |
| --- | --- | --- | --- |
|  |  | O2I | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 1.25 | 0.84 |
|  | σlgASD | 0.42 | 0.42 |

Conclusion

- RAN1 to continue study the number of clusters for InH, UMi, and UMa scenarios, and intra-cluster power angular profile modeling for all scenarios.

  - Study to further consider distribution of the number of clusters, resolvable clusters and subpaths of the measurements.

Conclusion

- Further study on introduction of polarization variability for each rays of cluster for NLOS component of the channel as an optional modeling component.

  - Polarization variability is introduced by amplitude scaling parameter η for XPR matrix as follows:

  - Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATηn.m.θθexpjΦn,mθθηn,m,θϕκn,m-1expjΦn,mθϕηn,m,ϕθκn,m-1expjΦn,mϕθηn,m,ϕϕexpjΦn,mϕϕ

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT⋅drx,uλ0expj2πrtx,n,mT⋅dtx,sλ0expj2πrrx,n,mT⋅vλ0t

  - The study should include distribution of the amplitude scaling parameters, power normalization of the amplitude scaling.

Agreement

Adopt the following equations for scaling of angles for CDL channel model. (This updates existing agreements and working assumption).

- Option 0) Scaling applies to both NLOS and LOS angles for all cluster paths, Mean Angle and AS include both NLOS and LOS cluster paths but not subpaths

  - ϕn,m,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired+cASA/ASD/ZSA/ZSDαm

  - ϕLOS,scaled=ϕLOS,intermediate-μϕ,intermediate+μϕ,desired

  - ϕn,intermediate=s∙WrapTo180ϕn,model-μϕ,model

  - ϕLOS,intermediate=s∙WrapTo180ϕLOS,model-μϕ,model

  - AS=-2lnexpjφLOSPLOS+n=1NexpjφnPnPLOS+n=1NPn

  - μϕ,model=argexpjϕLOS,modelPLOS+n=1Nexpjϕn,modelPn

  - μϕ,intermediate=argexpjϕLOS,intermediatePLOS+n=1Nexpjϕn,intermediatePn

Table X: Example Scale factor values for each CDL model

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-A | 5 | 0.0680 | 30 | 0.3531 | 5 | 0.0352 | 1 | 0.0352 |
|  | 10 | 0.1360 | 45 | 0.5268 | 10 | 0.1056 | 3 | 0.1056 |
|  | 15 | 0.2041 | 60 | 0.6981 | 15 | 0.1761 | 5 | 0.1761 |
|  | 25 | 0.3405 |  |  |  |  |  |  |
| CDL-B | 5 | 0.1238 | 30 | 0.5417 | 5 | 0.1940 | 1 | 0.1940 |
|  | 10 | 0.2475 | 45 | 0.8081 | 10 | 0.5822 | 3 | 0.5822 |
|  | 15 | 0.3710 | 60 | 1.0709 | 15 | 0.9705 | 5 | 0.9705 |
|  | 25 | 0.6168 |  |  |  |  |  |  |
| CDL-C | 5 | 0.1281 | 30 | 0.4307 | 5 | 0.3643 | 1 | 0.3643 |
|  | 10 | 0.2568 | 45 | 0.6447 | 10 | 1.0929 | 3 | 1.0929 |
|  | 15 | 0.3864 | 60 | 0.8585 | 15 | 1.8219 | 5 | 1.8219 |
|  | 25 | 0.6513 |  |  |  |  |  |  |
| CDL-D | 5 | 0.3231 | 30 | 9.8888 | 5 | 0.4477 | 1 | 0.4477 |
|  | 10 | 0.6652 | 45 | N/A | 10 | 1.3469 | 3 | 1.3469 |
|  | 15 | 1.0594 | 60 | N/A | 15 | 2.2579 | 5 | 2.2579 |
|  | 25 | 5.8637 |  |  |  |  |  |  |
| CDL-E | 5 | 0.3950 | 30 | 2.9733 | 5 | 0.9714 | 1 | 0.9714 |
|  | 10 | 0.8009 | 45 | N/A | 10 | 2.9180 | 3 | 2.9180 |
|  | 15 | 1.2330 | 60 | N/A | 15 | 4.8774 | 5 | 4.8774 |
|  | 25 | 2.3627 |  |  |  |  |  |  |

Note: the values are computed by

s=minx≥0{x: ASx=ASdesired}.	(A-3)

ASx=-2lnexpjx∙WrapTo180(φLOS-μϕ,model)PLOS+n=1Nexpjx∙WrapTo180ϕn,model-μϕ,modelPnPLOS+n=1NPn	(A-4)

Working Assumption

- SMa O2I will use the same values as SMa NLOS

| Scenarios |  | SMa (7 – 24 GHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | µlgDS | –7.23 | -7.12 | Copy NLOS |
|  | σlgDS | 0.38 | 0.33 | Copy NLOS |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 0.55 | 0.55 | 0.55 |
|  | σlgASD | 0.25 | 0.25 | 0.25 |
| AOA spread (ASA) lgASA=log10(ASA/1) | µlgASA | 1.07 | 1.39 | Copy NLOS |
|  | σlgASA | 0.13 | 0.34 | Copy NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | 0.95 | 1.14 | Copy NLOS |
|  | σlgZSA | 0.16 | 0.16 | Copy NLOS |
| Shadow fading (SF) [dB] | σSF | 4 | 8 | 8 |
| K-factor (K) [dB] | µK | 9 | N/A | N/A |
|  | σK | 7 | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0 | 0 | 0 |
|  | ASA vs DS | 0.8 | 0.7 | Copy NLOS |
|  | ASA vs SF | –0.5 | 0 | Copy NLOS |
|  | ASD vs SF | –0.5 | -0.4 | -0.4 |
|  | DS vs SF | –0.6 | -0.13 | Copy NLOS |
|  | ASD vs ASA | 0 | 0 | 0 |
|  | ASD vs K | 0 | N/A | N/A |
|  | ASA vs K | 0 | N/A | N/A |
|  | DS vs K | 0 | N/A | N/A |
|  | SF vs K | 0 | N/A | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0 | 0 | 0 |
|  | ZSA vs SF | -0.8 | -0.47 | -0.47 |
|  | ZSD vs K | 0 | N/A | N/A |
|  | ZSA vs K | 0 | N/A | N/A |
|  | ZSD vs DS | 0.5 | -0.5 | -0.5 |
|  | ZSA vs DS | 0 | 0 | 0 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 |
|  | ZSA vs ASD | 0 | -0.1 | -0.1 |
|  | ZSD vs ASA | -0.3 | 0 | 0 |
|  | ZSA vs ASA | 0.4 | 0.36 | 0.36 |
|  | ZSD vs ZSA | 0 | 0 | 0 |
| Delay scaling parameter r |  | 2.4 | 1.5 | 1.5 |
| XPR [dB] | µXPR | 8 | 4 | 4 |
|  | σXPR | 4 | 3 | 5 3 |
| Number of clusters N |  | 15 | 14 | 14 |
| Number of rays per cluster M |  | 20 | 20 | 20 |
| Cluster DS (cDS) in [ns] |  | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) 11 |
| Cluster ASD (cASD) in [deg] |  | 1.5 | 1.5 | Copy NLOS |
| Cluster ASA (cASA) in [deg] |  | 5 | 10 | 10 |
| Cluster ZSA (cZSA) in [deg] |  | 7 | 7 | 7 |
| Per cluster shadowing std ξ [dB] |  | 3 | 3 | 4 |
| Correlation distance in the horizontal plane [m] | DS | 6 | 40 | 40 |
|  | ASD | 15 | 30 | 30 |
|  | ASA | 20 | 30 | 30 |
|  | SF | 40 | 50 | 50 |
|  | K | 10 | N/A | N/A |
|  | ZSA | 15 | 50 | 50 |
|  | ZSD | 15 | 50 | 50 |

Agreement

For suburban scenario, adopt the following ZSD parameters as updated working assumption:

- Values in [ ] are working assumption

| Scenarios |  | SMa LOS (7 – 24 GHz) | SMa NLOS (7 – 24 GHz) | SMa O2I (7 – 24 GHz) |
| --- | --- | --- | --- | --- |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | µlgZSD | 0.14 | 0.14 | 0.14 |
|  | σlgZSD | 0.16 | 0.16 | 0.16 |
| ZOD offset | µoffset,ZOD | 0 | atanhBS-hcld2D-atanhBS-hUTd2DhUT<hcl0hUT≥hcl hcl= [3.5] |  |

Working Assumption

Introduce new penetration loss outdoor-to-indoor (O2I) building penetration loss model applicable for SMa:

- Values in [ ] are working assumption

|  | Path loss through external wall: PLtw in [dB] | Indoor loss: PLin in [dB] | Standard deviation σp in [dB] |
| --- | --- | --- | --- |
| Low-loss A model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lplywood10 | 0.5 d2D-in | 4.4 |

Agreement

Adopt the following absolute time of arrival parameters for SMa

- Values in [ ] are working assumption

| Scenarios |  | SMa |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | [-7.702] |
|  | σlgΔτ | [0.4] |
| Correlation distance in the horizontal plane [m] |  | [6 50] |

Agreement

- Adopt the following changes to Clause 7.2 of TR38.901.

< Unchanged parts are omitted > Table 7.2-1: Evaluation parameters for UMi-street canyon and UMa scenarios Parameters; UMi - street canyon; UMa. Cell layout; Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200m); Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m1). BS antenna height hBS; 10m; 25m. UT location; Outdoor/indoor; Outdoor and indoor; Outdoor and indoor. LOS/NLOS; LOS and NLOS; LOS and NLOS. Height hUT; Same as 3D-UMi in TR36.873; Same as 3D-UMa in TR36.873. Indoor UT ratio; 80%; 80%. UT mobility (horizontal plane only); 3km/h; 3km/h. Min. BS - UT distance (2D); 10m; 35m. UT distribution (horizontal); Uniform; Uniform. NOTE 1: UMa scenarios with ISDs between 200-500m can be used for evaluations < Unchanged parts are omitted >

- Adopt the following changes to Clause 6.2 of TR38.901.

## 6.2 Scenarios of interest Brief description of the key scenarios of interest identified (see note): (1) UMi (Street canyon, open area) with O2O and O2I: This is similar to 3D-UMi scenario, where the BSs are mounted below rooftop levels of surrounding buildings. UMi open area is intended to capture real-life scenarios such as a city or station square. The width of the typical open area is in the order of 50 to 100 m. Example: [Tx height:10m, Rx height: 1.5-2.5 m, ISD: 200m] (2) UMa with O2O and O2I: This is similar to 3D-UMa scenario, where the BSs are mounted above rooftop levels of surrounding buildings. Example: [Tx height:25m, Rx height: 1.5-2.5 m, ISD: 200m, 500m]

Working Assumption

- Reference orientation of the handheld UE as follows:

  - FFS how to capture the reference orientation of the handheld UE

- For calibration of handheld UE, use the following UE rotation based on reference UE orientation

  - For calibration with blockage:

    - For one-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

    - For dual-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 90 deg,

    - For hand and head blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg,

  - For all other calibration cases:

    - ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

  - Note: UT array orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle).

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 90 deg

Agreement

For CPE devices adopt the following device dimensions for UE antenna modeling:

- (x cm × y cm × z cm) = (0 cm × 20 cm × 20 cm)

- Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device.

- Note: a candidate antenna location (e.g., 9) can be used for multiple antennas

Agreement

For UE antenna modeling of handheld devices, introduce optional antenna imbalance modeling as part of antenna field pattern as follows:

- No imbalance is modelled by default.

- If modelled, Randomized loss is applied per UE antenna port. Details of the randomized value and it’s distribution will not be specified in TR 38.901 as part of Rel-19 SI.

- If modelled, Randomized loss can be applied independently for the UL and DL directions.

Agreement

- Confirm the downtilt value for SMa with ISD of 1299m, and introduce downtilt value [93] for SMa with ISD of 1732m

  - Downtilt value for SMa with ISD of 1732m is working assumption

| Parameter | Values |
| --- | --- |
| BS antenna electrical downtilting | 95 degrees for SMa for ISD = 1299m [93] degrees for SMa for ISD = 1732m |

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS) is determined as:

- For the nth UE, the Prn is generated using a truncated normal distribution within [0,1]

Prn=Nμ, σ2;     where 0≤Prn≤1.

  - FFS: the value of μ, σ.

    - Note: It should be fixed in RAN1#120-bis;

- For kth cluster of the nth user, if the xk,n is smaller than Prn, the cluster is impacted by the SNS, where the xk,n is defined as xk,n~U(0,1))

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, generate the visibility region as a rectangle randomly located at a corner of the antenna array with dimension a*b, where a~U(Vn*W,W), b=Vn*H*W/a, H is the antenna array height in vertical dimension and W is the antenna array width in horizontal dimension, Vn is calculated by

Vn=Α∙e-maxPn-Pnλ+Β+δ.

where A=0.15, B=0.45, λ=33, δ~N(0,0.0015) for UMa scenario, and n is a cluster index

- FFS: The value for other scenarios.

  - Note: It should be fixed in RAN1#120-bis;

- FFS: the details of randomly located VR

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted,

- For clusters without SNS, the power attenuation factor αs,n is set to 1.

- For clusters with SNS, the power attenuation factor αs,n is generated by

αs,n=1                             ,xs-x0,n≤a,ys-y0,n≤be-xs-xa,nDn∙C        ,xs-x0,n>a,ys-y0,n≤b  e-ys-yb,nDn∙C        ,xs-x0,n≤a,ys-y0,n>b e-xs-xa,n2+ys-yb,n2Dn∙C                          ,others

where, Dn=xA,n-xa,n2+yB,n-yb,n2 , xA,n, yB,n is the coordinate of the other corner of the antenna array on the diagonal with reference corner x0,n, y0,n, and xa,n, yb,n is the coordinate of the other corner of the rectangular VR on the diagonal with reference corner x0,n, y0,n. The xs, ys denotes the coordinate of the antenna element out of VR region. C is the roll-off factor between the visible and invisible regions.

- FFS: The value of C.

  - Note: It should be fixed in RAN1#120-bis;

Agreement

For the modelling of spatial non-stationarity at UE side, introduce 3 sets of fixed attenuation values, each set corresponding to a range of frequency,

| Antenna index | Power attenuation (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | One hand grip |  |  | Dual hand grip |  |  | Head and one hand grip |  |  |
|  | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz |
| 1 | - | 0.7 | 1.0 | - | 11.0 | 3.8 | - | 3.7 | 4.0 |
| 2 | - | 4.1 | 2.4 | - | 1.1 | 3.8 | - | 4.1 | 3.7 |
| 3 | - | 3.3 | 3.8 | - | 10.5 | 3.8 | - | 4.3 | 3.3 |
| 4 | 13.6 | 7.2 | 3.6 | 5.8 | 5.6 | 2.6 | 15.1 | 7.8 | 3.3 |
| 5 | - | 10.8 | 3.8 | - | 1.5 | 1.0 | - | 11.7 | 3.3 |
| 6 | - | 9.1 | 2.4 | - | 1.4 | 1.0 | - | 10.1 | 3.7 |
| 7 | - | 0.7 | 1.0 | - | 1.3 | 1.0 | - | 2.9 | 4.0 |
| 8 | 2.4 | 0.6 | 1.0 | 5.6 | 6.2 | 2.6 | 4.9 | 4.2 | 4.0 |

Note: For below 1GHz, the model is only applicable for 2-antenna system with antenna location index 4 and 8

Note: The selection of number of antenna and antenna index will be determined as part of the assumption for simulation.

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS):

- In UMa, the distribution of Prn is determined as: μ=0.56, σ=0.20;

Agreement

For near-field channel, the scaling factor (sUE,n) used to generate the d2 is:

- For number of k1 non-direct paths, sUE,n=1.

- For number of k2 non-direct paths:

  - Option-1: sUE,n=1-sTRP,n;

    - Note 1: It’s assumed that only single non-specular interaction along with specular reflection(s) is considered in the propagation channel

    - Note 2: At least most of the sources of non-specular interaction are assumed to be a point scatterer or diffractions.

Agreement

For near-field channel, to generate the d1 for the k2 non-direct paths, the distribution of scaling factor sTRP,n is:

- For the UMi scenarios, a Beta distribution with α=1.53, β=1.42

- For the UMa scenarios, a Beta distribution with α=1.93, β=1.33

- For the InH scenarios, a Beta distribution with α=1.25, β=1.27

Conclusion

For near-field channel, to generate the d1 for k2 non-direct paths:

- No additional lower bound (i.e., dmin) of d1 is defined.

Agreement

In the Indoor office scenario for near-field channel, k1= 4.

Agreement

In the Indoor factory scenarios, to generate the d1 non-direct path for the near-field channel model:

- k1= 4;

- For the k2 non-direct paths, the distribution of scaling factor sTRP,n is a Beta distribution with α=1.38, β=1.26

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, to generate the power attenuation factor αs,n for clusters with SNS, the value of C (i.e., the roll-off factor between the visible and invisible regions) is:

- C= 13 for UMa.

Agreement

For the modelling of spatial non-stationarity, the visibility region, i.e., a rectangle, is randomly located at a corner of the antenna array by following approach:

- The location of the VR is jointly determined by following conditions:

  - The VR is located at either left or right part in horizontal domain of antenna array with equal probability of 1/2.

  - The probability of VR at the upper part of antenna array is [0.8] and the probability of VR at the lower part of antenna array is [0.2];

Agreement

For the calibration parameters for the near-field channel,

| Parameter |  | Values |
| --- | --- | --- |
| Scenarios |  | UMi-street Canyon, Indoor-office (open office) |
| Carrier Frequency |  | 7 GHz (Optional) 15 GHz |
| Bandwidth |  | 20 MHz |
| BS antenna height |  | UMi-street Canyon: 10m Indoor-office: 3m |
| BS antenna downtilt |  | 102 degrees for UMi-street Canyon 180 degrees for indoor (i.e., array panel downward facing) |
| BS Tx power |  | 44 dBm for UMi-Street Canyon 24 dBm for Indoor |
| BS antenna configurations and port mapping |  | For UMi scenario: Config 1 for 7GHz - (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; - (Optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Config 2 for 15GHz - (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 8, 16), dH = dV = 0.5λ For Indoor-office scenario for 7GHz: - (M, N, P, Mg, Ng; MP, NP) = (8, 24, 2, 1, 1; 8, 8), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling |  | Model-2 in Clause 7.3.2 of TR38.901 |
| UE Location | Outdoor/indoor | UMi-street Canyon: 100% outdoor Indoor-office: 100% indoor |
|  | LOS/NLOS | 100% LOS |
|  | UE antenna height | UMi-street Canyon: 1.5m Indoor-office: 1m |
| UT antenna configurations |  | Config A: Mg = Ng = 1, M = 2, N = 1, P = 2, dH = dV = 0.5λ Config B: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern |  | Isotropic |
| UT Polarized antenna modelling |  | For UT Config A, Model-2 in Clause 7.3.2 of TR38.901 For UT Config B, following the Clause 7.3.2 of TR38.901 |
| Applicability of Nearfield Modeling |  | Component of NF channel to be considered: - Phase of Direct path - Phase of Non-direct path, applied only at the BS side (optionally) |
| Calibration method |  | Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric for a each distance Z For UMi-street Canyon: Z = 10, 30, 50, 100 m (optional) Z = 20, 40, 80 m For Indoor: Z = 0, 2, 6, 10 m (optional) Z = 4, 8 m |
| Metric |  | CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest) PRB singular value and the 1st PRB (largest) singular value (serving cell) at t=0 plotted in 10*log10 scale. - Note-1: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. - Note-2: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) |

Agreement

The following assumption is used for UE side SNS calibration:

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | (M, N, P, Mg, Ng; MP, NP) = (8, 16, 2, 1, 1; 8, 16), dH = dV =0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 of TR38.901 |
| UT Antenna Configuration | Config A for 7GHz: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1, 7, 3, 5) as described in Clause 7.3 Config B for 15 GHz: 8 antenna port with dual polarization based on handheld device antenna model using all candidate antenna locations (1,2,3,4,5,6,7,8) as described in Clause 7.3. |
| UT antenna pattern | Based on directional antenna for UT described in Clause 7.3 for each supported UT antenna configuration |
| UT Polarized antenna modelling | Following the Clause 7.3.2 of TR38.901 |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the UT | Case 1: one hand grip Case 2: dual hand grip Case 3: head and one hand grip Note: For each UE, the selection of case is determiend by the probability defined in Table 7.5.14-2. |
| Metric | - CDF of coupling loss (serving cell) received per UT port - CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest) PRB singular value and the 1st PRB (largest) singular value (serving cell) at t=0 plotted in 10*log10 scale. - Note-1: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. - Note-2: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) |

Agreement

The following assumption is used for BS side SNS calibration:

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | Config1 for 7GHz: - (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; - (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Config2 for 15 GHz: - (M, N, P, Mg, Ng; MP, NP) = (64, 32, 2, 1, 1; 8, 16), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 of TR38.901 |
| UT Antenna Configuration | Config 1: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3. |
| UT antenna pattern | Isotropic |
| UT Polarized antenna modelling | Following the Clause 7.3.2 of TR38.901 |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the BS | Case 1: Physical blocker-based approach (e.g., K = [1 per sector]) Case 2: Stochastic-based approach |
| Metric | 1) CDF of coupling loss (serving cell). 2) CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest) PRB singular value and the 1st PRB (largest) singular value (serving cell) at t=0 plotted in 10*log10 scale. - Note-1: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. - Note-2: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) |

##### 2.1.1.2	Conclusions/Observations/Agreements from RAN1 #121

Agreement

- Copy the following summary of observations, conclusions, and agreements to the cover sheet of the measurement data source Tdoc, intended to be captured as reference to the TR.

Based on measurement data provided, RAN1 concludes that following channel modeling parameters are validated and no updates to the TR are made. - Pathloss - InH-Office LOS and NLOS - RMa LOS and NLOS - InF LOS and NLOS - UMi LOS and NLOS - Delay Spread - InH LOS and NLOS - Shadow fading - InH LOS and NLOS - UMi NLOS - Angular spread - UMa LOS ZSD - UMa NLOS ZSD For the following channel modeling parameters, sources have observed both consistent and different measurements compared with model in TR38.901 v18.0.0. Due to lack of consensus, the following channel modeling parameters are not updated. - Pathloss - UMa LOS and NLOS - Delay spread - InF LOS and NLOS - Angular spread - ASD, ASA, ZSA, and ZSD of InH LOS and NLOS - ZSD of UMi LOS and NLOS - ZSA of UMa LOS - Shadow fading - UMi LOS While measurement of ZSD for rural macrocell deployments from a source observed lower ZSD values compared to ZSD for RMa at shorter distances, and measurement of ASD for rural macrocell deployments from a source observed lower ZSD values compared to ASD for RMa, there was no consensus to update RMa ASD and ZSD parameters due to lack of measurement data for each of LOS, NLOS, and O2I cases. For the following channel modeling parameters, RAN1 has identified necessary updates at least for 6 – 24 GHz frequency range. The updated parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI. The data points were divided into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz and weighted for processing. The data sets for each group was used to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1. Each group is given equal weightage. - Delay spread - UMi LOS and NLOS - UMa LOS and NLOS - Angular spread - ASA, ASD, ZSA of UMi LOS and NLOS - ASA, ASD, ZSA of UMa LOS and NLOS - Cluster ASD of UMa LOS, NLOS, and O2I - ASD of UMa O2I - ZSA of UMa NLOS For ASD of UMa O2I scenario, the update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz. UMa O2I ASD values of Rel-14 TR38.901 used values from UMa O2I ASD values of TR36.873. UMa O2I ASD values of TR36.873 used UMi O2I ASD values from ITU-RM.2135-1. IMT-2020 modeling used the same values from TR36.873. TR25.996 does not contain UMa O2I values. The UMi O2I ASD parameters were derived from Winner II report. For Cluster AoD spread of UMa LOS, NLOS, and O2I scenarios, the update parameter for LOS and NLOS was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1. Each group is given equal weightage. The update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz. The number of clusters for UMi was taken from Winner II which used the 95% CDF of the number of clusters assuming K-mean clustering algorithm for counting number of clusters. For material penetration loss model, IRR glass penetration loss model was updated based on single coating for IRR glass. For other materials, standard multi-panel glass, concrete, and wood, model was not updated. However, reference thickness of the materials were clarified. New material penetration loss model for plywood has been agreed. Angle scaling and shifting for CDL based channel model for link-level evaluations was corrected and updated. The number of clusters for UMi was taken from Winner II which appears to have use the 95%-tile value from the CDF of the number of clusters assuming K-mean clustering algorithm for counting number of clusters. For UMa NLOS case, the 90%-tile and maximum number of cluster measurements from sources was observed to be aligned. For UMa LOS and UMi LOS cases, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally higher than the values in current TR38.901 by 1 to 6 clusters. For InH LOS, InH NLOS, UMi NLOS, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally lower than the values in current TR38.901 by 1 to 6 clusters. For UMa LOS/NLOS, UMi LOS/NLOS, and InH LOS/NLOS cases, the mean number of cluster measurements were significantly lower than values in current TR38.901. For other fast fading channel model parameters not mentioned above, no observation and conclusions are made due to lack of measurement data inputs. Beyond changes to the existing channel model, the following new components were added: - new deployment scenario, suburban macro (SMa), - new antenna model for handheld UT and consumer premise equipment (CPE) UT, - absolute time of arrival modeling for InH, UMi, UMa, RMa, and SMa, - nearfield channel propagation, - spatial non-stationarity modeling for BS and UT.

Agreement

- Copy the excel sheet in R1-2504913 as additional tab sheets of the measurement data source excel file, intended to be captured as reference to the TR.

Agreement

- R1-2504696 to be captured as reference for TR38.901 when endorsed.

  - Note: R1-2504696 contains the latest update of measurement data sources for channel model as part of the Rel-19 channel modeling enhancements for 7-24 GHz SI.

Agreement

- Update the reference thickness of plywood to 1.75 cm.

Agreement

Add the following note to UMi and UMa pathloss:

- UMi and UMa pathloss formula converges for different frequencies when distances beyond the breakpoint distance is applied.

Conclusion

Angular spread parameters for InF scenarios are not studied further due to lack of measurement data inputs.

Agreement

- Confirm the working assumption (made in RAN1#120b) for absolute delay parameters of RMa scenario.

Agreement

- Adopt the following update for scaling factor for ZOA and ZOD generation

| # clusters | 8 | 10 | 11 | 12 | 14 | 15 | 19 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CθNLOS | 0.889 | 0.957 | 1.031 | 1.104 | 1.1072 | 1.1088 | 1.184 | 1.178 | 1.282 |

Agreement

- Update the physical blocker modeling parameters in Table 7.6.4.1-2 and 7.6.3.1-4 as follows:

Table 7.6.4.1-2: Blocking region parameters.

| Blocker index (k = 1, …, 4) |  |  |  |  | r |
| --- | --- | --- | --- | --- | --- |
| InH scenario | Uniform in [0o, 360o] | Uniform in [15o, 45o] | 90o | Uniform in [5o, 15o] | 2 m |
| UMi, UMa, SMa, RMa scenarios | Uniform in [0o, 360o] | Uniform in [5o, 15o] | 90o | 5o | 10 m |

Table 7.6.4.1-4: Spatial correlation distance for different scenarios.

| Spatial correlation distance dcorr in [m] for the random variable determining the centre of the blocker | UMi/UMa/SMa/RMa |  |  | UMa |  |  | RMa |  |  | InH |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS |
|  | 10 | 10 | 5 | 10 | 10 | 5 | 10 | 10 | 5 | 5 | 5 |

Agreement

- Confirm the working assumption (made in RAN1#120b) absolute delay parameters of SMa scenario.

Agreement

- Adopt the following updates to LSP correlation values for SMa LOS scenario:

  - Note: this is a copy of LSP correlation values for UMa LOS scenario.

|  | SF | K | DS | ASD | ASA | ZSD | ZSA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SF |  | 0 | -0.6 -0.4 | -0.5 | -0.5 | 0 | -0.8 |
| K |  |  | 0 -0.4 | 0 | 0 -0.2 | 0 | 0 |
| DS |  |  |  | 0 0.4 | 0.8 | 0.5 -0.2 | 0 |
| ASD |  |  |  |  | 0 | 0.5 | 0 |
| ASA |  |  |  |  |  | -0.3 | 0.4 |
| ZSD |  |  |  |  |  |  | 0 |
| ZSA |  |  |  |  |  |  |  |

Agreement

- Clarify outdoor UT for SMa are in-car deployments and subject to car O2I penetration loss.

Agreement

- For SMa scenario calibration, use value of 0% vegetation for LOS probability determination.

Agreement

- Revise d2D to d2D-out for LOS probability equations of SMa scenario.

Agreement

- Update the SMa pathloss equation as follows:

| SMa | LOS | PLSMa-LOS=PL110m≤d2D<dBPPL2dBP≤d2D≤5km, PL1=20log1040πdfc3+min0.03h1.72,10log10dd3D- min0.044h1.72, 14.77+0.002log10hdd3D, see note 5 PL2=PL1dBP+40log10dd3DdBP, see note 5 | For 10 m ≤ d2D < dBP, σSF=4 For dBP ≤ d2D < 5000 m, σSF=6 | 0.50.8≤fc≤ 37 GHz 10 m ≤ d2D < dBP dBP ≤ d2D < 5000 m 25 m < hBS < 35 m h = 10 m |
| --- | --- | --- | --- | --- |
|  | NLOS | PLSMa-NLOS=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10dd3D-3+20log10fc-3.2log1011.75hUT2-4.97 | σSF=8 | 0.50.8 ≤fc≤ 37 GHz 10 m ≤ d2D < 5000 m 1 m < hUT < 14 m h = 10 m, W = 10 m |

Agreement

- For SMa, d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UT considered to be inside commercial buildings, and between 0 and 10 m for UT considered to be inside residential buildings.

Agreement

- Low Loss, High loss, and Low Loss A penetration model are applicable for SMa.

  - Low Loss A penetration model is used for SMa scenario calibration purposes.

Agreement

- Confirm Working Assumption (made in RAN1#120b) for penetration loss model (low loss A model) applicable for SMa.

Agreement

- Update delay spread parameters for SMa scenario as follows

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -7.23 -7.42 | -7.12 -7.20 | -7.12 -7.20 |
|  | lgDS | 0.38 0.60 | 0.33 0.58 | 0.33 0.58 |

Agreement

- Confirm the working assumption for the following SMa parameters

| Scenarios |  | Sma (7 – 24 GHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Shadow fading (SF) [dB] | σSF | See Table 7.4.1-1 | See Table 7.4.1-1 | See Table 7.4.1-1 |
| Delay scaling parameter rt |  | 2.4 | 1.5 | 1.5 |
| XPR [dB] | µXPR | 8 | 4 | 4 |
|  | σXPR | 4 | 3 | 3 |
| Number of rays per cluster M |  | 20 | 20 | 20 |
| Cluster DS (cDS) in [ns] |  | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) |
| Per cluster shadowing std ξ [dB] |  | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 6 | 40 | 40 |
|  | ASD | 15 | 30 | 30 |
|  | ASA | 20 | 30 | 30 |
|  | SF | 40 | 50 | 50 |
|  | K | 10 | N/A | N/A |
|  | ZSA | 15 | 50 | 50 |
|  | ZSD | 15 | 50 | 50 |

Agreement

- Confirm working assumption (made in RAN1#120b) for channel bandwidth for large scale and full calibration and remove brackets.

Agreement

Update UMi LOS/NLOS ASD (Mean, Std) as follows

| Scenarios |  | Original TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | -0.05 log10(1+ fc) + 1.21 | -0.23 log10(1+ fc) + 1.53 | -0.05 log10(1+ fc) + 1.21 | -0.24 * log10(1+ fc) + 1.54 |
|  | σlgASD | 0.41 | 0.11 log10(1+ fc) + 0.33 | 0.08 * log10(1+ fc) + 0.29 | 0.10 * log10(1+ fc) + 0.33 |

Update UMi LOS/NLOS ZSA (Mean, Std) as follows

| Scenarios |  | Original TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | -0.10 log10(1+ fc) + 0.73 | -0.04 log10(1+ fc) + 0.92 | -0.11 log10(1+ fc) + 0.81 | -0.03 log10(1+ fc) + 0.92 |
|  | σlgZSA | -0.04 log10(1+ fc) + 0.34 | -0.07 log10(1+ fc) + 0.41 | -0.03 log10(1+ fc) + 0.29 | -0.05 log10(1+ fc) + 0.35 |

Update UMa LOS ZSA (Mean, Std) as follows

| Scenarios |  | Original TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | 0.95 | -0.3236 log10(fc) + 1.512 | 0.96 | Updated in RAN1#120b |
|  | σlgZSA | 0.16 | 0.16 | 0.15 | Updated in RAN1#120b |

Update UMa LOS/NLOS ASD (Mean, Std) as follows

| Scenarios |  | Agreement from RAN1 #120b UMa |  |  | Updated UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 0.95 | 1.12 | 0.84 | 0.92 | 1.09 | 0.58 |
|  | σlgASD | 0.31 | 0.42 | 0.42 | 0.31 | 0.44 | 0.70 |

Note: Same parameter derivation methodology as noted in RAN1 #120-bis for each parameter were re-used. For UMa ASD, the updates to previous agreement are made based on new measurement data inputs in RAN1 #121.

Agreement

Update the UMa O2I AOD spread as follows.

- Note: This would change the working assumption made in RAN1 #120-bis.

- Note: No UMa O2I data was provided in Rel-14. The UMa ASD O2I values were adopted from UMi ASD O2I in TR 38.901. The UMi ASD O2I in TR 38.901 was in turn adopted from winner II channel model. In the Winner II channel model only a single measurement at 5.25 GHz for UMi O2I ASD.

| Scenarios |  | UMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | Not part of this agreement | Not part of this agreement | 0.84 0.58 |
|  | σlgASD | Not part of this agreement | Not part of this agreement | 0.42 0.7 |

Agreement

Update the UMa LOS, NLOS, and O2I Cluster AOD spread as follows.

- Note: This would change the working assumption made in RAN1 #120-bis.

- Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit OR compute weighted mean. If a group has fewer data points, higher weight per data point is calculated. All points within a group have the same weight. Sum of weights for all groups is equal to 1.  Each group is given equal weightage.

| Scenarios | UMa |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS | O2I |
| Cluster ASD (cASD) in [deg] | 3.74 3.58 | 1.82 1.8 | 1.82 1.8 |

Observation:

- Measurement of ZSD for rural macrocell deployments from a source observed lower ZSD values compared to ZSD for RMa at shorter distances.

- Measurement of ASD for rural macrocell deployments from a source observed lower ZSD values compared to ASD for RMa.

Conclusion:

- No consensus to update RMa ASD and ZSD parameters due to lack of measurement data for each of LOS, NLOS, and O2I cases.

Agreement

- Introduce an optional Mmin parameter to potentially bound the number of rays per clusters for equation (7.6-8) of intra-cluster angular and delay spreads.

M=minmaxMtMAODMZOD,20 Mmin,Mmax (7.6-8) Mmin can be chosen based on the mean of dominant rays across all clusters for a specific frequency and deployment scenario. For example, the number of dominant rays in a cluster is defined as the minimum number of rays that contain 95% of the total cluster power, when the rays in a cluster are sorted in descending order of power. The default value of Mmin=20 is assumed.

Agreement

Add the following angle scaling formula for CDL-E and CDL-D that allow maintaining the LOS angle as the desired scaled LOS angle:

- ϕn,m,scaled=ϕn,intermediate-μϕ,intermediateϕLOS,intermediate+μϕ,desiredϕLOS,model+cASA/ASD/ZSA/ZSDαm,

- ϕLOS,scaled=ϕLOS,intermediate-μϕ,intermediate+μϕ,desiredϕLOS,model

- ϕn,intermediate=s*WrapTo180ϕn,model-μϕ,model,

- ϕLOS,intermediate=s*WrapTo180ϕLOS,model-μϕ,model,

- Note: This will be an additional (and alternative) formulation for CDL-D and CDL-E angle scaling.

Agreement

- Update ASA and ZSA parameters for SMa scenario as follows:

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOA spread (ASA) lgASA=log10(ASA/1°) | µlgASA | 1.07 1.43 | 1.39 1.63 | 1.39 1.63 |
|  | σlgASA | 0.13 0.12 | 0.34 0.26 | 0.34 0.26 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1°) | µlgZSA | 0.95 1.18 | 1.14 1.16 | 1.14 1.16 |
|  | σlgZSA | 0.16 0.05 | 0.16 0.14 | 0.16 0.14 |
| Cluster ASA (cASA) in [deg] |  | 5 | 5 10 | 10 |
| Cluster ZSA (cZSA) in [deg] |  | 7 | 7 | 7 |

Note: The updated SMa results for ZSA was based on a single source measurement data.

Agreement

- Update ASD parameters for SMa scenario and confirm the working assumption for ZSD parameters as follows:

| Scenarios |  |  | SMa |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1°) |  | µlgASA | Not part of this agreement | 0.55 0.73 0.51 | 0.55 0.73 0.51 |
|  |  | σlgASA | Not part of this agreement | 0.25 0.31 0.33 | 0.25 0.31 0.33 |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1°) |  | µlgZSA | 0.14 | 0.14 | 0.14 |
|  |  | σlgZSA | 0.16 | 0.16 | 0.16 |
| Cluster ASD (cASD) in [deg] |  |  | Not part of this agreement | 5 1.75 | 10 1.75 |
| ZOD offset | µoffset,ZOD |  | 0 | atanhBS-hcld2D-atanhBS-hUTd2DhUT<hcl0hUT≥hcl hcl= [3.5] |  |

Agreement

- Revise the Rx heigh example for UMa, UMi, and SMa in scenario description in Section 6.2 as follows:

  - UMi

    - “Example: [Tx height:10m, Rx height with reference to floor height: 1.5-2.5 m, ISD: 200m]”

  - UMa

    - “Example: [Tx height:25m, Rx height with reference to floor height: 1.5-2.5 m, ISD: 200m, 500m]”

  - SMa

    - “Example: [Tx height: 35m, Rx height with reference to floor height: 1.5-2.5m, ISD: 1299m, 1732m]”

Agreement

- Confirm the working assumption for the following SMa parameters

| Scenarios | Sma (7 – 24 GHz) |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS | O2I |
| Number of clusters N | 15 | 14 | 14 |

Agreement

- Update “UT array orientation” in TR 38.901 as “UT orientation”

Agreement (replaces previous agreement in RAN1 #121)

- Update the SMa pathloss equation as follows:

| SMa | LOS | PLSMa-LOS=PL110m≤d2D<dBPPL2dBP≤d2D≤5km, PL1=20log1040πdd3Dfc3+min0.03h1.72,10log10dd3D- min0.044h1.72, 14.77+0.002log10hdd3D, see note 5 PL2=PL1dBP+40log10dd3DdBP, see note 5 | For 10 m ≤ d2D < dBP, σSF=4 For dBP ≤ d2D < 5000 m, σSF=6 | 0.50.8≤fc≤ 37 GHz 10 m ≤ d2D < dBP dBP ≤ d2D < 5000 m 25 m < hBS < 35 m h = 10 m |
| --- | --- | --- | --- | --- |
|  | NLOS | PLSMa-NLOS=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10dd3D-3+20log10fc-3.2log1011.75hUT2-4.97 | σSF=8 | 0.50.8 ≤fc≤ 37 GHz 10 m ≤ d2D < 5000 m 1 m < hUT < 14 m h = 10 m, W = 10 m |

Agreement

- Update ASD parameters for SMa scenario as follows:

  - Note: The cluster ASD values in this agreement replaces the previous agreement in RAN1 #121.

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1°) | µlgASA | 0.55 0.48 | Not part of this agreement | Not part of this agreement |
|  | σlgASA | 0.25 0.27 | Not part of this agreement | Not part of this agreement |
| Cluster ASD (cASD) in [deg] |  | 5 2.08 | 1.75 1.33 | 1.75 1.33 |

Agreement

- Add the following note to SMa description

  - NOTE 1: SMa scenarios with ISDs between 1200-1800m can be used for evaluations

Agreement

Clarify downtilt simulation assumption parameter for calibrations as follows:

- For SMa,

  - downtilt refers to mechanical downtilt and no electrical downtilt applies.

  - Update the “Working assumption of 93 degree” to 92 degree for downtilt for SMa deployment scenario for ISD = 1732m.

- For InH,

  - Downtilt refers to mechanical downtilt and no electrical downtilt applies

- For UMa and UMi,

  - No mechanical downtilt applies, and downtilt values refer to electrical downtilt

Agreement

- Update the reference orientation of the handheld UE as follows:

  - Note: The representation will be same between GCS and UE LCS for ΩUT,α = 0 deg, ΩUT,β = 0 deg, ΩUT,γ = 0 deg,

- For calibration of handheld UE, use the following UE rotation based on reference UE orientation

  - For calibration with blockage:

    - For one-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

    - For dual-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 0 deg, ΩUT,γ = 45 deg,

    - For hand and head blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg,

  - For all other calibration cases:

    - ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

  - Note: UT orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle).

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 0 deg, ΩUT,γ = 90 deg

Agreement

- Reference radiation pattern of the UE antenna model is vertically polarized with all the gain in the theta field component, Fθ″″θ″,ϕ″=A″θ″,ϕ″ and Fϕ″″θ″,ϕ″=0 and referred to as polarization direction along the Z’’ axis.

- For cases when a candidate antenna placement location is used for one antenna field pattern (e.g., single polarization):

  - The polarization direction is indicated by the arrow in the following figure which is parallel with the plane of the handheld UE and perpendicular to the direction from the UE center to the candidate antenna location

- For cases when a candidate antenna placement location is used for two antenna field patterns (e.g., dual polarization) (not intended for FR1):

  - For the first antenna field pattern, the polarization direction is indicated by the arrows in the above figure but additionally rotated 45 degrees about the direction from the UE center to the candidate antenna location (i.e. rotated using the direction from the UE center to the candidate antenna location as the rotational axis).

  - For the second antenna field pattern, the polarization direction is perpendicular to the polarization direction of the first filed pattern and perpendicular to the direction from the UE center to the candidate antenna location

  - An example for candidate antenna location (6) is given in the figure below.

- RAN1 to perform calibration of antenna field pattern (Fu,θ''(θ',ϕ'), Fu, ϕ''(θ',ϕ')) for 8 antenna handheld UE in the UE local coordinate system

Agreement

Amend the previous agreement (from RAN1 #120-bis) as follows:

- Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation and polarization direction of the each of UE antennae to get Frx,u,θ''(θ',ϕ'), Frx,u, ϕ''(θ',ϕ') and based on the orientation of the UE in the global coordinate system to get Frx,u,θ(θ,ϕ) and Frx,u,ϕ(θ,ϕ) using the methods already specified in TR 38.901, equation (7.3-3)/(7.1-11).

Observation:

- The number of clusters for UMi was taken from Winner II which appears to have use the 95%-tile value from the CDF of the number of clusters assuming K-mean clustering algorithm for counting number of clusters.

- For UMa NLOS case, the 90%-tile and maximum number of cluster measurements from sources was observed to be aligned.

- For UMa LOS and UMi LOS cases, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally higher than the values in current TR38.901 by 1 to 6 clusters.

- For InH LOS, InH NLOS, UMi NLOS, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally lower than the values in current TR38.901 by 1 to 6 clusters.

- For UMa LOS/NLOS, UMi LOS/NLOS, and InH LOS/NLOS cases, the mean number of cluster measurements were significantly lower than values in current TR38.901.

Agreement

For section 7.6, introduce an optional modeling component for number of clusters

- Optional modeling components are used to evaluate a channel propagation environment with diverse cluster ranges.

Table: Number of clusters for the diverse cluster environment

| Scenario | LOS | NLOS | O2I |
| --- | --- | --- | --- |
| UMi | D1: 6, D2: 12 | D1: 6, D2: 19 | D1: 6, D2: 12 |
| UMa | D1: 10, D2: 12 | D1: 15, D2: 20 | D1: 10, D2: 12 |
| InH | D1: 7, D2: 15 | D1: 6, D2: 19 | N/A |

- Number of clusters is chosen between a closed range of [D1, D2]. The selection of the number of cluster for each link is determined by the user. This is used as the number of clusters in Table 7.5.6-Part 1 and Part 2.

- Rapporteur to reference METIS channel model for this optional modeling component.

- Note: D1 is the average of the mean of the number of clusters from the source data. D2 is the values from 38.901

- Note: O2I values for D1 and D2 is the minimum of LOS or NLOS value

Agreement

Introduce additional cluster angle scaling factors as follows:

Table 7.5-2: Scaling factors for AOA, AOD generation

| # clusters | 4 | 5 | 6 | 7 | 8 | 10 | 11 | 12 | 14 | 15 | 16 | 19 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CφNLOS | 0.779 | 0.860 | 0.921 | 0.973 | 1.018 | 1.090 | 1.123 | 1.146 | 1.190 | 1.211 | 1.226 | 1.273 | 1.289 | 1.358 |

Table 7.5-4: Scaling factors for ZOA, ZOD generation

| # clusters | 6 | 7 | 8 | 10 | 11 | 12 | 14 | 15 | 16 | 19 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CφNLOS | 0.788 | 0.847 | 0.889 | 0.957 | 1.031 | 1.104 | 1.1072 | 1.1088 | 1.1276 | 1.184 | 1.178 | 1.282 |

Agreement

- Introduce polarization variability for each rays of cluster for NLOS component of the channel as an optional modeling component.

Generate polarization variability powers ηθθ, ηθϕ, ηϕθ and ηϕϕ for each ray m of each cluster n. η is log-Normal distributed. Draw values as

η'n,m,θθ,θϕ,ϕθ,ϕϕ=10Qn,m,θθ,θϕ,ϕθ,ϕϕ/10,	(7.5-21b)

ηn,m,θθ,θϕ,ϕθ,ϕϕ=η'n,m,θθ,θϕ,ϕθ,ϕϕ2+2κn,m-1η'n,m,θθ+η'n,m,ϕϕ+η'n,m,θϕκn,m-1+η'n,m,ϕθκn,m-1	(7.5-21c)

where Qn,m,θθ,θϕ,ϕθ,ϕϕ~N(0,32) is Gaussian distributed.

Note that Qn,m,θθ,θϕ,ϕθ,ϕϕ is independently drawn for each ray, cluster, and polarization component.
Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATηn,m.θθexpjΦn,mθθηn,m,θϕκn,m-1expjΦn,mθϕηn,m,ϕθκn,m-1expjΦn,mϕθηn,m,ϕϕexpjΦn,mϕϕ

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT⋅drx,uλ0expj2πrtx,n,mT⋅dtx,sλ0expj2πrrx,n,mT⋅vλ0t	(7.5-22)

Note: The model is at least applicable for model-2 antenna polarization modeling.

Agreement

- Add new antenna Config C (with BS config 4) for 15 GHz for UT as optional configuration as part of full calibration assumption

  - (optional) Config C for 15 GHz: 16 antenna port with dual polarization based on handheld device antenna model using feasible candidate antenna locations in (1,2,3,4,5,6,7,8) as described in Clause 7.3.

  - Note: other UT configuration are not applicable for 15 GHz.

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted:

- To calculate the power attenuation factor for the clusters with SNS, the definition of xs, ys in RAN1#120bis agreement is revised as:

  - The xs, ys denotes the coordinate of the antenna element out of VR region.

- To generate the visibility region, the Pn refers to the power of cluster generated by (7.5-6) in dB scale. In the case of LOS condition, the LOS path is considered as an additional cluster, and the power ratio of the LOS path to NLOS clusters follows the Ricean K-factor generated in Section 7.5.

Agreement

The following value in the agreement of RAN1#120bis is revised as:

- The probability of VR at the upper part of antenna array is [0.8] 0.5 and the probability of VR at the lower part of antenna array is [0.2] 0.5;

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, the following parameters are adopted:

| Scenario | A | B | R | δ | C | PrSNS(μ,σ2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | μ | σ |
| UMa | 0.15 | 0.45 | 33 | Nμ,σ2~N0, 0.0015 | 13 | 0.56 | 0.20 |
| UMi | 0.12 | 0.48 | 50 | N(μ,σ2)~ N(0, 0.001) | 13 | 0.49 | 0.18 |
| InH | 0 | 0.60 | NA | N(μ,σ2)~N0, 0.0011 | 13 | 0.31 | 0.08 |

Agreement

To calculate the absolute time of arrival for d1 and d2, the Δτ refers to the excess delay, which is only applicable

- when it’s not in the LOS case, and generated according to the Section 7.6.9, otherwise Δτ is assumed to be 0.

Agreement

For the modelling of spatial non-stationarity at UE side, the value of the attenuation values for “Head and one hand grip” in the agreement made in RAN1#120bis is revised in Red:

| Antenna index | Power attenuation (dB) |  |  |
| --- | --- | --- | --- |
|  | Head and one hand grip |  |  |
|  | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz |
| 1 | - | 3.7 | 4.0 |
| 2 | - | 4.1 4.6 | 3.7 |
| 3 | - | 4.3 | 3.3 |
| 4 | 15.1 | 7.8 | 3.3 |
| 5 | - | 11.7 | 3.3 |
| 6 | - | 10.1 | 3.7 |
| 7 | - | 2.9 | 4.0 |
| 8 | 4.9 | 4.2 | 4.0 |

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, the following parameters are adopted:

| Scenario | A | B | R | δ | C | PrSNS(μ,σ2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | μ | σ |
| RMa | 0.16 | 0.74 | 60 | Nμ,σ2~N0, 0.0016 | 13 | 0.14 | 0.08 |
| SMa | 0.06 | 0.56 | 23 | Nμ,σ2~N0, 0.0013 | 13 | 0.24 | 0.07 |
| InF | 0 | 0.57 | NA | Nμ,σ2~N0, 0.002 | 13 | 0.32 | 0.06 |

Agreement

For the following agreement made in RAN1#121, it’s to confirm that the “new blocker type/size” is only applicable for SNS.

Agreement For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Billboard; Cartesian: w=2.4m; h=3.6m; Stationary. Outdoor; Street lamp; Cartesian: w=0.4m; h=0.8m; Stationary. Outdoor; Building edge; Cartesian: w=X m; h=Y m; Stationary. Indoor; Pillar; Cartesian: w=0.3m; h=3m; Stationary. - FFS: the value of X and Y for the blocker dimensions of building edge is needed. - FFS: the details related to the user hand/head: Indoor;Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor;Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS:The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head

Agreement

For the modelling of SNS, capture the following observation & recommendation in the CR:

- In Rel-19 Channel Model study,

  - the VR/VP based approach is recommended to be used in the simulation except for the case where it is not applicable

- From the perspective of the simulation requirements:

  - The physical blocker based approach can be considered for simulation where physical accuracy and consistency is desired.

  - The visibility region-based approach (i.e., Stochastic-based approach) can be considered for simulation where computational efficient SnS modelling is desired.

- From the perspective of the model mechanism:

  - To reflect the SNS phenomenon due to partial blockage, the physical blocker-based approach can be considered in simulation.

  - To reflect the SNS phenomenon due to incomplete scattering, the visible probability and visibility region-based approach (i.e., Stochastic-based approach) can be considered in simulation.

Agreement

For the terminology used in the CR:

- Keep the terminology “Near field channel model” with following updates, capturing the following TP in section 6.4:

< Unchanged text omitted > - Support near-field channel propagation (i.e., characteristics of spherical wavefront) and spatial non-stationarity < Unchanged text omitted >

##### 2.1.1.3	Agreements from Post-RAN1 #121 Email Discussion

Agreement:

Source data Tdoc in R1-2504960 endorsed to be captured as reference into TR38.901.

Draft CR in R1-2504688 Endorsed.

Final CR for TS38.901 CR0026 in R1-2504964 is Agreed.

#### 2.1.2	Remaining Open issues

None.

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

Reference from RAN1 #120-bis

- R1-2409402, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2409403, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2409441, “Discussion on validation of channel model,” Ericsson

- R1-2409442, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2409467, “Further Discussion on Angle Scaling for MIMO CDL Channel,” InterDigital, Inc.

- R1-2409468, “Further Discussion on Channel Model Extension for FR3,” InterDigital, Inc.

- R1-2409491, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2409610, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2409611, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2409694, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2409695, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2409724, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2409725, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2409738, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2409739, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2409778, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2409779, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2409819, “Validation of Channel Model,” Apple

- R1-2409821, “Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2409954, “Discussion on channel model validation for 7-24GHz,” CATT

- R1-2409955, “Discussion on channel model adaptation/extension for 7-24GHz,” CATT

- R1-2410015, “Channel model validation for 7-24 GHz,” Lenovo

- R1-2410016, “Channel model extension for 7-24 GHz,” Lenovo

- R1-2410099, “Channel model adaptation and extension for 7-24GHz,” OPPO

- R1-2410131, “Discussion on channel model adaptation/extension for 7-24 GHz,” Fujitsu

- R1-2410170, “Draft CR/TP to incorporate the agreement for Rel-19 7-24GHz Channel model,” ZTE Corporation,Intel Corporation

- R1-2410213, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT

- R1-2410214, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC

- R1-2410236, “Further Discussion of channel model validation of TR38.901 for 7–24GHz,” Sony

- R1-2410237, “Further discussion of channel model adaptation/extension of TR38.901 for 7–24GHz,” Sony

- R1-2410254, “Instantaneous directional channel measurements and parameter estimation at 14 GHz,” ROHDE & SCHWARZ

- R1-2410299, “Measurements of the angular spreads in a urban and suburban macrocells,” Vodafone, Ericsson

- R1-2410319, “Channel model validation of TR38.901 for 7-24 GHz,” Sharp, Nokia

- R1-2410320, “Views on channel modelling adaptation/extension for 7-24GHz,” Sharp, NYU WIRELESS

- R1-2410334, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2410402, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2410491, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2410492, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2410520, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek Inc.

- R1-2410594, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2410595, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2410628, “Channel model adaptation/extension of TR38.901 for 7-24 GHz,” CEWiT

- R1-2410631, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2410632, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410633, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410634, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410635, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410636, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2410655, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2410656, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2410767, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2410768, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2410769, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE Corporation)

- R1-2410910, “Summary #5 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

Reference from RAN1 #121

- R1-2500087, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2500088, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2500122, “Discussion on Evaluation of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2500123, “Discussion on Extension of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2500236, “Views on channel model validation for 7-24GHz,” CATT

- R1-2500237, “Views on channel model adaptation/extension for 7-24GHz,” CATT

- R1-2500252, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2500362, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2500363, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2500380, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2500381, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2500393, “Channel model validation of TR38.901 for 7-24 GHz,” Sharp

- R1-2500409, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2500410, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2500411, “Draft CR for Rel-19 7-24GHz Channel model,” Intel Corporation, ZTE Corporation

- R1-2500464, “Channel model adaptation and extension for 7-24GHz,” OPPO

- R1-2500506, “Discussion on validation of channel model,” Ericsson

- R1-2500507, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2500541, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT, Spark NZ Ltd

- R1-2500542, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC

- R1-2500561, “Discussions on FR3 Channel Modelling,” Lekha Wireless Solutions

- R1-2500627, “Near-field channel modelling for FR3,” VIAVI Solutions

- R1-2500628, “Near-field channel modelling for FR3,” VIAVI Solutions

- R1-2500640, “Measurements of the angular spreads in urban and suburban macrocells,” Vodafone, Ericsson

- R1-2500661, “Discussion of channel model validation of TR38.901 for 7-24GHz,” Sony

- R1-2500677, “Channel model validation for 7-24 GHz,” Lenovo

- R1-2500678, “Channel model extension for 7-24 GHz,” Lenovo

- R1-2500693, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2500694, “Channel model adaptation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2500798, “On Validation of Channel Model,” Apple

- R1-2500799, “On Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2500862, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2500863, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2501028, “Discussion on channel modelling enhancements for 7-24GHz for NR,” MediaTek Inc.

- R1-2501048, “Intra cluster power profile,” Keysight Technologies UK Ltd

- R1-2501083, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2501168, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2501169, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2501213, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2501281, “Discussion of channel model adaptation/extension of TR38.901 for 7-24GHz,” Sony

- R1-2501348, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2501349, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2501371, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2501426, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2501427, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501428, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501429, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501430, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501431, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2501482, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501483, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501484, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501485, “Summary#4 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2501549, “Session notes for 9.8 (Study on channel modelling enhancements for 7-24GHz for NR),” Ad-Hoc Chair (CMCC)

- R1-2501638, “Summary#5 of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

Reference from RAN1 #120-bis and RAN1 #121

- R1-2501749, “Remaining Issues on Evaluation of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2501750, “Remaining Issues on Extension of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2501755, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2501819, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2501820, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2501897, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2501898, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2501936, “Channel model adaptation of TR 38.901 for 7-24 GHz,” NVIDIA

- R1-2501937, “Channel model validation of TR 38.901 for 7-24 GHz,” NVIDIA

- R1-2501955, “Discussions on FR3 Channel Modelling,” Lekha Wireless Solutions

- R1-2502004, “On channel model validation for 7-24GHz,” CATT

- R1-2502005, “On channel model adaptation/extension for 7-24GHz,” CATT

- R1-2502186, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT, Spark NZ Ltd, X-Net

- R1-2502187, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GH,”	BUPT, CMCC, X-Net

- R1-2502217, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2502218, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2502287, “Channel model adaptation and extension for 7-24GHz,” OPPO

- R1-2502327, “Further discussion of channel model validation of TR38.901 for 7-24GHz,” Sony

- R1-2502328, “Further discussion of channel model adaptation/extension of TR38.901 for 7-24GHz,” Sony

- R1-2502339, “Channel model validation for 7-24 GHz,” Lenovo

- R1-2502340, “Channel model extension for 7-24 GHz,” Lenovo

- R1-2502341, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2502342, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2502343, “Draft CR for Rel-19 7-24GHz Channel model,” Intel Corporation, ZTE Corporation

- R1-2502380, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2502381, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2502415, “Curve fittings for UMi & UMa scenarios: DS, ASA, ASD LOS/NLOS,” Sharp, Nokia, Ericsson, Apple, BUPT

- R1-2502625, “Validation of Channel Model,” Apple

- R1-2502626, “Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2502737, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2502751, “Views on Channel model validation of TR38.901 for 7-24 GHz,” Sharp

- R1-2502777, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2502851, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2502852, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2502878, “Discussion on validation of channel model,” Ericsson

- R1-2502884, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2502899, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2502900, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2502909, “Measurements of the angular spreads in a urban and suburban macrocells,” Vodafone, Ericsson

- R1-2502951, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT, X-Net

- R1-2502957, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2502986, “Views on Channel model validation of TR38.901 for 7-24 GHz,” Sharp

- R1-2503024, “Summary#1 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2503025, “Summary#2 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2503026, “Summary#3 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2503027, “Summary#4 of channel model adaptation and extension,” Moderator (ZTE)

- R1-2503060, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation)

- R1-2503061, “Summary #1 of discussions for draft CR for 7-24 GHz SI,” Moderator (Intel Corporation)

- R1-2503062, “Summary #2 of discussions for draft CR for 7-24 GHz SI,” Moderator (Intel Corporation)

- R1-2503063, “Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation

- R1-2503064, “Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2503065, “Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2503066, “Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2503067, “Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2503114, “Session notes for 9.8 (Study on channel modelling enhancements for 7-24GHz for NR),” Ad-Hoc Chair (CMCC)

- R1-2503129, “Draft CR for Rel-19 7-24GHz Channel model,” Intel Corporation, ZTE Corporation

- R1-2503130, “Summary #5 of discussions for Rel-19 7-24 GHz Channel Modeling Validation,” Moderator (Intel Corporation)

- R1-2503256, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2503257, “Considerations on the 7-24GHz channel model extension,” Huawei, HiSilicon

- R1-2503374, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2503375, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2503427, “Discussion on channel modelling adaptation/extension for 7-24GHz,” LG Electronics

- R1-2503439, “Remaining Details of Evaluation of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2503440, “Remaining Details of Extension of FR3 Channel Modeling,” InterDigital, Inc.

- R1-2503578, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2503579, “Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2503605, “Draft CR for Rel-19 7-24GHz Channel model,” Intel Corporation, ZTE Corporation

- R1-2503606, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2503620, “Discussion on validation of channel model,” Ericsson

- R1-2503621, “Discussion on adaptation and extension of channel model,” Ericsson

- R1-2503633, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2503634, “Discussion on the channel model adaptation and extension,” ZTE Corporation, Sanechips

- R1-2503647, “Discussion on channel model adaptation/extension,” Intel Corporation

- R1-2503653, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT

- R1-2503654, “Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz,” BUPT, CMCC, X-Net

- R1-2503714, “Discussions on FR3 Channel Modelling,” Lekha Wireless Solutions

- R1-2503747, “LS to CTIA MOSG on TR38.901 updates,” Spirent Communications, Keysight Technologies

- R1-2503762, “Discussion on Channel Model Validation of TR38.901 for 7-24GHz,” SK Telecom

- R1-2503805, “Views on channel model validation for 7-24GHz,” CATT

- R1-2503806, “Views on channel model adaptation/extension for 7-24GHz,” CATT

- R1-2503962, “Views on Channel Model Validation,” Sharp

- R1-2503993, “Channel model validation of TR 38.901 for 7-24 GHz,” NVIDIA

- R1-2503994, “Channel model adaptation of TR 38.901 for 7-24 GHz,” NVIDIA

- R1-2504006, “Channel model validation for 7-24 GHz,” Lenovo

- R1-2504147, “Discussion on calibration results,” ETRI

- R1-2504222, “Channel model adaptation and extension for 7-24GHz,” OPPO

- R1-2504338, “Validation of Channel Model,” Apple

- R1-2504339, “Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz,” Apple

- R1-2504368, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2504406, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2504407, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2504512, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2504548, “Measurements of the angular spreads in urban and suburban macrocells,” Vodafone, Ericsson

- R1-2504629, “Discussion of channel model validation of TR38.901 for 7-24GHz,” Sony

- R1-2504630, “Discussion of channel model adaptation/extension of TR38.901 for 7-24GHz,” Sony

- R1-2504631, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

- R1-2504632, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz,” Nokia

- R1-2504650, “Measurements of the angular spreads in urban, suburban, and rural macrocells,” BT plc, Ericsson

- R1-2504684, “Channel Model Adaptation/Extension of TR38.901 for 7-24GHz,” Qualcomm Incorporated

- R1-2504685, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2504686, “Views on channel model adaptation/extension of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2504688, “Draft CR for Rel-19 7-24GHz Channel model,” Intel Corporation, ZTE Corporation

- R1-2504717, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2504721, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template