3GPP TSG RAN meeting #108										RP-251807

Prague, Czech Republic, June 9-13, 2025

Status Report to TSG

Agenda item:			9.2.2

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_Sensing_NR |  |  |  |  |
| Unique ID | 1020086 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242348 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2025 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 100% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Yingyang Li, Jerome Vogedes |
|  | Company | Xiaomi, AT&T |
|  | Email | liyingyang@xiaomi.com, jerome.vogedes@att.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #120bis, Wuhan, China, Apr. 7-11, 2025

ISAC deployment scenarios

Agreement

For the purposes of large scale calibration for UAV sensing targets, the following revised calibration parameters are proposed below in Table x. Note that the change bars are against the agreements from RAN1#120.

Table x. Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| UT height | 1.5m for terrestrial UTs, |
| UT Tx power | 23dBm |
| UT Distribution | • The overall number of UTs is 30 uniformly distributed in the center cell. • All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. • Vertical distribution of aerial UE: Fixed height value of 200 m. • FR1 is assumed for aerial UE. |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component A included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel |

Agreement

For the purposes of full calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x.

Table x. Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| UT height | 1.5m for terrestrial UTs |
| UT Tx power | 23dBm |
| UT Distribution | - The overall number of UTs is 30 uniformly distributed in the center cell. - All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. - Vertical distribution of aerial UE: Fixed height value of 200 m. - FR1 is assumed for aerial UE. |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| RCS for each scattering point | Component A: -12.81 dBsm Component B1: 0 dB Component B2: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Fast fading model | TR 36.777 Annex B.1.3 |
| (u, std) for XPR of target | Mean 13.75 dB, deviation 7.07 dB |
| The power threshold for path dropping after concatenation for target channel | FFS |
| The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel | FFS |
| Coupling loss for target channel | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Sensing Tx/Rx selection | Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel. |
| Absolute delay | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref] is reused for UMa-AV for all sensing modes. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996. |

Agreement

For the purposes of large scale calibrations for Automotive sensing targets, the following parameters are proposed below in Table x.

  - FFS: which type of UE is used for UT in different sensing mode

  - FFS: impact of spatial consistency, if any, in case of vehicle with 5 scattering points

  - FFS: cell layout for ISD = 250 m

Table x. Simulation assumptions for large scale calibration for Automotive sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | For FR1: Urban Grid (ISD=500m, BS height=25m) Highway (ISD=1732m, BS height=35m) For FR2: Urban Grid (ISD=250m, BS height=25m) Highway (ISD=500m, BS height=35m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic, UE monostatic |
| Target type | Vehicle type 2 [TR37.885] |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | Single dual-pol isotropic antenna, (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| UT height | 1.5m for pedestrian type UE 5m for RSU type UE 1.6m for vehicle type UE |
| UT Tx power | 23dBm |
| UT Distribution | Per TR37.885 |
| Sensing target distribution | Per TR37.885: - Option A - Vehicle type distribution: 100% vehicle type 2. - Clustered dropping is not used. - Highway: one target uniformly distributed (across multiple drops) within the simulation region. Vehicle speed is 140 km/h in all the lanes as baseline. - Urban Grid: one target is uniformly distributed (across multiple drops) within the center road grid. Vehicle speed is 60 km/h in all the lanes as baseline. NOTE: vehicle is dropped with 5 scattering points (front/left/right/back/roof) and each point has one location, or vehicle is dropped with 1 scattering points |
| Component A of the RCS for each scattering point | -20dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | As defined in urban grid/highway scenario |
| Coupling loss for target channel | Power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N= Tx-Rx pairs to be selected for the target. For urban grid N = 4 For Highway N = 4 NOTE: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel |

ISAC channel modelling

Agreement

In order to generate Tx-target link, target-Rx link and the background channel, the above table on reference TRs (excluding the already agreed part) is adopted for the mapping between reference TRs and a pair of nodes (STX, SRX, target)

  - Note: continue discussion for updating the table with RSU type UE

  - FFS: the generation of background channel based on reference TRs is subject to the addition of low-energy clusters

| Case | Node 1 | Node 2 | Existing TRs as starting point |
| --- | --- | --- | --- |
| 1 | TRP | TRP | Highway - TRP-UE link of scenario RMa in section 7 of TR 38.901 by setting hUE=35m for FR1 - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 Urban grid - TRP-TRP link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 HST - TRP-UE link of scenario RMa in section 7 of TR 38.901 by setting hUE=35m for FR1 - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 for FR2 |
| 4 | TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | normal UE | normal UE | For pedestrian type UE: Highway and Urban grid - P2P link in section 6 of TR 37.885 HST - TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1, e.g., hBS=1.5m, UE-UE link of scenario UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 for FR2 |
| 6 | normal UE | vehicle UE | UMi, UMa, RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m For pedestrian type UE: Highway and Urban grid - V2P link in section 6 of TR 37.885 |
| 7 | normal UE | aerial UE | UMi-AV, UMa-AV, and RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |
| 8 | vehicle UE | vehicle UE | Highway and Urban grid - V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - UE-UE link of scenario UMi, UMa following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m |
| 9 | aerial UE | aerial UE | UMi-AV, UMa-AV, RMa-AV - TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE for FR1 - LOS probability is not reused, FFS new LOS probability - FFS pathloss model, shadowing fading, angular spread - Working assumption: Reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 - The corresponding parameter values in FR2 are used |

Agreement

To generate the parameters (in the steps before concatenation), the large-scale parameters and the small-scale parameters used to generate the Tx-target link are respectively the same as that of the target-Rx link for monostatic sensing, where departure angle on one link and arrival angle on the other link are reciprocal.

  - FFS: whether this applies to initial phase

Agreement

Normalization on the product of three polarization matrixes of a direct/indirect path generated by stochastic cluster, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp is supported

  - The scaling factor is 2d112+d222

Agreement

Power normalization of target channel after path dropping of the target channel is not supported.

Agreement

On the monostatic RCS for human with RCS model 2

- The monostatic RCS for a scattering point of the target is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - FFS how many rows of the values/pattern A*B1 are defined for the target

    - Note: each row has a defined applicable range of θ and φ

  - Note: whether the RCS is elevation angle dependent or dependent on both elevation and horizontal angles can be separately discussed

Agreement

The following mean and standard deviation values of XPR of targets are agreed for monostatic sensing and bistatic sensing as follows:

- UAV: (13.75, 7.07) dB

- Human: (19.81, 4.25) dB

- Vehicle: (21.12, 6.88) dB

Agreement

When spatial consistency is enabled, the 1-by-1 random coupling generated by concatenation Option 3 is not updated per simulation drop even if Tx, target, Rx positions change during simulation.

Agreement

The following working assumption is confirmed

Working assumption Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF - Related model referring to Δτ values from 7-24GHz study item

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2) is applied to the  β within 0~180 degrees. k1= 6 and k2=1.65. β is the absolute bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - FFS: how to avoid angular discontinuity

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

Note: the working assumption agreed on Thursday was updated on Friday as follows: k1= 6 and k2=1.65

Agreement

On background channel for mono-static sensing, the following details are provided:

- Nrp=3 reference points are dropped for one Tx, based on the Gamma distribution for distance and height of reference point.

- The LOS AOD between Tx and the first reference point, which is denoted as AOD1, is generated based on uniform distribution unif-π,π.

- The LOS AOD between Tx and the second reference point is AOD1 + 2Nrpπ

- The LOS AOD between Tx and the third reference point is AOD1 + 4Nrpπ

- The background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point assuming NLOS condition.

- The antenna field pattern and array orientation of reference point are set same as Tx.

- Arrival angles for both azimuth and elevation φn,m,AOA and θn,m,ZOA are set equal to departure angles

- The absolute delay model d3D and Δτ as agreed for bistatic sensing for the same sensing scenario applies. Down-select one option from the following:

- Option 0: no scaling factor is applied to d3D

- Option 1: An offset is applied to d3D, i.e., d3D-c1

- Option 2: A scaling factor d_s is multiplied to d3D, i.e., d3D*d_s. d_s is a value within range [0, 1].

- Note: The adjustment of absolute delay doesn’t impact the generation of NLOS clusters between the Tx and each reference point

- The mono-static background channel for the Tx would be sum of channels of the links between the Tx and all related reference points, which is

HsNLOS(τ,t)=u=1NrpPLu,sNLOSHu,sNLOS(τ,t)

- FFS: Doppler frequency in background channel for monostatic sensing

- The rays in a stochastic cluster with ZOA at BS less than D degree are dropped

- D=[90] for RMa,

- D=[60] for UMa

- D=[50] for UMi

- Note: this threshold for ZOA is not applicable to other sensing scenarios

Agreement

To generate the background channel, the power threshold (-25 dB) for removing clusters in step 6 in section 7.5, TR 38.901 is reused.

Agreement

The ISAC background channel can be generated between a sensing Tx and a sensing Rx or RP (relevant for monostatic case) via the following steps:

- Step 1: generate a first set of clusters/rays according to TR 38.901(or other related TRs)

- Step 2: generate a second set of NLOS clusters/rays according to TR 38.901 (or other related TRs), where the power of the second set of clusters/rays should be scaled down such that

PnS2=PnS2nPnS2Pdrop=PnS2nPnS2P1S110G10

- P1S1 is the power of the NLOS cluster with the strongest power from the first set.

- PnS2 is the power of the n-th cluster from the second set.

- Where, N is the number of clusters, M is the number of rays within each cluster, value of G relates to power

  - N=360, M=1, G = -25dB, no further change from 38.901, 36.777, 38.858 (i.e., utilizing the same DS, ASA, ASD, ZSA, ZSD, Cθ, Cϕ as used for the first step)

- The step 2 is an additional modeling component

Agreement

For human as a sensing target with a single scattering point, the height of the scattering point is 1.5 m.

Agreement

In sensing scenario UMi, UMa, RMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation, down-selection one of the options below:

  - Option 4: use PL1 in Table 7.4.1-1: Pathloss models in TR 38.901

  - Option 5: use hUT 1.5 m for pathloss calculation

Agreement

For sensing scenario UMi, UMa, RMa, UMi-AV, UMa-AV and RMa-AV, the height of a scattering point of a target is used to calculate the LOS probability and pathloss, regardless of the lower bound in the existing TRs that are referred to generate ISAC channel.

  - FFS for the case where the height of a scattering point of target is less than 1.5m in sensing scenario UMi, UMa, RMa

Agreement ([Post-120bis-ISAC-02])

On the monostatic RCS of UAV of large size,

- The values/pattern of component A*B1 are generated by the following parameters

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 90° | 7.13° | 90° | 8.68° | 7.43 | 14.30 | [45°,135°] | [45°,135°] |
| Back | 180° | 10.09° | 90° | 11.43° | 3.99 | 10.86 | [45°,135°] | [135°,225°] |
| Right | 270° | 7.13° | 90° | 8.68° | 7.43 | 14.30 | [45°,135°] | [225°,315°] |
| Front | 0° | 14.19° | 90° | 16.53° | 1.02 | 7.89 | [45°,135°] | [-45°,45°] |
| Bottom | / | / | 180° | 4.93° | 13.55 | 20.42 | [135°,180°] | [0°,360°] |
| Roof | / | / | 0° | 4.93° | 13.55 | 20.42 | [0°,45°] | [0°,360°] |

  - When θ is in the range [0°,45° ] or [135°,180°], σHdB φ=0

- The standard deviation of component B2 is 2.50 dB

Agreement ([Post-120bis-ISAC-02])

On the monostatic RCS of AGV with single scattering point,

- The values/pattern of component A*B1 are generated by the following parameters

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.02 | 23.29 | [30°,180°] | [-45°,45°] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.33 | 17.60 | [30°,180°] | [45°,135°] |
| Back | 180° | 12.49° | 90° | 11.89° | 11.01 | 21.28 | [30°,180°] | [135°,225°] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.33 | 17.60 | [30°,180°] | [225°,315°] |
| Roof | / | / | 0° | 11.44° | 11.79 | 22.06 | [0°,30°] | [0°,360°] |

  - When θ is in the range [0°,30° ), σHdB φ=0

- The standard deviation of component B2 is 2.51 dB

Agreement ([Post-120bis-ISAC-02])

The values of the parameters to generate background channel for TRP monostatic sensing for each sensing scenario are provided in the following table

| Scenario |  | TRP monostatic sensing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Uma / Urban grid / Highway(FR2) / HST(FR2) | UMi | Rma / Highway(FR1) / HST(FR1) | Indoor office | Indoor Factory |
| Distribution of 2D distance between Tx and reference points | α1 | 10.3370 | 6.1996 | 6.2025 | 4.236 | 0.039836 |
|  | β1 | 0.1317 | 0.1558 | 0.0391 | 0.19255 | 0.179783 |
|  | c1 | 68.7778 | 15.2697 | 1.2940 | 4.99 | 1.130020 |
| Distribution of height of reference points | α2 | 16.2253 | 12.0487 | 0.0007 | 1.3293 | 0.283447 |
|  | β2 | 1.9218 | 2.3261 | 5.0146 | 0.1442 | 0.435965 |
|  | c2 | 2.6142 | 0.0157 | 0.0522 | -13.19 | -17.043530 |
| scaling factor d_s to d3D (if validated) |  |  |  |  |  |  |
| Threshold D for ZOA |  | 80 | 50 | 90 | N/A | N/A |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on UT given in TR 38.901 for the corresponding deployment scenario.

Note 2: The reference points for generating the TRP monostatic background channel have no mobility, i.e. 0 km/h.

Agreement ([Post-120bis-ISAC-02])

The values of the parameters to generate background channel for UT monostatic sensing for the following sensing scenarios are provided in the following table

| Scenario |  | UT monostatic sensing |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Uma / Urban grid / Highway(FR2) / HST(FR2) | UMi | Rma / Highway(FR1) / HST(FR1) | Indoor office | Indoor Factory |
| Distribution of 2D distance between Tx and reference points | α1 | 2.9072 | 10.0220 | 10.2421 | 4.3733 | 0.231418 |
|  | β1 | 0.1031 | 1.2522 | 0.0526 | 0.4457 | 0.128133 |
|  | c1 | 3.8471 | 11.0040 | 3.3131 | 4.6302 | 2.004903 |
| Distribution of height of reference points | α2 | 1.6640 | 3.0487 | 0.3175 | 0.2974 | 0.462968 |
|  | β2 | 1.6215 | 1.9128 | 1.4150 | 0.4103 | 0.281526 |
|  | c2 | -1.4205 | 0.1785 | 1.5906 | 2.9711 | -16.921515 |
| scaling factor d_s to d3D (if validated) |  |  |  |  |  |  |
| Threshold D for ZOA |  | 80 | 50 | 90 | N/A | N/A |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP given in TR 38.901 for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel has the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0

Agreement ([Post-120bis-ISAC-02])

The values of the parameters to generate background channel for UT monostatic sensing for the following sensing scenarios are provided in the following table

| Scenario |  | UT monostatic sensing |  |  |
| --- | --- | --- | --- | --- |
|  |  | UMa-AV | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | α1=0.83+0.00015h |  |  |
|  | β1 | β1=1536.305+1.0279h |  |  |
|  | c1 | c1=13.824+0.03085h |  |  |
| Distribution of height of reference points | α2 | α2=0.9054-0.0001117h |  |  |
|  | β2 | β2=138.672-0.04658h |  |  |
|  | c2 | c2=25.4898-0.02398h |  |  |
| scaling factor d_s to d3D (if validated) |  |  |  |  |
| Threshold D for ZOA |  |  |  |  |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP given in TR 38.901 for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel has the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0

RAN1 #121, St Julian’s, Malta, May. 19-23, 2025

ISAC deployment scenarios

Agreement

Updates to Table 7.9.1-1: Evaluation parameters for UAV sensing scenarios are as follows:

| Parameters | Value |
| --- | --- |
| Applicable communication scenarios | UMi, UMa, RMa, SMa UMi-AV, UMa-AV, RMa-AV [36.777] |
| Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc. | Can be considered in future evaluations |

NOTE1:	calibration for the UAV scenario is performed for UMa-AV scenario, but UMi-AV, RMa-AV, UMi, UMa, RMa, SMa can be considered for future evaluations of the UAV sensing target scenarios.

NOTE2:	A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Conclusion

Channel model for ISAC for SMa scenario will not be fully studied in Rel-19.

Agreement

Updates to Table 7.9.1-2: Evaluation parameters for Automotive sensing scenarios are as follows:

| Parameters | Values |
| --- | --- |
| Applicable communication scenarios | Highway, Urban Grid. UMi, UMa, RMa, SMa. NOTE1 |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. | EO Type 2 for Urban Grid - up to 4 walls modelled as EO type 2, per building of size 413m x 230m x 20m. Additional building sizes, building heights, materials, etc., can be considered in future evaluations |

NOTE1:	calibration for the automotive scenario will be performed for Highway and Urban Grid scenarios. UMi, UMa, RMa, SMa and related calibration parameters can be considered for future evaluations of the automotive sensing target scenarios.

NOTE2:	A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

Updates to Table 7.9.1-3: Evaluation parameters for Human (indoor and outdoor) sensing scenarios as follows:

| Parameters | Indoor Values | Outdoor Values |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 | Indoor office, indoor factory Indoor room [TR38.808] | UMi, UMa, RMa, SMa |
| Minimum 3D distance between sensing targets | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 1 m. | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 1 m. |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. | Can be considered in future evaluations | Can be considered in future evaluations |

Agreement

Updates to Table 7.9.1-4: Evaluation parameters for Automated Guided Vehicles sensing scenarios as follows:

| Parameters |  | Value |
| --- | --- | --- |
| Sensing Target | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - Material, Additional sizes, and AGV size distribution can be considered in future evaluations |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, 5 m. |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | Can be considered in future evaluations |

Agreement

Updates to Table 7.9.1-5: Evaluation parameters for objects creating hazards sensing scenarios as follows:

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Highway, Urban grid, HST (High Speed Train) UMi, UMa, RMa, SMa |
| Sensing Target | 3D mobility | Horizontal velocity: up to 10 km/h for humans and animals Additional velocities, trajectory can be considered in future evaluations |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - up to 4 walls modelled as EO type 2, per building of size 413m x 230m x 20m. Additional building sizes, building heights, materials, etc., can be considered in future evaluations |

NOTE1:	calibration for objects creating hazards scenario can be performed for Highway and Urban Grid scenarios. UMi, UMa, RMa, SMa and HST and related calibration parameters can be considered for future evaluations of the objects creating hazards scenarios.

Agreement

Updates to Table 7.9.7.1-3. Simulation assumptions for large scale calibration for Automotive sensing targets as follows:

| Parameters | Values |
| --- | --- |
| Scenario | For FR1: Urban Grid (ISD=500m, BS height=25m) Highway (ISD=1732m, BS height=35m) For FR2: Urban Grid (ISD=250m, BS height=25m) Highway (ISD=500m, BS height=25m) |
| Component A of the RCS for each scattering point | 11.25 dBsm |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. |

Agreement

Clarification for metrics for Simulation assumptions for full calibration sensing targets as follows:

| The power threshold for path dropping after concatenation for target channel | -40dB |
| --- | --- |
| The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel | -25dB |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) For monostatic sensing mode: delay spread and angle spread of the background channel is calculated separately for each reference point based on paths from all reference points. Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996. |

Agreement

Updates to Table 7.9.7.2-2: Simulation assumptions for full calibration for Human sensing targets as follows:

| Parameters | Indoor Values | Outdoor Values |
| --- | --- | --- |
| (u, std) for XPR of target | (19.81, 4.25) dB | (19.81, 4.25) dB |

Agreement

The following introductory text is added before each of the ISAC deployment scenarios;

- ISAC-UAV

In the ISAC-UAV scenario, the sensing targets are outdoor UAVs below or above the buildings in urban or rural areas. Monostatic or bistatic sensing can be performed using TRPs and/or UEs, including UEs on other UAVs.

- ISAC-Automotive

In the ISAC-Automotive scenario, the sensing targets are passenger vehicles or trucks and buses traveling on roads and streets in urban and rural areas. Monostatic or bistatic sensing can be performed using TRPs and/or UEs, including UEs on other vehicles and roadside UEs (RSU-type UEs).

- ISAC-Human

In the ISAC-Human scenario, the sensing targets are children and adult persons in indoor (room, office, factory) and outdoor (urban, rural) locations. Monostatic or bistatic sensing can be performed using TRPs and/or UEs in the corresponding communication scenarios.

- ISAC-AGV

In the ISAC-AGV scenario, the sensing targets are automated guided vehicles (AGVs) inside a factory. Monostatic or bistatic sensing can be performed using TRPs and/or UEs in the corresponding communication scenario.

- ISAC-Objects creating hazards

In the ISAC-Objects creating hazards scenario, the sensing targets are adult humans and children and animals in communication scenarios involving vehicles or high-speed trains. Monostatic or bistatic sensing can be performed using TRPs and/or UEs, including UEs on other vehicles and roadside UEs (RSU-type UEs).

Agreement

Updates to 7.9.7.1-4: Simulation assumptions for large scale calibration for AGV sensing targets as follows:

| Parameters | Values |
| --- | --- |
| Component A of the RCS for each scattering point | -4.25 dBsm Note: based on AGV option 1 |

Agreement

Updates to 7.9.7.2-4: Simulation assumptions for full calibration for AGV sensing targets as follows:

| Parameters | Values |
| --- | --- |
| (u, std) for XPR of target | (9.60, 6.85) dB |

Agreement

Resolve square brackets for Table 7.9.1-4: Evaluation parameters for Automated Guided Vehicles sensing scenarios:

| Sensing Target | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities {3, 10} km/h |
| --- | --- | --- |

ISAC channel modelling

Agreement

Confirm the following working assumption with updates in red.

Working assumption

For vehicle with single/multiple scattering points, the bistatic RCS is generated by

- The values/pattern of A*B1 of bistatic RCS is given by:

rcsdBθi,ϕi,θs,ϕs=maxGmax-min-σVdBθ+σHdB φ,σmax-Attenuatefactor,  Gmax-σmax,RCSdB,fsθi,ϕi,θs,ϕs

where

σVdBθ=-min12θ-θcenterθ3dB2, σmax

σHdB φ=-min12φ-φcenterφ3dB2, σmax

  - Attenuatefactor=k1×sin(k2×β2)-5log10cosβ2 is applied to the  β within 0~180 degrees. k1= 6 and k2=1.65. β is the bistatic angle between the incident ray and scattering ray within the plane of incident direction (θi,ϕi,) and scattering direction (θs,ϕs).

  - The angles of (θ,φ) are the projections of the bisector angle on the vertical plane and the horizontal plane, respectively.

    - FFS: RCS value when β is 180 degrees

  - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - 5 sets of parameters φcenter, φ3dB, n, θcenter, θ3dB,n, Gmax, σmax, Applicable Range of θ and Applicable Range of φ are applicable as defined for the monostatic RCS of vehicle with single/multiple SPSTs

  - Continue study on a new formula for Attenuatefactor to resolve the issue of angular discontinuity.

    - The new formula should retain following property: the linear bistatic RCS for a vehicle with single scattering point is the sum of the bistatic RCS of the multiple scattering points of the vehicle

    - the following formula can be a reference for the study

Attenuatefactor=k1sink2β2(1-k3β-k4*(Gmax-min-σVdBθ+σHdB φ,σmax))

Agreement

The agreement on bistatic RCS for vehicle is reused for large size UAV and AGV.

  - For large size UAV, k₁=6.05 and k₂=1.33

  - For AGV, k₁=12 and k₂=1.45

Agreement

AGV can be modelled with multiple scattering points.

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0° | 13.68° | 90° | 13.68° | 13.00 | 30.26 | [0,180] | [0,360] |
| Left | 90° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Back | 180° | 12.49° | 90° | 11.89° | 10.98 | 28.24 | [0,180] | [0,360] |
| Right | 270° | 15.53° | 75° | 20.03° | 7.27 | 24.53 | [0,180] | [0,360] |
| Roof | / | / | 0° | 11.44° | 11.77 | 29.03 | [0,180] | [0,360] |

  - Note: For the scattering point associated with roof of the AGV, σHdBϕ=0.

  - Note: the measurements from companies are done by AGV option 1.

Agreement

- The bistatic RCS of UAV with small size is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for UAV of small size

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for UAV of small size

- The bistatic RCS of Human with RCS model 1 is modelled as

  - The values/pattern of A*B1 is given by

rcsdBθi,ϕi,θs,ϕs=maxσRCS,A-Attenuatefactor,RCSdB,fsθi,ϕi,θs,ϕs

    - Component A, i.e., σRCS,A: same as component A of mono-static RCS for Human with RCS model 1

    - Attenuatefactor=3sin(β2) dB, where β is the bi-static angle between incident ray and scattered ray, β is within 0 and 180 degree

    - The effect of forward scattering RCSdB,fsθi,ϕi,θs,ϕs is -Inf in Rel-19

  - Component B2: same as component B2 of mono-static RCS for Human with RCS model 1

Agreement

On the monostatic RCS of human with RCS model 2,

- The values/pattern of component A*B1 are generated by the following parameters

|  | ϕcenter in [°] | ϕ3dB, n in [°] | θcenter in [°] | θ3dB,n in [°] | Gmax | σmax | Range of θ in [°] | Range of ϕ in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front | 0 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [-90, 90] |
| Back | 180 | 216.65 | 90 | 55.7 | 2.14 | 7.7 | [0,180] | [90,270] |

- The standard deviation of component B2 is 3.94 dB

Agreement

The agreement on bistatic RCS for vehicle with single scattering point is reused to model bistatic RCS of human with RCS model 2

  - k1=0.5714 and k2=0.1

Agreement

The following values of the RCS component A are applied to both monostatic and bistatic RCS of the target.

  - UAV with large size: -5.85 dBsm

  - Human with RCS model 2: -1.37 dBsm

    - Note: measurement is based on adult

  - Vehicle: 11.25 dBsm

    - Note: measurement is based on vehicle type 1 and 2

  - AGV: -4.25 dBsm

    - Note: measurement is based on AGV option 1

Note: component A on its own may not fully reflect the RCS in the target channel. This note will not be captured in the TR.

Agreement

The mean and standard deviation values of XPR of sensing target AGV for monostatic sensing and bistatic sensing are (9.60, 6.85) dB.

Conclusion

The component B2 of two different targets are generated independently.

Conclusion

The component XPR/initial random phase of two different targets are generated independently.

Agreement

In order to generate Tx-target link, target-Rx link and the background channel between a RSU-type UE and another node (TRP, pedestrian UE, vehicle UE, RSU-type UE), the following reference TRs are adopted

| Case | Tx/Rx | Rx/Tx | Existing TRs as starting point |
| --- | --- | --- | --- |
|  | TRP | RSU-type UE | Highway and Urban grid - B2R link in section 6 of TR 37.885 |
|  | RSU-type UE | normal UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | RSU-type UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |
|  | RSU-type UE | vehicle UE | Highway and Urban grid - V2V link in section 6 of TR 37.885, with antenna height at RSU is 5m |

Agreement

The initial random phase (generated in Step 10, section 7.5, TR38.901) is the same for the same ray in Tx-target link and target-Rx link of a target for monostatic sensing.

Agreement

For UMi-AV and RMa-AV with aerial UE as sensing transmitter or receiver, the values of parameters to generate background channel for UT monostatic sensing are provided in the following table

| Scenario |  | UT monostatic sensing |  |
| --- | --- | --- | --- |
|  |  | UMi-AV | RMa-AV |
| Distribution of 2D distance between Tx and reference points | α1 | 0.0156h+5.5399 | 4.423+0.001926h |
|  | β1 | 40.4517/(h+254.6318) | 13.8467+0.6547h |
|  | c1 | 0.0140h+15.1184 | 3.864+0.1538h |
| Distribution of height of reference points | α2 | 0.0123h+11.9569 | 1.4231+0.00192h |
|  | β2 | 17.8047/(h-0.2202) | 11.7157-0.00538h |
|  | c2 | 0.0532h-0.0120 | 2.6541-0.003851h |

Note 1: Distributions of height and distance of reference point are not subject to geographical constraints on TRP for the corresponding deployment scenario.

Note 2: The reference points for generating the UT monostatic background channel have the same velocity as UT.

Note 3: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset in the background channel should be set as 0

Agreement

To generate the background channel for TRP monostatic sensing and UT monostatic sensing, ‘maxd3D-c12+(c2-height_Tx)2c,0 +Δτ’ is used to model the absolute delay between the Tx and each reference point.

Agreement

Power threshold for path dropping after concatenation is up to -40dB for target channel for option 3. Up to company to choose a value in the implementation.

Power threshold for path dropping after concatenation is up to -25dB for target channel for option 0. Up to company to choose a value in the implementation.

For calibrations for both option 0 and option 3, power threshold for path dropping after concatenation is -40dB for target channel.

Agreement

To generate the absolute delay model for sensing scenarios Urban grid, highway and HST, for both target channel and background channel

- For Urban grid, the values of parameters for Δτ of scenarios UMa are reused.

- For Highway, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

- For HST, the values of parameters for Δτ of scenarios RMa and UMa are reused for FR1 and FR2 respectively.

Note: no measurements on Δτ of the 3 scenarios are submitted in Rel-19.

Agreement

Spatial consistency is not modelled for

- the links that are generated referring to channel models with parameter values of different communication scenarios

  - E.g., between TRP-target/UT link in one scenario and target/UT-UT link in another scenario

- the background channels for TRP monostatic sensing of different TRPs

Agreement

Spatial consistency is not modelled between TRP-target/UT link and target/UT-UT link for sensing scenario UMi, InH and InF.

Agreement

Spatial consistency is not modelled between TRP-TRP link and any other links for ISAC channel.

Agreement

Spatial consistency can be enabled for multiple scattering points of a target.

Spatial consistency, if enabled, for the links between BS/UT and multiple scattering points of a target are modelled as if multiple scattering points are multiple targets.

Agreement

The existing horizontal correlation distance in Table 7.6.3.1-2 in TR38.901 is used as the correlation distance for 3D spatial consistency for ISAC channel at least for UAV scenario, within same ‘Applicability range in terms of aerial UE height (defined in 36.777)’.

Agreement

EO type-2 can be modelled in NLOS condition.

Agreement

In sensing scenario UMi, UMa, if the height of a scattering point of target is less than 1.5m, for pathloss calculation,

  - use hUT 1.5 m for breakpoint distance (dBP) calculation

  - Note: hUT 1.5 m is only used for dBP calculation. The exact h_UT of the scattering point is still used to determine all other parameters of ISAC channel, e.g., delay, AOD/ZOD/AOA/ZOA, etc.

Agreement

On background channel modelling,

- Spatial consistency is not supported for TRP monostatic sensing across different TRPs

- Spatial consistency is not supported for UE monostatic sensing across different UEs

- Spatial consistency is not supported across different Reference Points for same TRP for TRP monostatic sensing

- Spatial consistency is not supported across different Reference Points for same UE for UE monostatic sensing

Agreement

- RCS component B2 of different direct/indirect paths of a target in the target channel are generated independently.

- On the RCS component B2 of a direct/indirect path of a target in the target channel, the same value of B2 applies to a path before the value of B2 is updated.

  - Note: whether/how/when to update B2 can be discussed in evaluation phase or up to companies’ choices

Agreement

- XPR of different direct/indirect paths of a target in the target channel are generated independently.

- On the XPR of a direct/indirect path of a target in the target channel, the same value of XPR applies to a path before the value of XPR is updated.

  - Note: whether/how/when to update XPR can be discussed in evaluation phase or up to companies’ choices

Agreement

- Initial random phase of different direct/indirect paths of a target in the target channel are generated independently.

- On the initial random phase of a direct/indirect path of a target in the target channel, the same value of initial random phase applies to a path before the value of initial random phase is updated.

  - Note: whether/how/when to update initial random phase can be discussed in evaluation phase or up to companies’ choices

Agreement

The follow TP is used generate the power (except for the impact of polarization matrix of EO type-2) of the ray specular reflected by an EO type 2 in the STX-SPST link or SPST-SRX link.

7.9.5.2 Type-2 environment object < Unchanged text omitted > - In Step 10 in Clause 7.9.4.1, Prx,0,m'k,p,Ptx,0,mk,p for a NLOS ray specularly reflected by a type-2 EO, if present, in the SPST-SRX link and the STX-SPST link is determined as follows. - If the STX-SPST link is in LOS condition, Ptx,0,mk,p=dtx,3Dk,pdtx,EO,mk,p2 - If the STX-SPST link is not in LOS condition, Ptx,0,mk,p=10-PLtx,k,pLOS+PLtx,k,p/10∙dtx,3Dk,pdtx,EO,mk,p2, where PLtx,k,pLOS is the pathloss of STX-SPST link assuming LOS condition. - If the SPST-SRX link is in LOS condition, Prx,0,m'k,p=drx,3Dk,pdrx,EO,m'k,p2 - If the SPST-SRX link is not in LOS condition, Prx,0,m'k,p=10-PLrx,k,pLOS+PLrx,k,p/10∙drx,3Dk,pdrx,EO,m'k,p2, where PLrx,k,pLOS is the pathloss of SPST-SRX link assuming LOS condition. < Unchanged text omitted >

Agreement

To generate the absolute delay model for sensing scenarios UMi-AV, UMa-AV and RMa-AV, for both target channel and background channel,

- For the TRP-TRP link and TRP- terrestrial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- terrestrial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the TRP- aerial UE link, the values of parameters for Δτ of scenarios UMi, UMa and RMa are respectively reused.

- For the terrestrial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

- For the aerial UE- aerial UE link, the values of parameters for Δτ of scenarios UMi are reused.

Note: no measurements on Δτ of the scenarios UMi-AV, UMa-AV and RMa-AV are submitted in Rel-19.

Agreement

- Remove the brackets for first sub-bullet under Step 4 for Clause 7.9.4.2 in the CR to TR 38.901.

- On the absolute delay of the background channel for both TRP and UE monostatic sensing, three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs.

Agreement

To generate the channel between an aerial UE and a normal UE,

- The LOS probability is generated by:

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |

- The pathloss and shadow fading are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting hBS =1.5m for FR1

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

Conclusion

No further study on power normalization of target channel and background channel of ISAC channel in Rel-19

  - Note: sub-section “7.9.5.3 Power normalization across target channel and background channel” in the TR remains as a placeholder with the following text.

    - To combine the target channel and the background channel, power normalization can be applied to keep the same/similar channel power as the background channel without sensing target.

Agreement

The polarization matrix CPMsp,i of a direct/indirect path i of a scattering point of a target is defined in LCS.

Agreement

To generate the channel between a first aerial UE with height h1 and a second aerial UE with height h2, abs(h1-hBS) <= abs(h2-hBS),

- The LOS probability between the two aerial UEs is generated by:

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to [high] UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 | 1 |

- The pathloss and shadow fading between two aerial UEs are generated using TRP-aerial UE link of UMi-AV in Annex A and B of TR 36.777 by setting height of TRP equal to the height of the first aerial UE.

Note:

- The height ranges of low-UAV, Mid-UAV and High-UAV are defined following the applicability range in terms of aerial UE height in Table B-1: LOS probability in TR 36.777

- The second height range for UMi-AV is further divided into 2 regions, i.e., [22.5, 100] and [100, 300] for mid-UAV and high-UAV, respectively.

Agreement

Update the agreements on LOS probability calculation for channel between an aerial UE and a normal UE as follows.

| Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- |
| UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |

Agreement

Update the agreements on LOS probability calculation for channel between two aerial UE as follows.

|  | Low-UAV | Mid-UAV | High-UAV |
| --- | --- | --- | --- |
| Low-UAV | UMi in Table 7.4.2-1 in TR 38.901 for UMi-AV/UMa-AV/RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV |
| Mid-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for RMa-AV | UMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region for UMi-AV/UMa-AV RMa-AV in Table B-1 in TR 36.777 for BS to mid UAV region | 1 |
| High-UAV | UMi-AV in Table B-1 in TR 36.777 for BS to high UAV region for UMi-AV/UMa-AV/[RMa-AV] RMa-AV in Table B-1 in TR 36.777 for BS to high UAV region for RMa-AV | 1 | 1 |

Agreement

To determine the LOS condition of any link in ISAC channel model, when EO type-2 is modelled, the following two options are agreed as solutions:

- Option A: If type-2 EO is in the LOS ray of the link, the LOS probability is p, p=0, and otherwise use the LOS probability equation defined in existing TRs to determine the LOS/NLOS condition

- Option C: Use the LOS probability equation to determine the LOS/NLOS condition of the link.

Note1: in which conditions/scenarios to use option A or option C can be determined in future evaluations.

Note2: as already agreed, monostatic background channel is always NLOS

Agreement

EO type-2 can be optionally modelled in background channel when EO type-2 is modelled in target channel.

Conclusion

Other than RCS for human, vehicle, AGV, UAV, no other RCS for other objects is introduced in Rel-19.

- Future studies are not precluded for adding RCS of other objects/sizes for modelling target or EO type-1, based on validation results from companies.

Conclusion

Delete subsection 7.9.6 from the draft CR. For ISAC, no enhancement to existing TR38.901 LLS channel model is introduced in Rel-19.

Agreement

- The existing blockage model A/B procedures can be reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of another target

- The location, orientation and size of the target as a blocker is known before applying the blockage model A/B.

Agreement

- The square brackets on formula 7.9.5-10 (copied below) in the draft CR are removed

The effective polarization matrix of the type-2 EO reflection path is given by

[CPMEO=cosγ2sinγ2-sinγ2cosγ2R∥00-R⊥cosγ1-sinγ1sinγ1cosγ1	(7.9.5-10)]

- Encourage companies to check and compare with the results that can be obtained with Alt2 and Alt3 below. If problem is found, RAN1 will revise TR 38.901 by new CR.

Alt2

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

With reusing the legacy transformation method for deriving γ1 and γ2.

Alt3

[CPMEO=cosγ2-sinγ2sinγ2cosγ2R∥00-R⊥cosγ1sinγ1-sinγ1cosγ1	(7.9.5-10)]

Where,

-	γ1=atan2-eθEO, ZODnplane,eϕEO, AODnplane. nplane represents the normal vector of the incident plane. nplane=vtx→w×vw→rx, in which vtx→w=xw-xtx,yw-ytx,zw-ztx and vw→rx=xrx-xw,yrx-yw,zrx-zw. eθEO, ZOD represents the spherical basis vector of incident ray in vertical direction. eθEO,ZOD=cosθEO, ZODcosϕEO, AOD,cosθEO, ZODsinϕEO, AOD,-sinθEO, ZODT. eϕEO,AOD represents the spherical basis vector of incident ray in horizontal direction. eϕEO,AOD=-sinφEO, AOD,cosφEO, AOD,0T.

-	γ2=atan2-eθEO, ZOAnplane,eϕEO, AOAnplane. eθEO, ZOA represents the polar basis vector of scattering ray in vertical direction. eϕEO, AOA represents the polar basis vector of scattering ray in horizontal direction. eθEO, ZOA=cosθEO, ZOAcosϕEO, AOA,cosθEO, ZOAsinϕEO, AOA,-sinθEO, ZOAT. eϕEO, AOA=-sinϕEO, AOA,cosϕEO, AOA,0T.

Conclusion

There is no consensus to introduce an exact formula for micro-Doppler in Rel-19. The placeholder in the channel impulse response is kept in the draft CR.

Agreements from Post-RAN1 #121 Email Discussion

Agreement:

Information on validations for ISAC is endorsed in R1-2504948.

Final CR to introduce channel model for ISAC for TS38.901 is agreed in R1-2505006.

2.1.2	Remaining Open issues

None

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #120bis

R1-2502552	Draft CR for TR 38.901 to introduce channel model for ISAC	Xiaomi, AT&T

R1-2503072	Draft CR for TR 38.901 to introduce channel model for ISAC	Xiaomi, AT&T

R1-2501817	Views on Rel-19 ISAC deployment scenarios	vivo

R1-2501839	Discussion on ISAC deployment scenarios and requirements	EURECOM

R1-2501926	Discussion on ISAC deployment scenarios	InterDigital, Inc.

R1-2501935	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

R1-2502002	Discussion on ISAC deployment scenarios	CATT, CICTCI

R1-2502029	Discussion on ISAC deployment scenarios	China Telecom

R1-2502051	Discussion on ISAC Deployment Scenarios	Nokia, Nokia Shanghai Bell

R1-2502054	Discussion on ISAC deployment scenarios	Tiami Networks

R1-2502062	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips

R1-2502067	Discussion on ISAC deployment scenarios	Panasonic

R1-2502170	Discussion on ISAC channel model calibration	CMCC, China Southern Power Grid

R1-2502207	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

R1-2502285	Discussion on ISAC channel model calibration	OPPO

R1-2502325	Discussion on ISAC deployment scenarios	Sony

R1-2502378	Discussion on ISAC deployment scenarios	Samsung

R1-2502416	Discussion on ISAC deployment scenarios	CALTTA

R1-2502418	Discussion on ISAC channel calibration	BUPT, CMCC

R1-2502451	Deployment scenarios and evaluation assumptions for ISAC channel model	Xiaomi

R1-2502465	Discussion on ISAC deployment scenarios	TOYOTA InfoTechnology Center

R1-2502588	Discussion on ISAC deployment scenarios 	Lenovo

R1-2502623	Discussion on ISAC deployment scenarios	Apple

R1-2502714	Discussion on ISAC deployment scenario	MediaTek Inc.

R1-2502725	Discussion on ISAC Deployment Scenarios	Ericsson

R1-2502820	Discussion on ISAC deployment scenarios	LG Electronics

R1-2502849	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

R1-2502922	Considerations on ISCA deployment scenarios	CAICT

R1-2502731	FL Summary #1 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2502732	FL Summary #2 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2502733	FL Summary #3 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2502734	FL Summary #4 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2501818	Views on Rel-19 ISAC channel modelling	vivo, BUPT

R1-2501840	Discussion on ISAC channel modeling	EURECOM

R1-2501878	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

R1-2501927	Discussion on ISAC channel modeling	InterDigital, Inc.

R1-2501933	Channel modelling for integrated sensing and communication with NR	NVIDIA

R1-2502003	Discussion on ISAC channel modelling	CATT, CICTCI

R1-2502030	Discussion on ISAC channel modelling	China Telecom

R1-2502052	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

R1-2502055	Discussion on ISAC Channel Modeling	Tiami Networks

R1-2502063	Joint views on mono-static background channel modeling	ZTE Corporation, Sanechips, OPPO, BUPT, BJTU, CAICT, Xiaomi

R1-2502171	Discussion on channel modeling methodology for ISAC	CMCC, BUPT, SEU, PML

R1-2502208	Channel modelling for ISAC	Huawei, HiSilicon

R1-2502286	Study on ISAC channel modelling	OPPO

R1-2502326	Discussion on Channel Modelling for ISAC	Sony

R1-2502379	Discussion on ISAC channel modelling	Samsung

R1-2502417	Discussion on channel modelling for ISAC	CALTTA, ZTE Corporation, Sanechips

R1-2502419	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO

R1-2502452	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

R1-2502466	Discussion on ISAC channel modelling	TOYOTA InfoTechnology Center

R1-2502565	Discussion on ISAC channel modelling	Tejas Network Limited

R1-2502572	Discussion on ISAC Channel Modeling	NIST

R1-2502587	Discussion on Channel Modelling for ISAC	Lenovo

R1-2502624	Discussion on ISAC channel modelling	Apple

R1-2502715	Discussion on ISAC channel modelling	MediaTek Inc.

R1-2502726	Discussion on ISAC Channel Modelling	Ericsson

R1-2502736	Discussions on ISAC Channel Modeling	AT&T

R1-2502776	Discussion on ISAC Channel Modelling	NTT DOCOMO, INC.

R1-2502814	Discussion on ISAC channel modelling	Panasonic

R1-2502821	Discussion on ISAC channel modelling	LG Electronics

R1-2502850	Discussion on ISAC channel modelling	Qualcomm Incorporated

R1-2502923	Considerations on ISAC channel modelling	CAICT

R1-2502553	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

R1-2502554	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

R1-2502555	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

R1-2502556	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)

R1-2503146	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)

RAN1 #121

R1-2504160	Draft CR for TR 38.901 to introduce channel model for ISAC	Xiaomi, AT&T

R1-2503247	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

R1-2503372	Views on Rel-19 ISAC deployment scenarios	vivo

R1-2503445	Discussion on ISAC deployment scenarios and requirements	EURECOM

R1-2503576	Discussion on ISAC deployment scenarios	Samsung

R1-2503697	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips, CAICT

R1-2503752	Discussion on ISAC deployment scenarios	InterDigital, Inc.

R1-2503760	Discussion on ISAC Deployment Scenarios	SK Telecom

R1-2503803	Discussion on ISAC deployment scenarios	CATT, CICTCI

R1-2503841	Discussion on full calibration of ISAC channel model	CMCC

R1-2503858	Discussion on ISAC channel calibration	BUPT, CMCC, X-Net

R1-2503892	Scenario and calibration discussion for ISAC CM	Xiaomi

R1-2503954	Discussion on ISAC Deployment Scenarios	Nokia, Nokia Shanghai Bell

R1-2503967	Discussion on ISAC deployment scenarios	Tiami Networks

R1-2503992	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

R1-2504012	Discussion on ISAC Deployment Scenarios	NIST

R1-2504053	Discussion on ISAC deployment scenarios	China Telecom

R1-2504068	Remaining issues on ISAC deployment scenarios	Sony

R1-2504126	Discussion on ISAC deployment scenarios	CALTTA

R1-2504146	Discussion on calibration results	ETRI

R1-2504220	Discussion on ISAC channel model calibration	OPPO

R1-2504239	Discussion on ISAC deployment scenarios	Lenovo

R1-2504268	Discussion on ISAC deployment scenario	MediaTek Inc.

R1-2504336	Discussion on ISAC deployment scenarios and Calibration	Apple

R1-2504367	ISAC scenarios and 7-24GHz alignment	AT&T, FirstNet

R1-2504404	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

R1-2504454	Discussion on ISAC Deployment Scenarios	Ericsson

R1-2504539	Discussion of calibration for UAV sensing targets	ITRI, Tron Future Tech Inc.

R1-2504566	Discussion on ISAC deployment scenarios	LG Electronics

R1-2504363	FL Summary #1 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2504364	FL Summary #2 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2504365	FL Summary #3 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2504366	FL Summary #4 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2503248	Channel modelling for ISAC	Huawei, HiSilicon

R1-2503373	Views on Rel-19 ISAC channel modelling	vivo, BUPT

R1-2503446	Discussion on ISAC channel modeling	EURECOM

R1-2503525	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

R1-2503577	Discussion on ISAC channel modelling	Samsung

R1-2503646	Discussion on ISAC channel modelling	Pengcheng Laboratory

R1-2503698	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips, CAICT

R1-2503720	Discussion on ISAC channel modelling	Tejas Network Limited

R1-2503726	Discussion on ISAC channel modelling	TOYOTA InfoTechnology Center

R1-2503753	Discussion on ISAC channel modeling	InterDigital, Inc.

R1-2503761	Discussion on ISAC Channel Modeling	SK Telecom

R1-2503804	Discussion on ISAC channel modelling	CATT, CICTCI

R1-2503842	Discussion on ISAC channel modeling	CMCC

R1-2503859	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO, X-Net

R1-2503893	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

R1-2503955	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

R1-2503969	Discussion on ISAC Channel Modeling	Tiami Networks

R1-2503991	Channel modelling for integrated sensing and communication with NR	NVIDIA

R1-2504013	Discussion on ISAC Channel Modeling	NIST

R1-2504054	Discussion on ISAC channel modelling	China Telecom

R1-2504069	Remaining issues on ISAC Channel Modeling	Sony

R1-2504110	Discussion on ISAC Channel Modelling	Panasonic

R1-2504119	Discussion on channel modelling for ISAC	CALTTA

R1-2504159	Discussion on ISAC channel modelling	Pengcheng Laboratory

R1-2504221	Study on ISAC channel modelling	OPPO

R1-2504240	Discussion on Channel Modelling for ISAC	Lenovo

R1-2504269	Discussion on ISAC channel modelling	MediaTek Inc.

R1-2504337	Discussion on ISAC channel modelling	Apple

R1-2504405	Discussion on ISAC channel modelling	Qualcomm Incorporated

R1-2504455	Discussion on ISAC Channel Modelling	Ericsson

R1-2504511	Discussion on ISAC Channel Modelling	NTT DOCOMO, INC.

R1-2504567	Discussion on ISAC channel modelling	LG Electronics

R1-2504161	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

R1-2504162	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

R1-2504163	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

R1-2504164	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)

R1-2504165	Summary #5 on ISAC channel modelling	Moderator (Xiaomi)

R1-2504945	Summary #6 on ISAC channel modelling	Moderator (Xiaomi)

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template