3GPP TSG RAN Meeting #108												RP-251818

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	CMCC

Title:	Text proposal for TR 38.914 for deployment scenarios

Document for:	Discussion/Decision

- Introduction

This contribution presents the Text proposal for TR 38.914 on deployment scenarios

======================Start of the changes========================

# 4.	Deployment scenarios

## 4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to 200 MHz (DL+UL) NOTE3 Around 7 GHz: Up to 400 MHz (DL+UL) NOTE3 Around 30 GHz: 1GHz (DL+UL) NOTE3 |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m (Equivalent to 12TRxPs per 120m x 50m) |
| BS antenna elements | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to [1024] Tx and Rx antenna elements Around 30 GHz: Up to [1024] Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to [8] Tx and Rx antenna elements Around 7 GHz: Up to [8 or 16] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP NOTE4 |
| Sensing targets | TBD |
| Sensing antenna elements | TBD |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For evalution of UL user experiecned data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account. For other evaluations, it is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the aggregated system bandwidth used for either DL or UL via switching in time-domain.

NOTE4:	10 users per TRxP is the baseline, other number of users per TRxP is not precluded.

## 4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz (single layer) Around 7 GHz (single layer) Around 4 GHz + Around 30GHz (two layers) |
| Aggregated system bandwidth NOTE2 | Around 4 GHz: Up to 200MHz (DL+UL) Around 7 GHz: Up to 400MHz (DL+UL) Around 30 GHz: 1GHz (DL+UL) |
| Layout | Single layer for 4 GHz or 7 GHz: - Hex. Grid Two layers for 4 GHz + 30 GHz: - Macro layer: Hex. Grid - Micro layer: Random drop Step 1 NOTE3: Around 4 GHz or 7 GHz in Macro layer (including macro cell only) Step 2 NOTE3: Both Around 4 GHz & Around 30 GHz may be available in Macro & Micro layers (including 1 macro layer, macro cell only) |
| ISD | Single layer: 200m Macro layer: 200m Micro layer: 3 micro TRxPs per macro TRxP All micro TRxPs are all outdoor |
| BS antenna elements | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to [1024] Tx and Rx antenna elements Around 30 GHz: Up to [1024] Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to [8] Tx and Rx antenna elements Around 7 GHz: Up to [8 or 16] Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP 10 users per TRxP with single-layer only 80% indoor (3km/h), 20% outdoor (30km/h) |
| Sensing targets | TBD |
| Sensing antenna elements | TBD |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For evalution of UL user experiecned data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account. For other evaluations, it is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the aggregated system bandwidth used for either DL or UL via switching in time-domain.

NOTE4:	10 users per TRxP is the baseline, other number of users per TRxP is not precluded.

## 4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz or Around 4 GHz (for ISD 1) Around 700 MHz and Around 2 GHz combined (for ISD 2) Around 7GHz |
| Aggregated system bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 120MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m |
| BS antenna elements | Around 700 MHz: up to 64 Tx and Rx antenna elements Around 2GHz: up to 128 Tx and Rx antenna elements] Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7GHz: Up to [1024] Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to [2 or 4] Tx and 4 Rx antenna elements Around 2GHz or 4 GHz: Up to 8Tx and Rx antenna elements Around 7 GHz: Up to [8 or 16] Tx and Rx antenna elements |
| User distribution and UE speed | 50% outdoor vehicles (120km/h) and 50% indoor (3km/h), 10 users per TRxP |
| Sensing targets | TBD |
| Sensing antenna elements | TBD |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4GHz is chosen as a proxy for this range. A range of bands from 6 425-7 125 MHz identified for WRC-23 are currently being considered and around 7GHz is chosen as a proxy for this range. A range of bands from 24.25 GHz – 52.6 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. For evalution of UL user experiecned data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account. For other evaluations, it is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE3:	"DL + UL" refers to either of the following two cases:

1.	FDD with symmetric bandwidth allocations between DL and UL.

2.	TDD with the aggregated system bandwidth used for either DL or UL via switching in time-domain.

NOTE4:	10 users per TRxP is the baseline, other number of users per TRxP is not precluded.

## 4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 120MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | 500m |
| BS antenna elements | Around 700 MHz: up to 64 Tx and Rx antenna elements Around 2GHz: up to 128 Tx and Rx antenna elements Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7GHz: Up to [1024] Tx and Rx antenna elements Around 30 GHz: Up to [1024] Tx and Rx antenna elements |
| UE antenna elements | Around 700 MHz: Up to [2 or 4] Tx and 4 Rx antenna elements Around 2GHz or 4 GHz: Up to [4 or 8] Tx and Rx antenna elements Around 7 GHz: Up to [8 or 16] Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | [20% Outdoor in cars: 30km/h or 10% Outdoor pedestrian: 3km/h + 10% Outdoor in cars: 30km/h] 10 users per TRxP |
| Sensing targets | TBD |
| Sensing antenna elements | TBD |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 2.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz |
| System bandwidth | Editor note: keep the same parameters for all scenarios |
| Layout | Single layer - Hex. Grid |
| ISD | 1000m |
| TRP antenna elements | Editor note: keep the same parameters as urban macro |
| UE antenna elements | Editor note: keep the same parameters as urban macro |
| User distribution and UE speed | 10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 50km/h, 80% Indoor in houses: 3km/h 10 users per TRxP NOTE4 |
| Sensing targets | TBD |
| Sensing antenna elements | TBD |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

## 4.6	High speed

## 4.7	Extreme long distance coverage in low density area

The extreme Long Range deployment scenario is defined to allow for the provision of services for very large areas with low density of users whether they are humans and machines (e.g. Low ARPU regions, wilderness, areas where only highways are located, etc). The key characteristics of this scenario are Macro cells with very large area coverage supporting basic data speeds and voice services, with low to moderate user throughput and low user density.

Table 2.10: Attributes for extreme rural

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Below 3 GHz With a priority on bands below 1GHz Around 700 MHz |
| System Bandwidth | 40 MHz (DL+UL) |
| Layout | Single layer: Isolated Macro cells |
| Cell range | 100 km range (Isolated cell) to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation (for example in some scenarios ranges up to 150-300km may be required). |
| User density and UE speed | User density: NOTE1 Speed up to 160 km/h |
| Traffic model | Average data throughput at busy hours/user: 30 kbps User experienced data rate: DL: up to 2 Mbps while stationary and 384 kbps while moving NOTE2 UL: up to 1 Mbps while stationary and 100 kbps while moving NOTE2 |

NOTE1:	Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.

NOTE2:	Target values for UL are lower than DL, 1/3 of DL is desirable.

## 4.8	Urban coverage for massive connection

Editor note: focusing on MTC

## 4.9	Air-to-Ground Scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g. Relay).

Some of the characteristics of this deployment scenario are listed in Table 4.9.

Table 4.9: Attributes for commercial Air to Ground Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro + relay: for BS to relay: 4 GHz |
| System Bandwidth | 100 MHz (DL+UL) |
| Layout | Macro + relay nodes NOTE1 |
| Cell range | Macro cell: 100 km range to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation. Relay: up to 80 m |
| User density and UE speed | End user density per Macro: NOTE2 UE speed: Up to 1200 km/h Altitude: Up to 15 km |
| Traffic model | End User experienced data rate: 384 kbps DL. NOTE3 |

NOTE1:	BS to relay link should be the priority for study compared to relay to UE link.

NOTE2:	Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.

NOTE3:	Target values for UL are lower than DL, 1/3 of DL is desirable.

## 4.10	Non-Terrestrial Network

This deployment scenario is defined to allow for the provision of services for those areas where the terrestrial service is not available and also for those services that can be more efficiently supported by the satellite systems such as broadcasting service. Satellite acts as a fill-in especially on roadways and rural areas where the terrestrial service isn’t available. The supported services via the Satellite system are not limited to just data and voice, but also for others such as machine type communications, broadcast and other delay tolerant services.

Some of its attributes are listed in Table 4.10.

Table 4.10: Examples for Satellite Deployment

## 4.11	[Urban grid]

## 4.12	[Highway Scenario]

## 4.xx	XXXX

Editor note: More deployment scenarios can be added based on companies input and discussion

======================End of the changes========================