3GPP TSG RAN Meeting #108 	RP-251850

Prague, Czech Republic, June 9-13, 2025	(revision of xx-yyxxxx)

# Source:	Nokia (moderator)

# Title:	New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3

# Document for:	Approval

# Agenda Item:	15.3.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3

Acronym:	FS_NR_AIML_NGRAN_Ph3

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X |  |  |
| No | X | X |  | X |  |
| Don't know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| FS_NR_ENDC_data_collection | 880076 | Study on enhancement for data collection for NR and ENDC | Preceding Study Item |
| NR_AIML_NGRAN-Core | 941110 | Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN | Preceding Work Item |
| FS_NR_AIML_NGRAN_enh | 1020083 | Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN | Preceding Study Item |
| NR_AIML_NGRAN_enh-Core | 1051124 | Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN | Preceding Work Item |

# 3	Justification

The integration of Artificial Intelligence (AI) and Machine Learning (ML) continues to be a key enabler for addressing the increasing complexity and dynamic behavior of 5G-Advanced networks. Building upon the outcomes of the Rel-18 normative work, which addressed AI/ML use cases for Load Balancing, Energy Saving, and Mobility Optimization, as well as the Rel-19 normative work focusing on AI/ML-based Network Slicing and Coverage and Capacity Optimization (CCO), there remains significant potential to further enhance NG-RAN functionalities through AI/ML.

In particular, UE mobility presents ongoing challenges in dense and heterogeneous network deployments, where handover decisions must be handled intelligently and efficiently. To this end, the Rel-20 study will focus on evaluating AI/ML-based mobility use cases, including multi-hop UE trajectory across gNBs, lower-layer triggered mobility (LTM), and other handover enhancements. These scenarios aim to improve handover reliability, reduce latency, and optimize mobility-related resource allocation through predictive and data-driven approaches.

This study will explore technical solutions, evaluate their benefits and feasibility, and assess potential impacts on 3GPP specifications. The outcomes are expected to guide potential normative work, supporting continued evolution of AI/ML integration in NG-RAN.

# 4	Objective

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts are expected.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 38.7XX | Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3 | TSG#110 | TSG#111 | Chen, Jiajun, ZTE Corporation, chen.jiajun1@zte.com.cn |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Primary rapporteur: Chen, Jiajun, ZTE Corporation, chen.jiajun1@zte.com.cn

Secondary rapporteur: Ma, Hui, NEC, hui.ma@EMEA.NEC.COM

# 7	Work item leadership

Responsible RAN WG: RAN3

# 8	Aspects that involve other WGs

Not applicable.

# 9	Supporting Individual Members

Supporting IM name

ZTE Corporation

NEC

CATT

China Telecom

China Unicom

CHTTL

Ericsson

FirstNet

HiSilicon

Huawei

III

InterDigital

ITRI

JIO Platforms

Lenovo

LG Electronics

Nokia

NVIDIA

Qualcomm

Ruijie Networks

Samsung

Sanechips

Sharp

SK Telecom

Telefónica

Transsion Holdings

Verizon

Xiaomi