3GPP TSG-RAN Meeting #108	RP-251854

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.6

Source:	Ericsson

Title:	Moderator's summary for Other RAN1 led 5G-Adv. REL-20 topics

Document for:	Report

# 1	Introduction

This document summarizes the discussion in RAN#108 and the contributions submitted to RAN#108 for “Other RAN1-led topics” under Agenda item 15.1.6 in RP-251759. Additionally, few additional contributions from Agenda items 15, 15.0 and 15.3.4 are also considered in this document to capture the companies’ views on the “Other RAN1 led topics”.

15.1.6 Other RAN1-led topics NOTE: Companies are encouraged to focus on NES, SBFD, and 200 MHz CBW or aggregated BW for n104 taking into account the company views captured in RP-250797 and RP-250802.

The following topics are organized in Sections 2 to 5:

- 200 MHz Channel bandwidth

- SBFD enhancements

- NES enhancements

- Other enhancements (MCE and channel modelling)

In each section, the corresponding discussion summary and conclusion from RAN#107, list of related contributions, proposed objectives and summary of companies positions and motivations are provided. Each section concludes with Moderator’s observation providing an overview of the status.

The summary of offline discussions and the online discussions outcomes are captured in Conclusion section 6.

For discussions and decisions, companies are encouraged to consider the general guideline below provided in RP-250859.

# 2	200 MHz Channel bandwidth

## 2.1	RAN#107 summary and conclusion

The following from RP-250797  shows the outcome of discussing this topic in the last meeting:

Summary/conclusions: The is no consensus during the offline discussions on how to proceed with the following 3 alternatives: - Define 200 MHz CBW for n104 in Rel-20 - Support for CA of 100 MHz + 100 Mhz for n104 seems acceptable for Rel-20 - Do nothing for n104 for Rel-20 There is common view that 200 MHz CBW is supported in 6G at least for upper 6 GHz

## 2.2	List of related contributions

|  | TDoc | Title | Source | Agenda item | Support 200 MHz CBW? |
| --- | --- | --- | --- | --- | --- |
| A1 | RP-251350 | Views on Rel-20 5G-Advanced general RAN1/RAN2/RAN3 scope | Apple | 15 | No |
| A2 | RP-251516 | General views on Rel-20 5G-Adv RAN1/2/3 package | Intel Corporation | 15.0 | No |
| A3 | RP-251026 | Views on Other RAN1-led topics | Samsung | 15.1.6 | No |
| A4 | RP-251280 | WF on 200 MHz Channel Bandwidth | KDDI, China Unicom, China Telecom, BT, Bouygues Telecom, Deutsche Telekom, KPN, Vodafone, Orange, Odido, Telefonica, Telenor, TIM, Telia Company, Spark NZ, Jio Platforms, Telstra, NTT DOCOMO, SoftBank, CKH IOD UK LTD, Ericsson, Huawei | 15.1.6 | Yes |
| A5 | RP-251424 | Supporting 200MHz channel bandwidth for Rel-20 5G-A | Lenovo | 15.1.6 | Yes |
| A6 | RP-251650 | Rel-20 Efficent Utilization of operating band larger than 100 MHz | Huawei, HiSilicon | 15.1.6 | Yes |
| A7 | RP-251711 | Qualcomm views on other RAN1-led topics | Qualcomm Incorporated | 15.1.6 | No |

## 2.3	Proposed objectives

| Topic | Proposed objectives |
| --- | --- |
| 1 | 200 MHz CBW [A4, A5, A6] Main proposed objective 1 - Support 200MHz channel bandwidth in Rel-20 5G-A. - Details by the following options. |
| 1-1 | Proposed objective Option 1-1 [A4] - Support 200 MHz Channel BW for n104 in Rel-20. - Sub-carrier spacing for 200MHz CBW - 30kHz recommended - Transmission bandwidth configuration and guard bands - Correct scaling of emissions mask - FRC bandwidth - 50MHz FRC could be defined for RX testing, since 20MHz FRC may lead to excessive test time - Impacts to RAN1/2 specification - RAN1 limit on number of PRBs - Note: In case 60kHz SCS would be selected, RAN4 demodulation requirements etc. needed |
| 1-2 | Proposed objective Option 1-2 [A6] - Specify a single carrier with channel bandwidth up to 200 MHz for FR1, at least for 30 kHz SCS - Specify the corresponding RF requirements for BS and UE (RAN4) - Note: At least gNB RF requirements would be simple extensions - Extend the configurations and parameters for channels and signals to support channel bandwidth up to 200 MHz (RAN1, RAN2, RAN3, RAN4) - Note: Only apply direct extensions of value ranges. - Table in [A6] shows all the simple/straightforward extensions |
| 1-3 | Proposed objective Option 1-3 [A5] - Support 200MHz channel bandwidth in Rel-20 5G-A. - Consider the following objectives: - Specify a solution for scheduling PUSCH/PDSCH on a 200MHz bandwidth [RAN1, RAN4] - Divide the wide carrier into multiple subbands and transmit SSB/PDCCH only in one subband for energy saving - Scheduling enhancement via a DCI format to schedule 200MHz bandwidth - Note: No impact on high layer protocol. - Note: No new DCI format is introduced. |

## 2.4	Companies’ positions and motivations

| Topic | Companies’ motivations and views on proposed objectives |
| --- | --- |
| 1 | 200MHz channel bandwidth in Rel-20 5G-A Proponents and Motivations: - The 200 MHz channel bandwidth in FR1, especially for band n104 (6425-7125 MHz), is critical to support emerging spectrum allocations in China, India, Brazil, UAE, and other markets. - At least 200 MHz spectrum per operator is expected in these markets, and also relevant in other markets. - Certainty needs to be given to operators and regulators that 200 MHz channel bandwidth will be supported by 3GPP. - Option 1-1) Proposes study technical areas across WGs to facilitate support - Option 1-2) Proposes simple extension of existing configuration parameters and RF requirements for single carrier operation with 200 MHz BW - Option 1-3) Proposes scheduling enhancements to support PUSCH/PDSCH over 200 MHz with single DCI and subband-based SSB/PDCCH transmission for energy saving - Proponents: [A4, A5, A6] KDDI, China Unicom, China Telecom, BT, Bouygues Telecom, Deutsche Telekom, KPN, Vodafone, Orange, Odido, Telefonica, Telenor, TIM, Telia Company, Spark NZ, Jio Platforms, Telstra, NTT DOCOMO, SoftBank, CKH IOD UK LTD, Ericsson, Huawei, HiSilicon, Lenovo Opponents and Motivations: - CA is acceptable and less complex at this stage of 5G; single carrier support may be deferred to 6G. - Supporting 200MHz with 30kHz SCS would require 8k FFT, which is deemed infeasible in single-mode 5G-Advanced - Supporting 200MHz with 60kHz would not require new FFT; however, the support of a new SCS is equally burdensome for implementation and testing. - Supporting 200 MHz channel BW requires considerable work loads in multiple WGs. - Opponents: [A1, A2, A3, A7] Apple, Intel, Samsung, Qualcomm |

## 2.5	Moderator’s observations

- Based on the submitted contributions, the situation as compared to RAN#107 remains the same, i.e. there is strong interest, in particular from operators to support 200 MHz BW for single carrier in FR1. However, there is opposition from UE vendors due to implementation complexity and feasibility at this stage of 5G-advanced.

  - Regarding proposed objective 1 (200MHz channel bandwidth in Rel-20 5G-A):

  - 24 supporting companies (20 operators)

  - 3 not supporting companies and one company with strong concern.

    - QC has strong concern.

- Among the opponents, some companies can accept to support 200 MHz BW by a 100+100MHz intra-band contiguous CA in RAN4 introduced in Rel-19.

- 100+100 MHz intra-band contiguous CA for n104 was introduced in Rel-19 in 38.101-1 V19.1.0 (2025-03) as follows.

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n104C | CA_n104C | 20, 30, 40, 50 | 60, 70, 80, 90, 100 |  |  |  | 200 | 0 |
|  |  | 60, 70, 80, 90 | 60, 70, 80, 90, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n104 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  |  | 200 | 4 and 5 |

## 2.6	Summary of offline discussion (Tuesday June 10th)

- Moderator minutes from the offline discussion:

  - Due to limited time, the discussion was very rushed and limited.

  - Concern was raised that not all the proponents (specially operators) could be present in the discussion due to collision.

  - QC: QC has strong concern (in moderator summary was listed as “non-supportive”) while listed as non-supportive.

    - Moderator changed the status in section 2.5 accordingly.

  - In this offline discussion, there was no consensus to support the enhancement.

  - Summary of Q&A:

    - QC – strong concern

    - Xiaomi, Nokia, MTK are not supportive either.

    - BT: FR2 400MHz is used for FR2. What is the issue for 200 MHz for FR1?

      - Apple: Different architectures are used for FR1 and FR2

    - HW: This is optional UE capability with strong interests from operators. No UE is forced to implement it.

    - HW: We understand the issue of timeline. Due to the strong interest from operators, and finding a solution, can be considered for Rel-21 5G-adv?

    - QC: It should be raised by Rel-21 discussion. It is long in future whether or not to express commitment.

    - Not OK with HW proposal: QC, NEC, Apple, Samsung, OPPO

# 3	SBFD enhancement

## 3.1	RAN#107 summary and conclusion

The following from RP-250797  shows the outcome of discussing this topic in the last meeting:

Summary/conclusions: There are limited to no TUs available throughout all RAN WGs for the “other” topics as part of Rel-20. Interest has been identified in SBFD with the clear understanding that only a very limited work can be done in Rel-20. For further discussion and consideration for the final approval of the Rel-20 5G-Advanced package in RAN#108 (June) is possible and interested companies might work together on the following two areas related to SBFD - There is interest but also concerns to specify inter gNB co-channel UE-UE CLI signalling for measurement resource configuration in RAN3 - There is interest but also concerns on subband-level power backoff It is common understanding that the work on the following topic would require substantially more work than the initial two above and hence no sufficient time as part of the Rel-20 might be found. So, it is not encouraged to further promote this as part of 5G-Advanced: - There is no consensus but still some interest to work on dynamic SBFD in Rel-20

## 3.2	List of related contributions

|  | TDoc | Title | Source | Agenda item | Support SBFD enh? |
| --- | --- | --- | --- | --- | --- |
| B1 | RP-251350 | Views on Rel-20 5G-Advanced general RAN1/RAN2/RAN3 scope | Apple | 15 | No |
| B2 | RP-251516 | General views on Rel-20 5G-Adv RAN1/2/3 package | Intel Corporation | 15.0 | No |
| B3 | RP-250869 | NR SBFD Phase 2 in Release-20 | KT Corp., China Unicom, Turkcell, Vodafone, CEWiT, ETRI, Fraunhofer IIS, Fraunhofer HHI, Google, Huawei, HiSilicon, IIT Kanpur, NEC, Sharp, Tejas Networks, Wilus, Yonsei University | 15.1.6 | Yes |
| B4 | RP-251026 | Views on Other RAN1-led topics | Samsung | 15.1.6 | No |
| B5 | RP-251267 | Views on Rel-20 Suband Full Duplex | Ericsson | 15.1.6 | No |
| B6 | RP-251649 | Rel-20 SBFD | Huawei, Hisilicon, Deutsche Telekom | 15.1.6 | Yes |
| B7 | RP-251711 | Qualcomm views on other RAN1-led topics | Qualcomm Incorporated | 15.1.6 | Yes |
| B8 | RP-251750 | Enhancements to support inter-UE CLI signaling for measurement resource configuration | Charter Communications, Qualcomm, Fraunhofer IIS, Fraunhofer HHI | 15.3.4 | Yes |

## 3.3	Proposed objectives

| Topic | Proposed objectives |
| --- | --- |
| 1 | Subband-level Power Backoff [B3, B6] - Proposed objective 1) Specify subband-level downlink power back off including corresponding enhancements in CSI measurement and report [RAN1, RAN4] - Support frequency-dependent PDSCH EPRE to CSI-RS EPRE ratio for CSI derivation [RAN1, RAN4] - Extend gNB RE power control dynamic range as needed [RAN4] - Nokia: To be discussed in RAN4 - Apple: With changes, it is not only RAN4 - Supportive of proposal with red changes: Charter, NEC, HW; Cablelabs - Not supportive of proposal with red changes:: Nokia, Ericsson, - Moderator: suggests the proponent to discuss with RAN4 |
| 2 | Dynamic SBFD [B3] - Proposed objective 2) Specify signaling support of dynamic transition between SBFD symbols and non-SBFD symbols for small cell deployment [RAN1, RAN2] - Note: the detailed schemes are to be down-selected from those in TR38.858 |
| 3 | Inter-gNB UE-UE CLI Signaling [B7, B8] - Proposed objective 3) Specify signaling enhancements to support inter-gNB coordination of CLI measurement resources (SRS configuration and CLI resources) for co-channel inter-UE CLI measurements [RAN3] |
| 4 | SBFD-SBFD Adjacent Channel Coexistence [B6] - Proposed objective 4) Continue the standard work to define RF requirements for SBFD-SBFD adjacent operator scenario in Rel-20 5G-Advanced WI [RAN4] - Define FR1 Macro/Micro RF requirements for SBFD-SBFD adjacent operator scenario, considering the expected spectrum allocation and sub-band sizes, and relevant co-existence and co-location deployment scenarios - TX unwanted emission - RX in-band blocking - The proposed objective (4) is RAN4 topic and not considered further here. |

## 3.4	Companies’ positions and motivations

| Topic | Companies’ motivations and views on proposed objectives |
| --- | --- |
| 1 | Subband-level Power Backoff Proponents and Motivations: - Subband power backoff is necessary to effectively reduce CLI from aggressor TDD cells to SBFD uplink subbands in victim cells. - Up to 15 dB subband-level PDSCH power backoff in aggressor gNBs is necessary, supported by CSI enhancements and extended gNB RE power control dynamic range. - Proponents: [B3, B6] KT Corp., China Unicom, Turkcell, Vodafone, CEWiT, ETRI, Fraunhofer IIS, Fraunhofer HHI, Google, Huawei, HiSilicon, IIT Kanpur, NEC, Sharp, Tejas Networks, Wilus, Yonsei University, Deutsche Telekom Opponents and Motivations: - No strong need motivated. No real field experience. Strive to reduce/limit the scope of 5G-Advanced work. Focus on 6G. - Considering the study in Rel 18 duplex, no clear benefit or motivation to use power domain CLI handling in SBFD (limited performance study) - Large subband power backoff requires impact on BS implementation. - Opponents: [B1, B2, B4, B5, B7] Apple, Intel, Samsung, Ericsson, Qualcomm |
| 2 | Dynamic SBFD Proponents and Motivations: - Facilitate practical SBFD deployments and accommodate varying DL/UL traffic demands via support of dynamic SBFD and intra-subband CLI enhancement. - Proponents: [B3] KT Corp., China Unicom, Turkcell, Vodafone, CEWiT, ETRI, Fraunhofer IIS, Fraunhofer HHI, Google, Huawei, HiSilicon, IIT Kanpur, NEC, Sharp, Tejas Networks, Wilus, Yonsei University Opponents and Motivations: - No strong need motivated. No real field experience. Strive to reduce/limit the scope of 5G-Advanced work. Focus on 6G. - No clear benefit or motivation to use power-domain CLI handling in SBFD (In Section 8.3.5A of TR38.858, only 2 companies submitted result for dynamic TDD, not for SBFD showing marginal gain). - Opponents: [B1, B2, B4, B5, B7] Apple, Intel, Samsung, Ericsson, Qualcomm |
| 3 | Inter-gNB UE-UE CLI Signaling Proponents and Motivations: - More accurate CLI-RSSI or CLI-RSRP due interference by neighbouring gNBs. - Proponents: [B7, B8] Charter Communications, Qualcomm, Fraunhofer IIS, Fraunhofer HHI Opponents and Motivations: - No strong need motivated. No real field experience. Strive to reduce/limit the scope of 5G-Advanced work. Focus on 6G. - In Section 7.4.3 of TR38.858, only 2 companies submitted result, but performance gain is very marginal gain over no inter-UE CLI handling. Considering the number of sources and also results, no clear benefit or motivation to support. - Opponents: [B1, B2, B4, B5] Apple, Intel, Samsung, Ericsson |

## 3.5	Moderator’s observations

- Based on the submitted contributions, the situation as compared to RAN#107 remains the same, i.e. there is strong interest in particular for the proposed objectives 1 and 2, but also disagreement to support any proposed SBFD enhancement for 5G-Adv.

- Regarding proposed objective 1 (Sub-band level Power backoff):

  - 17 supporting companies

  - 5 non-supporting companies

    - Offline: 10

    - HW: Operators are interested in deploying and there is strong interference

      - HW: provided updated simulation results. RAN4 part can’t be done by implementation

    - Nokia: disagree

    - NEC: want to study

    - QC: if down-scope not to have anu UE impact

    - Apple: Prefer 6G

    - Samsung: similar as QC. It also has impact on gNB implementation. Whether there is a RAN4 only solution (requirement)

    - Xiaomi: feasibility and benefit has not been sufficienctly studies. Not convince.

- Regarding proposed objective 2 (Dynamic SBFD):

  - 16 supporting companies

  - 5 non-supporting companies

    - Offline: 11 companies

- Regarding proposed objective 3 (Inter-gNB UE-UE CLI signaling):

  - 4 supporting companies

    - Offline: 4

    - Cable labs:  support

    - Charter: explained the details of solution

    - QC, MTK, ZTE: Left over, very limited work in RAN3. small WI

  - 4 non-supporting companies

    - Offline : 6

    - Apple :  not real field issue

    - Ericsson: problem is UE-UE adjacent channel . Does this solve the issue?

      - Charter: answer: we try to address single operator(inter-cell).

    - HW: not convinced by motivation. Not consistency with overall argument. Single vendor can fix it by impleentaiton

  - NEC: consider it as TEI

## 3.6	Summary of offline discussion (Tuesday June 10th)

- Moderator minutes from the offline discussion:

  - Due to limited time, the discussion was very rushed and limited.

  - Concern was raised that not all the proponents could be present in the discussion.

  - Moderator suggested to focus only on proposed objective 1 and 3.

    - ZTE requested to discuss proposed objective 3. Moderator clarified that the suggestion was due to limited time.

    - It was requested to check the status for different proposal:

  - Based on show-of-hands of companies present in offline session:

    - Proposed objective 1: 10 companies not supportive

      - Moderator didn’t check the supportive companies.

    - Proposed objective 2: 11 companies not supportive

      - Moderator didn’t check the supportive companies.

    - Proposed objective 3: 6 companies not supportive

      - 5 companies supportive

  - In this offline discussion, there was no consensus to support the enhancement.

  - Summary of Q&A:

    - Regarding proposed objective 1 (Sub-band level Power backoff):

      - 17 supporting companies

      - 5 non-supporting companies (offline: 10 companies)

      - HW: Operators are interested in deploying SBFD and there is strong interference that impacts the performance and should be addressed.

        - HW: Regarding concern on performance benefits, HW provided updated simulation results to show gain.

        - HW: The solution can not be achieved implementation

      - Nokia: Disagreed that the issue can not be addressed by implementation and questioned the need for the specification.

      - NEC: With performing study we can have proper assessment.

      - QC: The proposed solution has UE impact. Hence, not supportive. They questioned if the proposal can be down-scoped not to have UE impact

      - Samsung: Similar comment as QC. The proposed solutions (reducing power, etc) also has impact on gNB implementation. Whether there is a RAN4 only solution (requirement)

      - Xiaomi: Not convinced. The feasibility and benefits of the proposed solutions have not been sufficiently studied.

      - Apple: Prefer to defer to 6G.

    - Regarding proposed objective 2 (Dynamic SBFD):

      - 16 supporting companies

      - 5 non-supporting companies (Offline: 11 companies)

      - No technical Q&A.

    - Regarding proposed objective 3 (Inter-gNB UE-UE CLI signaling):

      - Charter: Explained the details of problem and solution

      - Cable labs:  Supports the proposal

      - QC, MTK, ZTE: This is a left over, with very limited work in RAN3. Supportive of a small RAN3 WI.

      - Apple: This is not real field issue

      - Ericsson: Problem is UE-UE adjacent channel. Does this solve the issue?

        - Charter: answer: We try to address single operator(inter-cell).

      - HW: Not convinced by motivation. Not consistency with overall argument as compared to arguments made for example to proposed objective 1. Usually, the situation of single operator, is single vendor. In that case, the issue can be handled by implementation.

      - NEC: The work is limited. Can be considered as TEI.

        - QC: Small work but rather small WI than TEI.

## 3.7	Summary of offline discussion (Wednesday June 11th)

In this offline sessions, proposed objectives 1 and 3 were discussed.

### 3.7.1	Offline discussion on Subband-level Power Backoff

The proponent suggested to update the proposal as the following:

  - Proposed objective 1- updated) Specify subband-level downlink power back off including corresponding enhancements in CSI measurement and report [RAN1, RAN4]

  - Support frequency-dependent PDSCH EPRE to CSI-RS EPRE ratio for CSI derivation [RAN1, RAN4]

  - Extend gNB RE power control dynamic range as needed [RAN4]

The offline discussion focused on updated proposal:

- Nokia: To be discussed in RAN4 with other RAN4 topics

- Apple: Concerns if the updated proposal has no RAN1 impact and it is really only RAN4

  - HW: The update proposal is only RAN4.

- Supportive of updated proposal: Charter, NEC, HW, Cable labs

- Not supportive of updated proposal: Nokia, Ericsson

- Moderator: Suggests the proponent to discuss the updated proposal with RAN4.

### 3.7.2	Offline discussion on Inter-gNB UE-UE CLI Signaling

The following proposal was discussed during the offline session:

- Inter-gNB UE-UE CLI Signaling [B7, B8]

Proposed objective 3) Specify signaling enhancements to support inter-gNB coordination of CLI measurement resources (SRS configuration and CLI resources) for co-channel inter-UE CLI measurements [RAN3]

The following is a summary of the discussion during the offline session:

- Apple: OK with limited scope.

- Samsung: OK with limited scope. What is estimated TU? It is a RAN3 topic and should be discussed  w RAN3 chair.

  - Charter: Our assessment is 0.5

- DCM: OK.

- KT: Supportive.

- Nokia: The L1 SRS configuration whether has any RAN1 impact.

- Charter (answer): Does not have RAN1 impact

- Ericsson: Concern that there would be RAN1 impact. If proposal goes through, there should be not have any RAN1 impact. Then, how many meetings?

  - Chater: No RAN1 impact. Expected to be done within 1 quarter.

- Ericsson: If the WID goes through, it should be reflected that the exchange of measurement resource configuration is on Xn/F1 interface

- Samsung: Same comment as Ericsson.

- CATT: It is a RAN3 objective and should be discussed w RAN3 and RAN3 chair

- Chater and Ericsson and other companies work on updating the proposal to address the concern.

- Updated proposal:

Specify exchange over Xn/F1 of configuration info for one or more SRS resources used for measurement of cochannel UE-UE cross link interference

- Moderator asked if there is objection to support the proposal assuming the concerns above is addressed. There was no objection in the offline session.

# 4 	NES enhancements

## 4.1	RAN#107 summary and conclusion

The following from RP-250802 shows the outcome of discussing this topic in the last meeting:

During the Thursday offline session, the moderator checked company views on the need for any NES enhancement on Release 20 5G-Adv - Positive (NES enhancement in Release 20, 16 companies): ZTE, Telecom Italia, Huawei, CEWiT, ITRI, Futurewei, Google, China Telecom, Honor, KT, Nokia, NEC, Fraunhofer, Sharp, TCL, Tejas - Negative (No NES enhancement in Release 20, 15 companies): LGE, IDC, Spreadtrum, Qualcomm, Panasonic, Xiaomi, Lenovo, Ericsson, Samsung, Fujitsu, ETRI, Apple, OPPO, AT&T, Intel In RAN#107, there is no consensus on the support of NES enhancement in Release 20 5G-Adv.

## 4.2	List of related contributions

|  | TDoc | Title | Source | Agenda item | Support NES enh? |
| --- | --- | --- | --- | --- | --- |
| C1 | RP-251350 | Views on Rel-20 5G-Advanced general RAN1/RAN2/RAN3 scope | Apple | 15 | No |
| C2 | RP-250989 | Views on Rel-20 NES Enhancement | Google | 15.1.6 | Yes |
| C3 | RP-251026 | Views on Other RAN1-led topics | Samsung | 15.1.6 | No |
| C4 | RP-251249 | Views on Rel-20 Network energy saving for 5G-Advanced | Fujitsu Limited | 15.1.6 | No |
| C5 | RP-251521 | Network Energy Saving Enhancements for 5G-Advanced in Rel. 20 | CEWiT | 15.1.6 | Yes |
| C6 | RP-251648 | Network energy saving enhancements for 5G-Advanced in Rel-20 | Huawei, HiSilicon, TIM, Orange, Deutsche Telekom, Telefonica, China Unicom, Fraunhofer IIS, Fraunhofer HHI, CAICT, TCL, CEWiT, Sharp, Rohde & Schwarz | 15.1.6 | Yes |
| C7 | RP-251687 | Views on Rel-20 Network Energy Saving | Ericsson | 15.1.6 | No |
| C8 | RP-251711 | Qualcomm views on other RAN1-led topics | Qualcomm Incorporated | 15.1.6 | No |
| C9 | RP-251724 | Views on further network energy saving enhancement in 5G-A Rel-20 | ZTE Corporation, Sanechips | 15.1.6 | Yes |

## 4.3	Proposed objectives

| Topic | Proposed objectives |
| --- | --- |
| 1 | UE recommended Cell DTX/DRX via UAI for RRC connected UEs [C2, C5, C6] - Proposed objective 1) Specify UE recommended cell DTX/DRX via UAI (UE Assistance Information) [RAN2] - Note: The candidate cell DTX/DRX configurations to be reported by UE can refer to the cell DTX/DRX configurations developed in Rel-18 |
| 2 | SSB periodicity extension [C5, C6] - Proposed objective 1) Specify an extended SSB periodicity by UE during initial access [RAN1, RAN4, RAN2] - [C6] Note 1: Consider default 160ms SSB periodicity for certain specific band(s), e.g. n104 - [C6] Note 2: The number of frequency raster points the UE is expected to search during initial frequency scan will be proportionally reduced, i.e. by 1/8 if the SSB periodicity is increased from 20ms to 160ms |
| 3 | On-demand SSB for RRC idle/inactive UEs [C2, C5, C9] - Proposed objective 3) Further enhancement on SSB transmission reduction, such as on-demand SSB for idle/inactive mode UEs, can be specified in 5G-Advanced Rel-20. |
| 4 | Cell DTX/DRX enhancements for RRC IDLE/INACTIVE UEs in Scells [C5] - Proposed objective 4) Specify procedures and signaling methods for extending cell DTX/DRX operation to common signals in Scells - Transmission of SSBs with certain periodicities within active period of cell DTX/DRX pattern and no transmission in non active period |
| 5 | On-demand TRS [C2] - Proposed objective 5) Specify on-demand TRS for Rel-20 NES. |

## 4.4	Companies’ positions and motivations

| Topic | Companies’ motivations and views on proposed objectives |
| --- | --- |
| General | Proposal: Support NES enhancements for 5G advanced - Proponents’ general motivation: - Strong need to continue the NES enhancement for 5G-Advanced in Rel-20 to increase the chance of successful deployment of NES techniques in the field within the ongoing 5G timeframe. - NES is critical for sustainable network operation and aligns with 6G energy efficiency goals. - NES aims to reduce energy consumption of 5G networks by optimizing transmission of common signals, control channels, and downlink reference signals, and by enabling cell DTX/DRX operations. - Rel-18 and Rel-19 introduced NES features with limited scenarios to protect legacy UEs. For Rel-20, proposals focus on extending SSB periodicity and on-demand SSB transmission to idle/inactive UEs, UE-initiated on-demand SSB, cell DTX/DRX enhancements, and on-demand TRS. - NES features should preferably be applied in new or re-farmed spectrum without legacy UEs to avoid backward compatibility issues. - Opponents’ general motivation: - NES is a key technology in 6G as well as 5G NR. It is important to focus Rel-20 work on 6G. - Key aspect is that both network and UE energy savings features need to be introduced in Day 1 to maximize UE footprint - In contrast, design of further NES features late in 5G Advanced would be hampered by constraints due to legacy UEs. Need to avoid “constrained” 5GA designs being inherited into 6G, thus risking 6G energy efficiency requirements. - Risk for duplicated discussion happens between 5G-A and 6G tracks if Rel-20 5G-A NES WI is agreed. - Most specified solutions require a large UE footprint to realize network energy savings. - In 5G-Advanced, the backward compatible options were mostly exhausted. - The commercial outlook for adoption of non-backward compatible options is not promising - Market take-up of standardized features is low, while savings can be achieved through implementation - Disagreement with the viewpoint that for newly introduced bands, backward compatibility is unimportant. - The guiding principle of introducing new bands has been to make them release independent. That means that supporting new bands should be possible with adding RF components, without having to upgrade baseband components, or limiting deployments to devices of newer releases |
| 1 | UE recommended Cell DTX/DRX via UAI for RRC connected UEs Proponents and Motivations: - In Rel-18, Cell DTX/DRX is specified, which is beneficial for both network and UE from energy saving perspective. However, network would be conservative to enable cell DTX/DRX in practical deployment, if the traffic information of a UE is not available at the network side. - UE is aware of its traffic information and can determine the optimal cell DTX/DRX pattern, providing better energy savings and user experience. - Proponents: [C2, C5, C6] Google, Huawei, HiSilicon, TIM, Orange, Deutsche Telekom, Telefonica, China Unicom, Fraunhofer IIS, Fraunhofer HHI, CAICT, TCL, CEWiT, Sharp, Rohde & Schwarz Opponents and Motivations: - Risk for duplicated discussion happens between 5G-A and 6G tracks if Rel-20 5G-A NES WI is agreed. - Most specified solutions require a large UE footprint to realize network energy savings. - Market take-up of standardized features is low, while savings can be achieved through implementation. - Opponents: [C1, C3, C4, C7, C8] Apple, Samsung, Fujitsu, Ericsson, Qualcomm |
| 2 | SSB periodicity extension Proponents and Motivations: - See Proponents’ general motivation above - SSB periodicity extension can achieve very promising energy saving gain according to the Rel-18 study as shown in TR 38.864. - SSB periodicity extension is applicable to cells in greenfield (new frequency band deployment) or where legacy UE constraints are minimal. - Legacy UEs can latch to frequency band supporting 20ms SSB periodicity and still work with extended SSB periodicity beyond 20ms to the additional bands - Impact on initial cell selection and RRM (Radio Resource Management) can be minimized by proper design of sync raster or with the help of frequency bands supporting 20ms SSB periodicity. - Proponents: [C5, C6] Huawei, HiSilicon, TIM, Orange, Deutsche Telekom, Telefonica, China Unicom, Fraunhofer IIS, Fraunhofer HHI, CAICT, TCL, CEWiT, Sharp, Rohde & Schwarz Opponents and Motivations: - See Opponents’ general motivation above - SSB periodicity extension is NBC and NES gain are marginal beyond the current maximum SSB periodicity value of 160 msec. - Opponents: [C1, C3, C4, C7, C8] Apple, Samsung, Fujitsu, Ericsson, Qualcomm |
| 3 | On-demand SSB for RRC idle/inactive UEs [C2, C5, C9] Proponents and Motivations: - See Proponents’ general motivation above - The idle/inactive mode UEs can acquire the synchronization and WUS configuration of the NES cell through the coverage cell before the SSB is triggered. - The on-demand SSB transmission can be further enhanced considering the new or re-farming spectrum without commercialized UEs, and leaner common signals/channels design can be expected to provide better spectrum sharing between 5G-A and 6G. - Proponents: [C2, C5, C9] Google, CEWiT, ZTE, Sanechips Opponents and Motivations: - See Opponents’ general motivation above - The enhancements are NBC, relevant scenarios are for cells not serving any UE, is likely to require extensive specification support, and is unlikely to offer NES gains over the gNB using longer transmission periodicities or as the gNB may need to be receiving WUS-type signals and transmitting responses. - Opponents: [C1, C3, C4, C7, C8] Apple, Samsung, Fujitsu, Ericsson, Qualcomm |
| 4 | Cell DTX/DRX enhancements for RRC IDLE/INACTIVE UEs in Scells Proponents and Motivations: - See Proponents’ general motivation above - Extending Cell DTX/DRX operation to common signals in multi cell scenario should be supported in Rel.20 - Transmissions of common signals reduces sleep time of gNB, results in frequent switching between active and sleep states and deduces NES gains significantly - Proponents: [C5] CEWiT Opponents and Motivations: - See Opponents’ general motivation above - Extending cell DTX/DRX to common signals and to RRC idle/inactive was discussed in Rel-18 and was not adopted as it is NBC, has impact at least on UE measurement/mobility procedures, and is unlikely to offer material NES gains - Opponents: [C1, C3, C4, C7, C8] Apple, Samsung, Fujitsu, Ericsson, Qualcomm |
| 5 | On-demand TRS Proponents and Motivations: - See Proponents’ general motivation above - TRS is always on signal. Adaptation of TRS can increase gNB sleep opportunity. - Proponents: [C2] Google Opponents and Motivations: - See Opponents’ general motivation above - No meaningful NES gains from TRS adjustments are expected as such adjustments would be infrequent (i.e. RRC ON/OFF is sufficient) and the cell is not idle before or after a TRS occasion (regardless of TRS transmission). - Opponents: [C1, C3, C4, C7, C8] Apple, Samsung, Fujitsu, Ericsson, Qualcomm |

## 4.5	Moderator’s observations

- Based on the submitted contributions, the situation as compared to RAN#107 remains the same, i.e. there is strong interest in particular for the proposed objectives 1 and 2, but also disagreement to support any proposed NES enhancement for 5G-Adv.

  - Regarding proposed objective 1 (UE recommended Cell DTX/DRX via UAI  for RRC connected UEs)

  - 15 supporting companies

  - 5 non-supporting companies

  - Regarding proposed objective 2 (SSB periodicity extension):

  - 14 supporting companies

  - 5 non-supporting companies

  - Regarding proposed objective 3 (On-demand SSB for RRC idle/inactive UEs)

  - 4 supporting companies

  - 5 non-supporting companies

  - Regarding proposed objective 4 (Cell DTX/DRX enhancements for RRC IDLE/INACTIVE UEs in Scells)

  - 1 supporting companies

  - 5 non-supporting companies

  - Regarding proposed objective 5 (On-demand TRS)

  - 1 supporting companies

  - 5 non-supporting companies

## 4.6	Summary of offline discussion (Tuesday June 10th)

- Moderator minutes from the offline discussion:

  - Due to limited time, the discussion was very rushed and limited.

  - Concern was raised that not all the proponents could be present in the discussion.

  - Moderator suggested to focus only on proposed objective 1. The reason was that other proposal have NBC issues and there is fundamental concern. Also, only few minutes very left for discussion.

  - In this offline discussion, there was no consensus to support the enhancement.

  - Summary of Q&A (on proposed objective 1):

    - Concerning comments:

      - Samsung: What does the UE know that the NW doesn’t know. How UE indication can be tested to ensure proper operations?

      - Xiaomi: Scope is limited but benefit is not clear

      - Apple: What s the metric for the UE recommendation?

      - ZTE: Similarity with UE CDRX.

      - NEC: Real benefit needs large number of UEs supporting the feature.

    - Clarification comments:

    - HW: Some questions (above) can be discussed later. Delegates not present in offline due to collision.

    - HW: NW can take into account the UE recommendation to align transmission to reduce on-time.

## 4.7	Summary of offline discussion (Wednesday June 11th)

Discussion topic:

UE recommended Cell DTX/DRX via UAI for RRC connected UEs [C2, C5, C6]

- Proposed objective 1) Specify UE recommended cell DTX/DRX via UAI (UE Assistance Information) [RAN2]

- Note: The candidate cell DTX/DRX configurations to be reported by UE can refer to the cell DTX/DRX configurations developed in Rel-18

  - HW: explained motivation. Queston on the actual needed procedures and requirements

- MTK: Update the objective to address the target. How to align Cell DTX/DRX to the traffic pattern.

- Nokia: Not realistic. Scenario (traffic ) to bring benefit. Not good way of using RAN2 time.

- HW: We showed some results for some scenarios.

  - Nokia: Not convinced

- Ericsson: If UAI is provided, is single UE recommendation is useful to set cell level DTX/CRX

- NEC: Supportive. Additional information for gNB

- CATT: Should be discussed in RAN2 and with RAN2 Chair.

- RAN2 Chair: No time in RAN2. If we spend time, should be strong support.

  - Proponents can bring as TEI, co-sourced by many companies. The scope and solutions should be really clear to minimize the time needed to support it.

# 5	Other topics

## 5.1	RAN#107 summary and conclusion

The following from RP-250797  shows the outcome of discussing this topic in the last meeting:

Ray tracing model contributions: - Moderator’s observations - As there were no contributions to this topic to RAN#107, hence it is suggested to not consider Ray tracing model enhancements as part of Rel-20 5G-Advanced evolution. “Other” contributions, including MCE - Moderator’s observations - 3 companies (no operator) support the Multicarrier Enhancements. - Moderator suggests to not pursue any work on these topics as part of Rel-20 5G-Advanced based on the limited interest expressed.

## 5.2	List of related contributions

|  | TDoc | Title | Source |
| --- | --- | --- | --- |
| D1 | RP-251423 | Multi-carrier enhancements for Rel-20 5G-A | Lenovo |
| D2 | RP-251674 | Keysight's 5G-Advanced REL-20 RAN-1 led topics proposal | Keysight Technologies UK Ltd |

## 5.3	Proposed objectives

| Topic | Proposed objectives |
| --- | --- |
| 1 | Multi-carrier enhancements [D1] Proposed objectives 1) - Specify enhancement for PUSCH/PDSCH scheduling on multiple carriers with a single DCI [RAN1, RAN4] - Multiple carriers are within a single band - SSB and PDCCH are received on only one carrier - New UE capability to indicate support of multiple carriers with PDCCH monitoring and SSB reception on only one carrier - Note: No new DCI format is introduced. - Specify a solution for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Up to two scheduling cells for a scheduled cell is supported - One scheduling cell for multi-cell scheduling using DCI format 0_3/1_3 and another scheduling cell for single-cell scheduling using DCI format 0_0/1_0/0_1/1_1/0_2/1_2 if configured. |
| 2 | Ray tracing channel modelling [D2] Proposed objectives 2) - Study how to specify ray tracing channel model with sufficient level of detail. Existing optional map-based hybrid model of 38.901 could be used as a starting point. [RAN1] - Study applicable parameters, such as the maximum order of reflections and diffractions to be considered. [RAN1] - Study possibility for specifying reference maps with meshing and material characteristics information and other necessary details for chosen scenarios to enable alignment between different implementations. [RAN1] - Analyze alternatives for a feasible channel model validation process. [RAN1] |
| 3 | - Hz Channel model enhancement [D2] Proposed objective 3) - Revise 3GPP TR 38.901 channel model by introducing specular paths in addition to diffuse scattering. [RAN1] - Revise the cluster structure by introducing non-equal gain sub-paths. [RAN1] |

## 5.4	Companies’ positions and motivations

| Topic | Companies’ motivations and views on proposed objectives |
| --- | --- |
| 1 | Multi-carrier enhancements Proponents and Motivations: - Current CA frameworks require SSB and CORESET configuration on each carrier, leading to complexity and potential PDCCH congestion on scheduling cells. Rel-20 proposals focus on simplifying UE capabilities by allowing SSB and PDCCH monitoring on only one carrier and supporting multiple scheduling cells per scheduled cell to offload PDCCH. - Proponent: [D1] Lenovo. |
| 2 | Ray tracing channel modelling Proponents and Motivations: - Study specification of ray tracing channel models with environment maps, reflection/ diffraction orders, and validation processes. - Dynamic channel conditions, i.e., large scale time evolving channels with continuous motion of transceivers or the environment, and spatially consistent models, could be really useful for ISAC, 7-24 GHz and AI/ML use cases evaluations. - Numerous aspects such as spatial consistency, radiated near-field condition, blockage by objects, and spatial non-stationarity that are difficult for the current stochastic model of TR 38.901 could be inherently included in ray tracing modelling. - Proponent: [D2] Keysight tech. |
| 3 | 7-24 GHz Channel model enhancement Proponents and Motivations: - Current cluster structure defined in TR 38.901 leads to overestimated performance in the case of massive-MIMO caused by the lack of realistic amplitude representation of highly resolved sub-paths. - Proponent: [D2] Keysight tech. |

## 5.5	Moderator’s observations

- Regarding Multicarrier enhancements, similarly, to RAN#107, a single company has proposed the corresponding enhancement. No other company has provided support at RAN#108. This indicates lack of interest to support this enhancement.

- Regarding 7-24 and Ray tracing channel modelling enhancements, there was no submission in RAN#107. Hence, this area of the enhancement was not indicated as focused topics for this meeting. No other company than proponent has provided support at RAN#108.

## 5.6	Summary of offline discussion (Tuesday June 10th)

The enhancements in this section were not discussed.

These enhancements were down-prioritized by Moderator for offline discussion. No concern was raised to this recommendation.

# 6	Conclusion

For the proposed enhancements summarized in this document, based on the submitted contributions, the situation as compared to RAN#107 remains the same, i.e. the views are not converged for none of the proposed enhancements.

## 6.1 Overview of offline discussions

An offline session on June 10th (for 30 minutes) was allocated to discuss the topics. Prior to offline session, Moderator suggested to focus the discussion on the following:

- Support of 200 MHZ CBW

- Support of SBFD enhancements

  - Focus on proposed objectives 1 and 3

- Support for NES enhancements

  - Focus on proposed objectives 1 and 2

Please see the corresponding summary of discussions in Sections 2.6, 3.6 and 4.6. In summary:

- Due to limited time, the discussion was very rushed and limited.

- Concern was raised that not all the proponents could be present in the discussion.

- In this offline discussion, there was no consensus to support any of the enhancement.

- Some new proposals/way forward by some companies are as the following:

  - Regarding support of 200 MHz CBW:

    - HW proposed whether it can be considered as Rel-21 5G-adv.?

  - Regarding SBFD proposed objective 1 (Sub-band level Power backoff):

    - QC asked whether if the proposal can be down-scoped not to have UE impact

  - Regarding SBFD proposed objective 3 (Inter-gNB UE-UE CLI signaling)

    - QC, MTK, ZTE proposed to consider a very small RAN3 WI (amount of work expected as a TEI)

Another offline discussion was scheduled on June 11th. The following topics were discussed during the offline

- Support of SBFD enhancements

  - Focus on proposed objectives 1 and 3

- Support for NES enhancements

  - Focus on proposed objectives 1

Please see the corresponding summary of discussions in Sections 3.7 and 4.7. In summary:

- SBFD enh: Proposed objective 1:

    - It was updated as only RAN4 topic.

    - No consensus offline to support. Proponents can discuss it with other RAN4 topics.

- SBFD enh.: Proposed objective 3:

    - Concerns raised to update the proposal.

    - No objection offline to support if concerns are reflected in the update proposal.

- NES enh.: Proposed objective 1:

    - No consensus offline to support.

    - RAN2 Chair indicated no time as WI. Proponents can propose as TEI in RAN2.

## 6.2	Moderator’s report from offline discussions

### 200 MHZ Channel bandwidth

- There was no consensus to support the enhancement for 5G-advanced.

- It was also proposed to consider 200 MHz CBW for Rel-21 5G-adv , but this proposal was not accepted.

### SBFD enhancements

“Subband-level Power Backoff”

- Updated proposed objective 1) Specify subband-level downlink power back off [RAN4]

  - Extend gNB RE power control dynamic range as needed

- Proposed objective was updated by proponent as above as RAN4 only objective.

- There was no consensus to support updated proposal.

- Moderator suggested the proponents to discuss with other RAN4 topics.

“Dynamic SBFD”

- There was no consensus to support updated proposal.

“Inter-gNB UE-UE CLI signalling”

- Proposed objective 3) Specify signaling enhancements to support inter-gNB coordination of CLI measurement resources (SRS configuration and CLI resources) for co-channel inter-UE CLI measurements [RAN3]

- Summary of offline discussion

- Proposal should be updated to address the following:

  - Only RAN3 WI (no impact on other WGs, specially RAN1)

  - Limited time (preferably one quarter)

  - Scope should clarify: the exchange of measurement resource configuration is on Xn/F1 interface

  - RAN3 Chair assessment

- Considering the above concerns are addressed, no objection from offline discussion.

  - The proposal is updated as the following:

Updated Proposed objective 3) Specify exchange over Xn/F1 of configuration info for one or more SRS resources used for measurement of cochannel UE-UE cross link interference [RAN3]

### NES enhancements

“UE recommended Cell DTX/DRX via UAI for RRC connected UEs ”

- There was no consensus to support the proposal.

- RAN2 Chair: No time in RAN2. Proponents can propose it as TEI.

  - If proposed as TEI, it should be co-sourced by many companies. The scope and solutions should be really clear to minimize the time needed to support it.

“Other proposed NES enhancements (SSB periodicity, on-demand SSB/TRS, etc)”

- There was no consensus to support any of the proposals.

### Other enhancements

- There was no consensus to support the proposals.

## 6.3	Outcome of RAN#108

The following captures the outcome of the discussions from the online session in RAN#108.

On SBFD enhancements:

Updated Proposed objective 3) Specify exchange between gNBs over Xn/F1 of configuration info for one or more SRS resources used for L1 measurement of cochannel UE-UE cross link interference [RAN3]

- Note: RAN3 will not specify network signaling to support inter-operator coordination for CLI handling

- Update Rel-19 SBFD WID to reflect the above

On NES enhancements:

The following proposal can be considered in RAN2 for TEI19 or TEI20

- UE recommended Cell DTX/DRX via UAI for RRC connected UEs

# References

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| RP-251350 | Views on Rel-20 5G-Advanced general RAN1/RAN2/RAN3 scope | Apple | 15 |
| RP-251516 | General views on Rel-20 5G-Adv RAN1/2/3 package | Intel Corporation | 15.0 |
| RP-250869 | NR SBFD Phase 2 in Release-20 | KT Corp., China Unicom, Turkcell, Vodafone, CEWiT, ETRI, Fraunhofer IIS, Fraunhofer HHI, Google, Huawei, HiSilicon, IIT Kanpur, NEC, Sharp, Tejas Networks, Wilus, Yonsei University | 15.1.6 |
| RP-250989 | Views on Rel-20 NES Enhancement | Google | 15.1.6 |
| RP-251026 | Views on Other RAN1-led topics | Samsung | 15.1.6 |
| RP-251249 | Views on Rel-20 Network energy saving for 5G-Advanced | Fujitsu Limited | 15.1.6 |
| RP-251267 | Views on Rel-20 Suband Full Duplex | Ericsson | 15.1.6 |
| RP-251280 | WF on 200 MHz Channel Bandwidth | KDDI, China Unicom, China Telecom, BT, Bouygues Telecom, Deutsche Telekom, KPN, Vodafone, Orange, Odido, Telefonica, Telenor, TIM, Telia Company, Spark NZ, Jio Platforms, Telstra, NTT DOCOMO, SoftBank, CKH IOD UK LTD, Ericsson, Huawei | 15.1.6 |
| RP-251423 | Multi-carrier enhancements for Rel-20 5G-A | Lenovo | 15.1.6 |
| RP-251424 | Supporting 200MHz channel bandwidth for Rel-20 5G-A | Lenovo | 15.1.6 |
| RP-251521 | Network Energy Saving Enhancements for 5G-Advanced in Rel. 20 | CEWiT | 15.1.6 |
| RP-251648 | Network energy saving enhancements for 5G-Advanced in Rel-20 | Huawei, HiSilicon, TIM, Orange, Deutsche Telekom, Telefonica, China Unicom, Fraunhofer IIS, Fraunhofer HHI, CAICT, TCL, CEWiT, Sharp, Rohde & Schwarz | 15.1.6 |
| RP-251649 | Rel-20 SBFD | Huawei, Hisilicon, Deutsche Telekom | 15.1.6 |
| RP-251650 | Rel-20 Efficent Utilization of operating band larger than 100 MHz | Huawei, HiSilicon | 15.1.6 |
| RP-251674 | Keysight's 5G-Advanced REL-20 RAN-1 led topics proposal | Keysight Technologies UK Ltd | 15.1.6 |
| RP-251687 | Views on Rel-20 Network Energy Saving | Ericsson | 15.1.6 |
| RP-251711 | Qualcomm views on other RAN1-led topics | Qualcomm Incorporated | 15.1.6 |
| RP-251724 | Views on further network energy saving enhancement in 5G-A Rel-20 | ZTE Corporation, Sanechips | 15.1.6 |
| RP-251750 | Enhancements to support inter-UE CLI signaling for measurement resource configuration | Charter Communications, Inc | 15.3.4 |