3GPP TSG RAN Meeting #108	RP-251860

Prague, Czech Republic, June 9-13, 2025	(revision of RP-yyxxxx)

# Source:	Qualcomm

# Title:	New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

# Document for:	Approval

Agenda Item:	15.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

Acronym:	NR_AIML_air_Ph2

Unique identifier:	XXX

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-20

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
| X | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_NR_AIML_air | RAN1 | 940084 | Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface |
| FS_NR_AIML_air_Ph2 | RAN1 | 1060078 | Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| NR_AIML_air | 1020093 | Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface | Preceding work item. |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

Dependency on non-3GPP (draft) specification:

None.

# 3	Justification

The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_air and FS_NR_AIML_air_Ph2. Normative support for the general framework for AI/ML for air interface and some of the use cases studied in FS_NR_AIML_air was provided through the NR_AIML_air related project.

In this work item, we provide the normative support for the CSI feedback enhancement (two-sided model) use case as well as standards-based UE data collection.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

Provide specification support for the following aspects:

AI/ML framework for two sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_air_Ph2 and NR_AIML_air projects for CSI feedback enhancement [RAN2]:

- Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model selection, activation, deactivation, switching, fallback

  - ID related signalling is part of the above objective

- Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring,.

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]:

- CSI spatial/frequency compression without temporal aspects (“Case 0”),

- Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including:

  - Model pairing procedure including ID and applicability reporting

  - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports

  - NW and UE side data collection for training,

    - Target CSI format

    - Note The framework defined in BM and CSI prediction use cases could be reused

  - Specify performance monitoring

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Data collection [RAN2]:

- Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection

Interoperability and RRM requirement [RAN4]:

- To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration

- Necessary RRM requirement for specified LCM procedures.

- NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Performance requirements [RAN 4]:

- Demodulation requirement for CSI compression Case 0

- RRM performance test cases for the corresponding RRM requirement for LCM procedures.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.300 | Feature introduction. | TSG#115 | Core part |
| 38.214 | Feature introduction. | TSG#115 | Core part |
| 38.331 | Feature introduction. | TSG#115 | Core part |
| 38.133 | Feature introduction. | TSG#115 | Core part |
| 38.401 | Feature introduction. | TSG#115 | Core part |
| 38.133 | Feature introduction. | TSG#117 | Perf. Part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Juan Montojo, Qualcomm Inc., juanm@qti.qualcomm.com [RAN1]

Xiaofeng Liu, CAICT, liuxiaofeng1@caict.ac.cn [RAN4]

Marco Belleschi, Ericsson, marco.belleschi@ericsson.com, [RAN2]

Fahad Syed Muhammad, Nokia, fahad.syed_muhammad@nokia.com [RAN3]

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

Primary: RAN1

Secondary: RAN2, RAN3, RAN4

# 8	Aspects that involve other WGs

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: possible values: "Not applicable"

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

Qualcomm

CAICT

Ericsson

Nokia

CATT

CEWiT

China Telecom

China Unicom

ETRI

FirstNet

Futurewei

III

IITM

Intel

InterDigital

ITRI

Kyocera Corporation

LG Electronics

MediaTek

NEC

Nvidia

Ofinno

Panasonic

Samsung

Sanechips

SK Telecom

Spreadtrum

Telefonica

UNISOC

vivo

ZTE