3GPP TSG RAN Meeting #108 											RP-251877

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.3

Source:	Moderator (RAN1 Vice-Chair)

Title:	Moderator's summary for Enhancements for solutions for Ambient IoT (Internet of Things) in NR

Document for:	Discussion and decision

# Introduction

At RAN#108, 29 contributions were submitted to address the scope of Rel-20 work on Ambient IoT. This document provides draft SID objectives and draft WID objectives as per the scope and workplan proposed by the RAN Chair and WG Chairs in RP-250859, considering input from contributions provided to RAN#108. The moderator proposal also considers focusing the scope were possible to fit within the allocated time units, while leaving sufficient room for WGs to decide on specific technical solutions.

# Background and Workplan

RP-250859 recommends the following workplan and high-level scope for Ambient IoT in Rel-20:

- Study item (9 months, start in 2025.Q3) – RAN1 and RAN4

  - Study outdoor scenarios including whether new device type is necessary or existing device type can be used (e.g. device 2b in outdoor scenarios)

  - In RAN#111 (March, 2026), decide whether or not to include outdoor scenarios as part of Rel-20 A-IoT WI

- Work item (start in 2025.Q4 in RAN2 and RAN3, start in 2026.Q1 in RAN1)

  - Specify support for D1T1 + Device 2b (RAN1-led), D2T2 + Device 1/2b (RAN2-led), DO-A traffic

  - Update the WID to reflect outcome of Rel-20 A-IoT study item on outdoor scenarios in RAN#111

The draft SID and draft WID in sections 4 and 5 are provided separately according to the above. Note that for practical purpose, the moderator suggests that PHY design for device 2b should also be part of the study for outdoor scenarios, and reused for specifying device 2b for indoor scenarios in the WID.

The justification section provided in section 3 is suggested to be kept common for the SID and the WID.

# Justification section for SID and WID

In recent years, IoT has attracted much attention in the wireless communication world. More ‘things’ are expected to be interconnected for improving productivity efficiency and increasing comforts of life. Further reduction of size, complexity, and power consumption of IoT devices can enable the deployment of tens or even hundreds of billion IoT devices for various applications and provide added value across the entire value chain.

TR 22.840 and TS 22.369 have been developed by SA1 to capture use cases, traffic scenarios, device constraints of ambient power-enabled Internet of Things and identify new potential service requirements as well as new KPIs.

Considering the limited size and complexity required by practical applications for A-IoT devices, the output power of energy harvester is typically from 1µW to a few hundreds of µW. Existing cellular devices may not work well with energy harvesting due to their peak power consumption of higher than 10mW.

Example applications in TR 22.840 and TS 22.369 are asset identification or inventory, which presently has to resort mainly to barcode and RFID in most industries. Other examples are sensor applications, which have characteristics such as the operating conditions or cost requirements, etc., which mean they cannot be addressed by 3GPP’s LPWA technologies. In general, the use cases described by SA1 span indoor and outdoor applications. Applications are also shown to have requirements for positioning, with accuracy targets varying between indoor and outdoor scenarios.

Through Rel-18 and Rel-19 a set of SIs and WIs were completed across 3GPP’s WGs, leading to the specification of Device 1 (having ~1 μW power consumption, and with D2R transmission by backscattering) for use in Deployment Scenario 1 with Topology 1, i.e. by direct connection between a BS reader and the A-IoT device.  The traffic types supported are DO-DTT (device originated by device terminated trigger) and DT (device terminated), with a focus on supporting use cases for indoor inventory and indoor command applications.

In Rel-20, it is proposed to follow-up by supporting

- Topology 2

- A-IoT devices which do not rely on backscattering

- Outdoor scenarios and use cases

- Sensor applications.

- [Positioning] (pending RAN#109 decision)

This will address several aspects of the preceding SIs which were not standardized in Rel-19, and for which there are commercial demands to satisfy.

As with earlier Releases, Rel-20 A-IoT technology shall provide complexity and power consumption orders of magnitude lower than the existing 3GPP LPWA technologies (e.g. NB-IoT and eMTC), and the features of 6G relevant to IoT, and shall address use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies.

# Draft SID Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

## Objective

This SI is to study a second phase of RAN aspects for outdoor scenarios of Ambient IoT, a 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power, nor the features of 6G relevant to IoT.

Note: This SI shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc., as well as the features of 6G relevant to IoT. The study shall not aim to replace existing 3GPP LPWA technologies.

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI and WI in RAN WGs are taken into this SI.

A-IoT Devices

These are the A-IoT Devices referred to in the objectives:

- Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions.

~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

- Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

- SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19.

Objectives

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]:

- Deployment scenario 4 with topology 1 with no external carrier wave.

- Traffic types DO-DTT, DT, DO-A.

- Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command).

- Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.

- FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

- Assumption of the same air interface for Device 2b and Device C.

  - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

- Positioning for outdoor scenarios is under objective 2.

This objective begins after RAN#108 (June 2025)

RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work.

This objective includes:

RAN1-led

- Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios

  - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios

- Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C.

- While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming

  - Maximum distance between Reader and Device of 50-500 m

  - Maximum transmit power between -3 dBm and +5 dBm for device C

    - RAN1 to recommend feasible values within this range

  - Maximum transmit power of -20 dBm and -10 dBm for device 2b

Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources.

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations.

- Define link budget calculation for coverage.

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

- RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item

## Expected Output and Time scale

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TR 38.769 | Study on solutions for Ambient IoT (Internet of Things) in NR | RAN#111 | Core part |

# Draft WID on Solutions for Ambient IoT in NR Phase 2

## Objective

### Objective of SI or Core part WI or Testing part WI

This WI is to study and standardize a second phase of RAN aspects of Ambient IoT, a 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power, nor the features of 6G relevant to IoT.

Note: This WI shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc., as well as the features of 6G relevant to IoT. The study shall not aim to replace existing 3GPP LPWA technologies.

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI and WI in RAN WGs are taken into this WI.

A-IoT Devices

These are the A-IoT Devices referred to in the objectives:

- Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions.

~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

- Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

- SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19.

Specifications shall support active Devices 2b/C that are capable of operating using energy harvesting only.

Work on objectives for active devices and outdoor scenarios will not start until the completion of the study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices.

Objectives

- Specify necessary support for active Device(s), taking the Rel-19 specifications as the starting point, in indoor scenarios under the following conditions [RAN1]:

- Deployment scenario 1 with topology 1, according to D1T1-C.

- Traffic types DO-DTT, DT, DO-A.

  - For DO-A traffic, WGs begin discussions from Clause 6.10 “DO-A assessment” in TR 38.769.

- Representative use cases rUC1 (indoor inventory), rUC2 (indoor sensor), and rUC4 (indoor command).

- Assumption that A-IoT BS readers are deployed on the same sites as existing indoor NR micro BSs.

- FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

This objective begins after RAN#111 (March 2026)

This objective includes:

RAN1-led

- In specifying support for the above, re-use the evaluation assumptions of TR 38.769

- Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

- Air interface for active device(s) studied for outdoor will be reused for supporting active device(s) in indoor scenarios

RAN2-led

- DO-A specific procedure design.

- Paging and random access for active device(s).

RAN3-led

- Specify DO-A specific NGAP procedure.

RAN4-led

- Study coexistence between active Device(s) and NR/LTE in indoor scenarios [RAN4-led]

- Specify RF requirements for Type 1-C Ambient IoT BS where differences are needed from Rel-19.

- Specify RF requirements for active Device(s) with IF/ZIF receiver architecture

- Specify RRM core requirements for active Device(s) in D1T1

- Use band n8 as an example band

- Study and specify the test methodology for AIoT active devices

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2.

- The following are not supported:

  - Coordination between UEs operating as UE readers

  - New Uu protocol layer(s)

  - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band

This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1.

The work is subject to the following constraints:

RAN1

- The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS.

- No physical layer impact for signaling A-IoT resources to the intermediate UE.

- No additional specification change on R2D and D2R for Device 1 in Rel-20

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

RAN3

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

RAN4

- Specify RF requirements for intermediate UE towards the A-IoT device.

- Specify necessary updates to RF requirements for A-IoT device 1.

- Specify A-IoT active device(s) RF requirements in D2T2

- Specify RRM requirements for intermediate UE due to A-IoT operation, if any. Combination with existing RRM procedures/requirements is excluded.

- Specify RRM requirements for Device(s) in D2T2 if necessary

- Use band n8 as an example band.

- Specify device (un)availability via Direction 2 in TR 38.769, [RAN2-led, RAN1].

- This objective is applicable only for active Device(s)

- Solutions shall be unified for Topology 1 and Topology 2.

- This objective begins after RAN#111 (March 2026)

- NOTE: Device for outdoor scenarios may be added here, pending RAN#111 decision.

Note: Coexistence between Device 1 and Device 2b/C is not considered in the same deployment in the same band.

### Objective of Performance part WI

- Specify the necessary conformance testing for Ambient IoT BS

- Specify the necessary RRM performance requirements for Device 1 and active Device(s)

- Specify the necessary demodulation performance requirements for Device 1 and active Device(s), and Ambient-IoT BS

- Specify the necessary performance requirements for intermediate UE towards Device 1 and active Device(s)

## Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| New RAN4 TR | TR 38.xxx | Ambient IoT coexistence study for for active devices | N/A | RAN#115 | RAN4 coexistence study |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.201 | NR; Physical layer; General description | RAN#114 | Core part |
| TS 38.202 | NR; Services provided by the physical layer | RAN#114 | Core part |
| TS 38.212 | NR; Multiplexing and channel coding | RAN#114 | Core part |
| TS 38.213 | NR; Physical layer procedures for control | RAN#114 | Core part |
| TS 38.214 | NR; Physical layer procedures for data | RAN#114 | Core part |
| TS 38.291 | Ambient IoT Physical layer | RAN#114 | Core part |
| TS 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#115 | Core part |
| TS 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#115 | Core part |
| TS 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#115 | Core part |
| TS 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#115 | Core part |
| TS 38.391 | Ambient IoT Medium Access Control (MAC) protocol | RAN#115 | Core part |
| TS 38.401 | NG-RAN; Architecture description | RAN#115 | Core part |
| TS 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#115 | Core part |
| TS 38.410 | NG-RAN; NG general aspects and principles | RAN#115 | Core part |
| TS 38.423 | NG-RAN; Xn application protocol (XnAP) | RAN#115 | Core part |
| TS 38.470 | NG-RAN; F1 general aspects and principles | RAN#115 | Core part |
| TS 38.473 | NG-RAN; F1 Application Protocol (F1AP) | RAN#115 | Core part |
| TS 38.191 | Ambient IoT device radio transmission and reception | RAN#115 | Core part |
| TS 38.194 | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception | RAN#115 | Core part |
| TS 38.101-1 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN#115 | Core Part |
| TS 38.133 | NR; Requirements for support of radio resource management | RAN#115 | Core Part |
| TS 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#115 | Core part |
| TS 38.133 | NR; Requirements for support of radio resource management | RAN#117 | Performance Part |
| TS 38.101-4 | NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements | RAN#117 | Performance Part |
| TS 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#117 | Performance part |
| TS 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | RAN#117 | Performance part |
| TS 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | RAN#117 | Performance part |
| TS 38.195 | Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing | RAN#117 | Performance part |
| TS 38.191 | Ambient IoT device radio transmission and reception | RAN#117 | Performance part |
| TR 38.769 | Study on solutions for Ambient IoT (Internet of Things) in NR | RAN#115 | RAN4 coexistence study |

# References

- RP-250863	Ambient-IoT Rel-20 WI scope	Futurewei

- RP-250972	Discussion on NR Ambient IoT work scope in Rel-20	OPPO

- RP-250978	Views on AIoT WI in Rel-20	Fujitsu Limited

- RP-250981	LGE views on Rel-20 Ambient IoT	LG Electronics Inc.

- RP-251023	Views on scope of 5G-A Rel-20  Ambient IoT	Samsung

- RP-251042	View on enhancements for Ambient IoT in NR	NEC

- RP-251135	Rel-20 RAN1-led items: Ambient IoT	Nokia

- RP-251196	Enhancements for solutions for Ambient IoT (Internet of Things) in NR	Tejas Networks Limited

- RP-251205	Scope of Ambient IoT in 5G-A Rel-20	vivo

- RP-251222	WID scope consideration on Rel-20 Ambient IoT	Spreadtrum, UNISOC

- RP-251236	Views on the Scope of Rel-20 AIoT	Sony Europe Limited

- RP-251261	Rel-20 Ambient IoT scope	Huawei, HiSilicon

- RP-251265	Views on Rel-20 Ambient IoT	Ericsson, Nokia, NTT DOCOMO, Verizon

- RP-251282	Views on Rel-20 Ambient IoT scope	Xiaomi

- RP-251297	Rel-20 Ambient IoT Scope	MediaTek Inc.

- RP-251327	Views on Rel-20 Ambient IoT	IIT Kanpur

- RP-251333	Views on Rel-20 for Ambient IoT	Everactive

- RP-251349	Operator Views on Ambient IoT enhancement in 5G-A Rel-20	T-Mobile USA Inc.

- RP-251382	Consideration on Ambient IoT for Rel-20	CMCC

- RP-251404	Scope discussion on Enhancements for solutions for Ambient IoT	Panasonic

- RP-251413	Views on Rel-20 Ambient IoT enhancements	NTT DOCOMO, INC.

- RP-251422	Enhancements for Rel-20 Ambient IoT	Lenovo

- RP-251434	Views on Ambient IoT in Rel-20	Lekha Wireless Solutions

- RP-251519	Enhancements for solutions for Ambient IoT (Internet of Things) in NR	CEWiT

- RP-251636	Views on Ambient IoT works in Rel-20	CATT

- RP-251655	Scope for Ambient IoT for Rel-20	InterDigital, Inc.

- RP-251660	Views on enhancements for Ambient IoT in NR	Apple

- RP-251709	Qualcomm views on Ambient IoT WI in Rel-20	Qualcomm Incorporated

- RP-251721	Views on Ambient IOT enhancement in 5G-A Rel-20	ZTE Corporation, Sanechips