3GPP TSG RAN Meeting #109	RP-251955

# Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.1

Source:	Samsung, Verizon

Title:	Requirements for 6G-6G Spectrum Aggregation

Document for:	Discussion and Decision

1	Introduction

6G Radio standard has to guarantee efficient use of diverse spectrum that will be a typical deployment scenario to provide higher throughput as well as good coverage.

In this contribution, we propose requirements for 6G-6G spectrum aggregation for efficient utilization of diverse spectrum resource.

2	Discussion

In 6G systems, multi-band spectrum aggregation is expected to play a critical role in enabling higher throughput, enhanced spectral utilization, and flexible resource coordination across different frequency ranges. As the deployment of 6G spans diverse spectrum bands—including mid-band (e.g., ~7 GHz) and mmWave—spectrum aggregation scenarios will increasingly involve both co-site and inter-site configurations.

In the co-site case, where multiple carriers are transmitted from the same physical location or baseband processing unit, maintaining a low-latency interface between bands is essential to support time-sensitive operations such as fast scheduling decisions, hybrid ARQ (HARQ) feedback alignment, and synchronized uplink transmission. Without stringent latency constraints, critical inter-band coordination procedures may suffer, resulting in degraded spectral efficiency or unreliable control signalling. Therefore, a well-defined latency requirement for the co-site aggregation is necessary to ensure consistent performance and predictable scheduling behaviour. In the inter-site scenario, component carriers may originate from geographically separated radio units or distributed cells (e.g., macro + small cell). While such deployments offer greater spectrum flexibility and capacity scaling, they may introduce transport and synchronization delays across sites, making it difficult to maintain cross-carrier scheduling and uplink timing alignment.

Thus, introducing clear latency requirements for both co-site and inter-site 6G-6G spectrum aggregation and the associated mechanism is essential to enable scalable, robust, and high-performance multi-band aggregation, especially under heterogeneous deployment scenarios expected in 6G.

3	Proposal

We propose to agree the following text proposal for TR 38.914.

=== Start Text Proposal ===

5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

< Proposed new section >

5.3.1.x Support of 6G-6G spectrum aggregation

6G Radio shall support intra-6G radio level spectrum aggregation of different bands, e.g., FR1 and FR2-1, including non-collocated deployments with non-negligible transport and synchronization delays across sites.

< End new section >

=== End Text Proposal ===