3GPP TSG RAN #109	RP-251971

Prague, Czech Republic, June 9th – 13th 2025

Title:	Discussion on support of A-IoT positioning or proximity determination in Rel-20

Source:	OPPO

Agenda:	9.2.3

# Introduction

According to SA1 TR 22.840 [1], a total of 30 A-IoT indoor and outdoor use cases are captured and some of which do require the support of a positioning/location service. For example, it is expected to provide positioning service for A-IoT devices, which can be for device management purpose (e.g., to automatically identify the locations of the devices) or for security reason (e.g., to make sure the devices have not been removed). However, the positioning for these purposes is not expected to be performed frequently.

According to the approved Rel-20 SID [2] on enhancements for solutions for A-IoT in NR outdoor for active devices, the pending objective on the support of positioning or proximity determination for A-IoT devices is scheduled to be decided in this RAN#109 meeting.

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - … - RAN#109 to decide whether to include an objective on positioning / proximity determination in the scope for Rel-20 study item

In this contribution, firstly a quick survey on the positioning requirements detailed in [1] and overviews of some existing positioning techniques and proximity determination enhancements are provided. It is then followed by our views and recommendations on whether/how to start this study objective also taking into consideration of number of WG meetings (RAN1) remained in this SI.

# Discussion

## Positioning requirements in SA1 A-IoT use cases

In the following Table 1, a quick survey on the use cases studied in SA1 and captured in TR 22.840 [1] with an outdoor positioning requirement is given.

Table 1: Survey of outdoor positioning requirements in TR22.840

| Use case | positioning latency | positioning availability | positioning accuracy | Note |
| --- | --- | --- | --- | --- |
| 5.3 substations in smart grids | NA | NA | several 10m |  |
| 5.7 airport terminal / shipping port | NA | 90% | cell level |  |
| 5.8 remote lost item | NA | 90% | ~10m | with help of surrounding UEs |
| 5.9 location service (LCS) | 10s | 95% | cell level |  |
| 5.12 personal belongings finding | 1s | 99% | several 10m |  |
| 5.16 automated supply chain distribution | NA | NA | cell level |  |

As it can be seen, except for the use case 5.8 (remote lost item) which may need the help of surrounding UEs, the positioning accuracy requirements in most A-IoT cases are relatively relaxed (i.e., 10s of meters or a cell level) compared to the target horizontal accuracy requirements for NR positioning below [3].

## 5.1 Target requirements In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100 ms) - Physical layer latency for position estimation of UE (< 10 ms) In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< 0.2 m) for 90% of UEs - Vertical position accuracy (< 1 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired) - Physical layer latency for position estimation of UE (<10ms) Note 1: Target positioning requirements may not necessarily be reached for all scenarios and deployments Note 2: For some scenarios the requirement for Horizontal position accuracy can be relaxed to < 0.5 m in IIoT use cases. Note 3: All positioning techniques may not achieve the target positioning requirements over all scenarios

Observation 1: A quick survey of positioning requirements for SA1 A-IoT use cases has shown that the positioning accuracy for outdoor is either 10s of meter or at a cell level, which are considerably more relaxed than the NR positioning requirements for commercial and IIoT use cases (with a horizontal accuracy < 1m and < 0.2m, respectively).

## Supported A-IoT positioning solution in Rel-19

As part of Rel-19 A-IoT study outcome in TR 38.769 [4], it is captured that locating an A-IoT device at "reader ID granularity" is useful for improving the A-IoT operation and providing location information to the customer of the A-IoT service. In Topology 1, for support of A-IoT device location, the A-IoT RAN node may report the A-IoT RAN node ID to the A-IoT CN in the Inventory Report or Command Response messages. The A-IoT RAN node may additionally report a Reader ID to the A-IoT CN in the Inventory Report and/or the Command Response messages, which corresponds to the reader to which the A-IoT device responded. As such, since a RAN node provides coverage for one cell, the reporting of the Reader ID essentially provides a cell-level positioning information to the A-IoT CN.

Furthermore, 2 separate proximity determination solutions were identified in Rel-19, where if the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader with/without measurements at the reader side. It was further discussed in RAN plenary that either solution would not require any air interface support and specification impact in RAN1 and RAN2. In the end, it was RAN3 that needs to specify “device location reporting at reader ID granularity” [5].

Observation 2: In Rel-19, proximity determination for A-IoT devices is supported based on a reader successfully receives D2R transmission from the device in response to R2D transmission and a device location reporting at reader ID granularity to the A-IoT CN. Essentially, the cell level positioning is already supported in Rel-19.

## Existing NR and UE positioning techniques

Positioning techniques studied and developed in NR and LTE are categorised into network-based (NR positioning) and UE-based (UE positioning) solutions.

NR positioning

In NR positioning (BS-based) solutions, techniques studied to determine user location accurately include combination of one or more use of low and high frequency bands (i.e., especially the wide bandwidth in high bands), massive antenna array at the BS to improve the degree of freedom in the spatial and angular domains, and measurement of timing differences in signal arrivals. For some NR positioning solutions, a positioning RS (PRS) transmitted in a wideband manner from the BS and/or UE transmission of SRS is required. To improve performance and accuracy, aggregating multiple DL positioning frequency layers for both intra-band and inter-band scenarios could be also used. Furthermore, techniques for improving the estimations of UL-AoA and DL-AoD were also studied and developed.

For practical use of A-IoT in an outdoor scenario with very low power-consumption and low computational complexity devices (equipped with a single antenna, a maximum/optional R2D reception BW of 540 kHz and D2R transmission BW of not more than 60 kHz), it is not expected for the reader and device to make use of NR positioning techniques mentioned in the above. That is, transmitting a new D2R PRS signal, receiving a reader PRS in R2D and make use of multiple frequency layers (even in intra-band) would seem almost infeasible to achieve an accuracy of several 10s of meters.

Observation 3: For existing/studied NR positioning techniques where the use of wideband signal transmission in more than one carrier/band, massive antenna arrays and sophisticated measurement of time difference in signal arrivals are commonly utilized, but it is not expected these techniques can be adopted for A-IoT usage in the outdoor scenario of device equipped with a single TX/RX antenna, R2D BW of 540 kHz and D2R BW of 60 kHz).

UE positioning

The basic mechanism of UE positioning studied in NR to determine the location of a UE was based on UE involvement in two main steps

-	signal measurements; and

-	position estimate and optional velocity computation based on the measurements.

It is captured in [6] that the signal measurements may be made by the UE or by the network (i.e., serving ng-eNB or gNB). The basic measured signals for terrestrial position methods are typically the LTE or NR radio transmissions; however, other methods may make use of other transmissions such as general radio navigation signals including those from Global Navigation Satellites Systems (GNSSs). The positioning function need not be limited to a single method or measurement, where it should be possible and capable of utilising other standard methods and measurements, as such methods and measurements are available and appropriate, to meet the required service needs of the location service. This additional information could consist of readily available measurements at the network. The position estimate computation can be made by the UE or by the Location Management Function (LMF) at the network side.

Some of standard positioning methods supported can be categorized into:

-	network-assisted GNSS

-	Multi-Round Trip Time Positioning (Multi-RTT) based on NR signals;

-	Observed Time Difference of Arrival (OTDOA) positioning based on LTE signals;

-	Uplink Angle-of-Arrival (UL-AoA) and Downlink Angle-of-Departure (DL-AoD) based on NR signals;

-	Uplink and Downlink Time Difference of Arrival (UL-TDOA and DL-TDOA) based on NR signals;

-	enhanced cell ID (CID) methods based on LTE or NR signals;

Hybrid positioning from using multiple above methods and standalone mode (e.g. autonomous, without network assistance) are both supported.

For supporting A-IoT positioning function in an outdoor environment with coverage up to 500m, as described earlier for a simple low power consumption device with only a single TX/RX antenna and a SFO and CFO that are much larger than a normal UE, most of the above advanced UE positioning methods would be infeasible for the device support (e.g., GNSS, timing and angle estimation related methods).

In the CID positioning method, the position of a UE is estimated with the knowledge of its serving base station and cell. The information about the serving base station and cell may be obtained by paging, registration, or other methods. Enhanced Cell ID (E-CID) based on LTE signals positioning refers to techniques which use additional UE measurements, reporting to the network and/or network radio resource and other measurements to improve the UE location estimate. To reuse this E-CID positioning method in A-IoT, some modifications would be required, and it is further required for a device 2b/C to be capable of measuring a R2D transmission from a reader and performing a feedback reporting procedure. Both of which are unclear at this stage of the study in RAN, and would involve also RAN2 study on the reporting procedure.

Observation 4: Among the UE positioning methods studied and specified in NR, most of the advanced UE positioning methods would be infeasible for support by a A-IoT device (e.g., GNSS, timing and angle estimation related methods).

Observation 5: Modifications to the existing E-CID UE positioning method could be possibly supported by a A-IoT device 2b/C if the device is capable of performing measurement(s) of R2D transmission(s).

Observation 6: So far, it has not been identified or concluded in RAN1 during the Rel-20 study item whether and how device 2b/C has or will have a capability of performing measurement(s) of R2D transmission (e.g., RSRP, RSRQ) and/or a pathloss estimation.

Observation 7: Reporting of measurement(s) of R2D transmission from device to reader would require a reporting procedure to be studied in RAN2.

## Overall

Currently, A-IoT positioning of a device at a cell-level is already supported in Rel-19 based on reader successfully receives D2R transmission from the device in response to R2D transmission and a device location reporting at reader ID granularity to the A-IoT CN. For some of relevant use cases with outdoor positioning requirement, the existing A-IoT proximity determination mechanism can be considered suffice. If further enhancement is needed to improve the positioning accuracy to 10s of meters, majority of existing Uu positioning methods are infeasible for use as they would require much larger TX/RX bandwidths for PRS transmission, define new R2D/D2R PRS, device multi-antenna support for estimation, much stringent SFO/CFO tolerance requirements, measurement of R2D signals, and etc.

From project management planning perspective, if the E-CID positioning method is reused and modified, it is unclear whether the remaining 3 RAN1 meetings would be sufficient to study the necessary and feasible modifications, perform A-IoT positioning evaluations and investigate potential measurement reporting procedure(s), which requires RAN2 involvement.

Observation 8: Considering only very limited RAN1 meetings remaining to complete the study item for active devices in outdoor operation, the inclusion/commencement of a A-IoT positioning or proximity determination method in the SI scope seems very ambitious.

Proposal 1: It is recommended not to commence the objective 2 in SID on positioning / proximity determination. If it is absolute essential including this objective, the study scope should to be confined to:

- Firstly, A-IoT positioning enhancement in Rel-20 should use the existing E-CID positioning method as a starting point and study necessary modifications/enhancements for A-IoT outdoor operation.

- Secondly, no or minimal additional device function(s) / capability(ies) should be required to support A-IoT proximity determination that is not already supported in Rel-19 (i.e., device location reporting at reader ID granularity to the A-IoT CN).

# Conclusions

Overall, our observations and proposal on the feasibility to commence and potential scope of objective 2 on positioning or proximity determination for A-IoT outdoor active devices in Rel-20 are as followed.

Observation 1: A quick survey of positioning requirements for SA1 A-IoT use cases has shown that the positioning accuracy for outdoor is either 10s of meter or at a cell level, which are considerably more relaxed than the NR positioning requirements for commercial and IIoT use cases (with a horizontal accuracy < 1m and < 0.2m, respectively).

Observation 2: In Rel-19, proximity determination for A-IoT devices is supported based on a reader successfully receives D2R transmission from the device in response to R2D transmission and a device location reporting at reader ID granularity to the A-IoT CN. Essentially, the cell level positioning is already supported in Rel-19.

Observation 3: For existing/studied NR positioning techniques where the use of wideband signal transmission in more than one carrier/band, massive antenna arrays and sophisticated measurement of time difference in signal arrivals are commonly utilized, but it is not expected these techniques can be adopted for A-IoT usage in the outdoor scenario of device equipped with a single TX/RX antenna, R2D BW of 540 kHz and D2R BW of 60 kHz).

Observation 4: Among the UE positioning methods studied and specified in NR, most of the advanced UE positioning methods would be infeasible for support by a A-IoT device (e.g., GNSS, timing and angle estimation related methods).

Observation 5: Modifications to the existing E-CID UE positioning method could be possibly supported by a A-IoT device 2b/C if the device is capable of performing measurement(s) of R2D transmission(s).

Observation 6: So far, it has not been identified or concluded in RAN1 during the Rel-20 study item whether and how device 2b/C has or will have a capability of performing measurement(s) of R2D transmission (e.g., RSRP, RSRQ) and/or a pathloss estimation.

Observation 7: Reporting of measurement(s) of R2D transmission from device to reader would require a reporting procedure to be studied in RAN2.

Observation 8: Considering only 3 RAN1 meetings remaining to complete the study item for active devices in outdoor operation, the inclusion/commencement of a A-IoT positioning or proximity determination method in the SI scope seems very ambitious.

Proposal: It is recommended not to commence the objective 2 in SID on positioning / proximity determination. If it is absolute essential including this objective, the study scope should to be confined to:

- Firstly, A-IoT positioning enhancement in Rel-20 should use the existing E-CID positioning method as a starting point and study necessary modifications/enhancements for A-IoT outdoor operation.

- Secondly, no or minimal additional device function(s) / capability(ies) should be required to support A-IoT proximity determination that is not already supported in Rel-19 (i.e., device location reporting at reader ID granularity to the A-IoT CN).

# References

[1] 3GPP TR 22.840 V19.0.0 (2023-12) “Study on Ambient power-enabled Internet of Things”

[2] RP-251884 “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108, June 2025

[3] 3GPP TR 38.857 V17.0.0 (2021-03) “Study on NR Positioning Enhancements”

[4] 3GPP TR 38.769 V19.0.0 (2024-12) “Study on solutions for Ambient IoT (Internet of Things) in NR”

[5] RP-250796 “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#107, March 2025

[6] 3GPP TS 38.305 V18.6.0 (2025-06) “Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN”