3GPP TSG RAN Meeting #109					RP-251989

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	T-Mobile USA, AT&T, Ericsson, Nokia, SK Telecom

Title:	On support of sensing in 6G

Document for:	Approval

# 1	Introduction

Radio sensing describes the capability to detect passive objects by means of transmitting a radio signal, receiving the signal after it interacted with the environment/objects, and drawing conclusions about the environment/objects, based on the received signal. Integration of this capability into a wireless communication network is termed Integrated Sensing and Communication (ISAC) (a.k.a Joint Communication and Sensing, JCAS).

ISAC is a candidate feature to be included in NR Rel-20 with focus to detect and track Unmanned Aerial Vehicles (UAV) using base stations as sensing units. Many different use cases have been mentioned in the past, including among others use cases related to UAV detection, smart transportation, industries, public safety.

# 2	Discussion

The RAN1 study “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR” (RP-240799) develops a channel model supporting object detection and/or tracking which includes among others the following objects: UAVs, humans indoors and outdoors, automotive vehicles, automated guided vehicles (e.g. in indoor factories), objects creating hazards on roads/railways. The developed channel model should support TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, and UE monostatic sensing modes.

Detection and tracking of UAVs is one of the currently most discussed applications of ISAC. A concrete example relates to protection of critical infrastructure where the already deployed mobile communication networks are used to detect UAVs in prohibited airspace. ISAC could also be used to track commercial drones and provide assistance data to the UAV traffic management system for navigation and collision avoidance. In many jurisdiction UAVs must send beacons to avoid collisions among UAVs, ISAC could complement such a system.

ISAC can also play an important role in smart transportation. Most modern cars are equipped with onboard sensors – radar, lidar, cameras just to mention a few – which helps the car to get an overview of its immediate surrounding. However, onboard systems fail to provide global contextual information. An ISAC system can complement onboard sensory data with more “global” data that cannot be obtained by sensors mounted on the vehicle. Another automotive related example is detection of pedestrians or animals on highways.

Even non-automotive transportation such as rail transportation can benefit from ISAC: Mobile networks – and especially dedicated railway deployments – can be used to detect persons, animals, obstacles on train tracks and provide this information to train guidance systems to take counter measures.

Modern industries and logistic operations often use Automated Guided Vehicles (AGVs) and Autonomous Mobile Robots (AMRs). At the same time more and more industries rely on private wireless networks deployed to cater for the special needs of industrial and logistical operations. ISAC can be deployed using these networks to assist in AGV/AMR navigation but also collision avoidance among them and humans.

The challenge in detecting/localizing/tracking ground targets is the environment: The line-of-sight link between base stations serving as transmitter and receiver and the target may often be blocked by objects in the environment such as buildings. A solution to improve line-of-sight likelihood is densification of sensing units. This can either be achieved by adding TRPs, and/or to enable UEs as sensing units. To avoid challenges arising from using “classical smartphones” as sensing units (unknown UE location and/or orientation, UE movements) such UEs should be assumed stationary.

6G shall therefore provide support for TRP monostatic and TRP-TRP bi-/multi-static sensing modes.

In addition to this we see a need for 3GPP to evaluate and conclude on the merits of TRP-UE bi-/multi-static sensing, UE-TRP bi-/multi-static sensing, UE-UE bistatic sensing and UE mono-static sensing modes. After the evaluation and study has been done, TSG RAN will decide whether or not they are supported.

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x	Support of sensing in 6G

The 6GR and 6G RAN architecture shall support detection and/or tracking of passive objects, in the air and on the ground. Passive objects to detect and/or track could include UAVs, pedestrians, cars, traffic obstacles, AGVs/AMRs in industries. The supported sensing modes include TRP mono-static and TRP-TRP bi-/multi-static.

Editor’s Note: Additional sensing modes can be added after the study of them is completed.

# 4	Conclusion

In this contribution we propose

- To add the text proposal in section 3 to TR 38.914

- TSG RAN tasks the RAN working groups and SA2 to study and evaluate the feasibility of TRP-UE bi/multi-static, UE-TRP bi/multi-static sensing, UE-UE bistatic sensing and UE mono-static sensing modes.

  - After the study is concluded TSG RAN will decide whether they should be supported.