3GPP TSG RAN Meeting #109	                                 RP-252025

Beijing, China, September 15-18, 2025

Source: 	ZTE Corporation, Sanechips

Title:	Text proposal to draft TR 38.914 on KPIs

Agenda item:	8.2.1.2

Document for:	Agreement

- Introduction

This contribution gives text proposal for TR 38.914 on key performance indicators (KPIs) for the RAN 6G SI “Study on 6G Scenarios and requirements”. This TP is to implement the suggested TPR / KPI values as described in RP-252023 and RP-252024.

- Text Proposal for TR 38.914

<start of change 1>

### 5.1.1	Peak data rate

Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is 100 Gbit/s.

–	Uplink peak data rate is 50 Gbit/s.

### 5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is 75 bit/s/Hz.

–	Uplink peak spectral efficiency is 37.5 bit/s/Hz.

### 5.1.3	5th percentile user data rate

Editor note: name of this KPI may need to be changed

5th percentile user data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For the evaluation of uplink 5th percentile user data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account.

For the evaluation of downlink 5th percentile user data rate, in case of one frequency band and one layer of transmission reception points (TRxP), the 5th percentile user data rate could be derived from the 5th percentile user spectral efficiency, let W denote the channel bandwidth and SEuser denote the 5th percentile user  spectral efficiency. Then the 5th percentile user data rate, Ruser is given by:

Ruser = W × SEuser

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the 5th percentile user data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

–	Downlink 5th percentile user data rate is 500 Mbit/s.

–	Uplink 5th percentile user data rate is 50 Mbit/s.

### 5.1.4	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for 5th percentile user spectral efficiency for various [test environments] are summarized in Table 5.1.4-1.

Table 5.1.4-1: 5th percentile user spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – IC | 0.9 | 0.63 |
| Dense Urban – IC | 0.675 | 0.45 |
| Rural – IC | 0.36 | 0.135 |

### 5.1.5	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency for various [test environments] are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – IC | 27 | 20.25 |
| Dense Urban – IC | 23.4 | 16.2 |
| Rural – IC | 9.9 | 4.8 |

### 5.1.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirement for area traffic capacity in downlink is 50 Mbit/s/m² in the Indoor Hotspot – IC [test environment].

<end of change 1>

<start of change 2>

### 5.1.9	Connection density

Connection density is the number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is 106 connections per km2.

<end of change 2>

<start of change 3>

### 5.1.12	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for mobility interruption time is 0 ms.

<end of change 3>

<start of change 4>

### 5.1.14	Positioning

Positioning is the ability to estimate the position of mobile devices. The positioning accuracy is the 90% point of the CDF of the device positioning error for both horizontal and vertical directions.

Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude).

The minimum requirements for the device positioning accuracy are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| Test Environment | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- |
| Indoor Factory - ISAC | 20cm-1m | 5m |
| Urban Macro – ISAC | 3m | 5m |

<end of change 4>

<start of change 5>

### 5.1.16	Composite requirement

Editor note: KPI related to data rate, latency, reliability, capacity. Name of this KPI may need to be changed

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per km2.

This requirement is defined as the number of users per km2 of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

# The composite requirement KPI is summarized in Table 5.1.16-1.

Table 5.1.16-1 Composite requirement in each scenario

| Scenario | Data rate (Mbits/s) | Latency (ms) | Packet success probability | Number of users (/km2 ) |
| --- | --- | --- | --- | --- |
| Dense Urban | DL = 45 Mbps video + 100 bytes/s haptic UL = 20 Mbps | DL = 10 ms UL = 10 ms | 99% | FFS |
| Indoor Factory | FFS | FFS | FFS | FFS |

### 5.1.17	Sensing

5.1.17.1	Detection Probability and False Alarm Probability

Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

5.1.17.2	Localization Accuracy

Localization accuracy is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error.

5.1.17.3	Velocity Estimation Accuracy

Velocity estimation accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

The minimum requirements for sensing related capabilities are summarized in Table 5.1.17-1.

Table 5.1.17-1 Sensing related capabilities

| Requirement |  | Test Environment |  |
| --- | --- | --- | --- |
|  |  | Indoor factory-ISAC | Urban Macro-ISAC |
| Detection Probability |  |  | 95% |
| False Alarm Probability |  |  | 5% |
| Localization accuracy | Horizontal |  | 2m |
|  | Vertical |  | 5m |
| Velocity accuracy |  |  | 1m/s |

<end of change 5>

-