3GPP TSG RAN Meeting #109                                                 RP-252034

Beijing, China, September 15-18, 2025

Title: 	Views on Rel-20 BS RF topics for 5G-A

Source: 	ZTE Corporation,Sanechips

Agenda item:	9.1.4.2

Document for:	Discussion

Introduction

In RAN#108 meeting, there were lots of discussions on Rel-20 RAN4 package and the discussion outcomes are endorsed in the way forward on RAN4 R20 package [1]. In this contribution, we would like to share some further views on Rel-20 BS RF topics for 5G-A.

Discussion

- 2.1. BS RF enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ Rel-20 BS RF proposals so far are summarized in the following table.

The high-level scope for WI on BS RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - Enhancement of co-location requirements - It is FFS on the applicable frequency ranges, e.g. >3GHz requirements, for OTA - R19 outcome should be considered - Coexistence requirement enhancement can be further discussed in RAN#109 - Down select from the following four options in RAN#109 - Additional Hybrid requirement sets for BS type 1-H/large antenna array - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - It is limited to RAN4 only solution - Neither - Both - No RAN1/2 impact is expected - Target 0.5RF TU in Q4’25 and 1 TU allocation from Q1’26.

ZTE’s views on Additional Hybrid requirement sets for BS type 1-H/large antenna array, we suggest to discuss this issue in 6G SI phase since the physical layer needs also to be optimized for the analog beamforming in FR1.

Proposal 1: Postpone the FR1 hybrid beamforming into 6G SI phase.

Proposal 2: Consider the justifications and objectives in sections 2.1.2 and 2.1.3 for the WID of BS RF.

- 2.1.2 Justification

Objective 1:

The existing BS co-location requirement in FR1 relies on a Maximum Coupling Loss (MCL) assumption of 30 dBc, which was adopted from FR1 low bands dating back to the UTRA phase. However, during the Release 19 phase, it was identified that the 30 dBc MCL assumption was no longer valid for FR1 high bands. The detailed analysis is captured in clause 6.4.4 in TR 37.941. In the Release 20 phase, it is crucial to resolve these issues, particularly for FR1 BSs with massive MIMO operation. Appropriate requirement definitions can effectively guide and support the RF design e.g. front-end filters.

Objective 2:

SBFD (Subband Full Duplex) is a critical BS evolution technique in the 5G-Advanced phase. SBFD coexisting with SBFD BS is also one important scenario to make sure that SBFD BS could be well deployed especially for some bands without any legacy TDD BS deployed so far. During the RAN4#115 meeting, it was agreed to postpone further study of this scenario to a future release. We propose to include the study of SBFD coexistence with SBFD BS in Release 20 of the 5G-Advanced phase.

- 2.1.3 WID objective

Core part objective:

- Enhancement of co-location requirements

  - The applicable FR1 frequency ranges, e.g. >3.5GHz requirements;

  - Focus on the following RF requirements:

    - Transmitter OFF power requirement

    - Transmitter spurious emission requirement for Co-location with other base station/Protection of BS receiver of own or different BS

    - Transmitter intermodulation requirement

    - Co-location OOBB requirement

- SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios

  - Focus on FR1 WA and MR BS.

  - Example band: n104.

  - To specify the additional unwanted emissions and in-band blocking for SBFD coexisting with SBFD case

Performance part objective:

- To define the conformance testing requirement of co-location requirements

- To define the conformance testing requirement of additional SBFD-SBFD coexistence requirements;

- 2.1.4 Impacted specification

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.104 | NR; Base Station (BS) radio transmission and reception To capture the following aspects: - Additional RF requirement for SBFD coexisting with SBFD BS - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#115 | Core part |
| TS 37.104 | NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#115 | Core part |
| TS 37.105 | Active Antenna System (AAS) Base Station (BS) transmission and reception To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#115 | Core part |
| TS 38.106 | NR repeater radio transmission and reception To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#115 | Core part |
| TS 38.174 | NR; Integrated Access and Backhaul (IAB) radio transmission and reception To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#115 | Core part |
| TS 36.104 | Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#115 | Core part |
| TS 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing To capture the following aspects: - Additional RF requirement for SBFD coexisting with SBFD BS - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Per part |
| TS 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing To capture the following aspects: - Additional RF requirement for SBFD coexisting with SBFD BS - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |
| TS 37.141 | NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |
| TS 37.145-1 | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: conducted conformance testing To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |
| TS 37.145-2 | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |
| TS 38.115-1 | NR; Repeater conformance testing - Part 1: Conducted conformance testing To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |
| TS 38.176-1 | NR; Integrated Access and Backhaul (IAB) conformance testing; Part 1: Conducted conformance testing To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |
| TS 38.176-2 | NR; Integrated Access and Backhaul (IAB) conformance testing; Part 2: Radiated conformance testing To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |
| TS 36.141 | Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing To capture the following aspects: - Improved RF requirement for Enhancement of co-location requirements if needed | RAN#117 | Perf part |

Conclusions

In this contribution, we shared some further views on Rel-20 BS RF enhancement based on RAN#108 moderator’s way forward.

Proposal 1: propose to postpone the FR1 hybrid beamforming into 6G SI phase.

Proposal 2: Consider the justifications and objectives in sections 2.1.2 and 2.1.3 for the WID of BS RF

References

[1] RP-251879,Way forward on RAN4 R20 package, RAN4 Chair (Apple), Approved.