3GPP TSG-RAN Meeting #109	RP-252080

# Beijing, China, 15-18 September, 2025

Agenda item:		8.2.1.3

Source:	Xiaomi

Title:	Requirements for architecture and migration

Document for:		Discussion and Decision

# Introduction

In RAN#106 meeting, RAN plenary study item on “Study on 6G scenarios and requirements” [1] was approved. In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [2] was approved.

In this contribution, we discuss requirements for architecture and migration for 6G.

# Discussion

## Architecture requirements

### 5G	requirements applicable to 6G

5G requirements on architecture and migration (as in Annex) can be a reference for 6G and following 5G requirements can be applicable for 6G RAN:

-	The RAN architecture shall support connectivity through multiple transmission points (e.g. TRPs), either collocated or non-collocated.

-	The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall allow for the operation of Network Slicing.

-	The RAN architecture shall support network sharing of the RAN between multiple operators.

-	The RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

-	The RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	RAN-CN interfaces and RAN internal interfaces (both between 6G logical nodes/functions and between 6G and 5G NR logical nodes/functions) shall be open for multi-vendor interoperability.

### Function split related requirements

The function split related requirements defined in 5G are:

-	The RAN architecture shall support connectivity through multiple transmission points (e.g. TRPs), either collocated or non-collocated.

-	The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites.

-	The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	The RAN architecture shall allow for C-plane/U-plane separation.

5G NR supports CU-DU split as well as CP-UP split (i.e. CU-CP/CU-UP split). For CP-UP split, considering its complexity and no commercial deployment, it is proposed to not consider CP-UP split in 6G. A simplified 6G function split at RAN is shown in Figure 1 below.

Figure 1: Simplified 6G function split at RAN

Proposal 1: The RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

In addition, current CU-DU split architecture has a drawback that DU related L1/L2 configurations needs to be configured via CU RRC, and therefore the signalling has to go through the route of DUCUDUUE, which adds significant latency compared with direct transmission from DU to UE. Improvements are needed to avoid unnecessary configuration exchange between CU and DU.

Proposal 2: The RAN architecture shall support internal function split with low control signalling latency.

RAN WG 6G SID RP-251881 [2] has the following architecture related objectives:

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

5G NR signalling transfer between UE/RAN and CN, e.g., LMF is routed via AMF. There is scalability issue due to the load of AMF and potential single point of failure. These issues were already realized during 5G NR, and some workarounds were introduced. For example, originally LPP signalling was transferred via AMF in control plane, and enhancements were introduced to transport LPP signalling via user plane. For 6G, there is the opportunity to address above issues of RAN-CN interface in a systematic manner.

Proposal 3: The RAN architecture shall support signalling transfer between UE/RAN and CN for diverse services, e.g., sensing, AI in efficient and scalable way.

### Support of new services (AI and sensing)

6G RAN architecture should have efficient support of new services (AI/ML and sensing) from day 1. There is large amount of data (e.g. training data, sensing data) exchanged between UE, RAN, and CN, as shown in Table 1 and Figure 2 below.

Table 1: Data transfer between UE, RAN and CN

|  | UE and RAN | UE and CN | RAN and CN |
| --- | --- | --- | --- |
| AI / ML | UE collects / reports training data (e.g., measurement results) for NW-side model training. | For AI / ML UE-side data collection, UE transfer its collected data for UE-side model training to its own OTT server via application layer (i.e., transparent to 3GPP). | RAN forward the received training data from UE to OAM. To support two-sided model, NW transfers collected/measured information (e.g., dataset / model parameter) to UE-side OTT server for two-sided model training. Efficient data exchange and computing sharing between RAN and Core for AI/ML RAN use cases. |
| Sensing | Sensing results measured by UE is reported to RAN for communication optimization. | Sensing UE reports sensing results to either SF in CN or to sensing client directly | RAN forward the received sensing results from UE to sensing client. When TRP-monostatic/TRP-TRP bistatic is used, RAN can report sensing results to SF in CN. |

Figure 2: Data transfer between UE, RAN and CN

Different types of data could have different QoS requirements. For example, as discussed in Rel-19 AI/ML, training data for offline training is delay tolerant while data for inference is delay sensitive.

In light of above data transfer and processing requirements of AI/ML and sensing, RAN architecture shall allow for efficient transmission of large amount of data between UE, RAN and CN. It is envisioned that data plane between UE and RAN, UE and CN, and RAN and CN can be introduced with the following goals:

- Support innate data(e.g. AI / Sensing) transmission between UE and RAN, UE and CN, and RAN and CN.

- Reduce signaling overhead and transmission latency by bypassing AMF for AI/Sensing service.

Proposal 4: The RAN architecture shall support efficient transmission: 1) for diverse data types and QoS requirements; 2) between different end point pairs (UE and RAN, UE and CN, RAN and CN); 3) for diverse services (e.g. AI, sensing, SON/MDT).

## Migration requirements

For migration, RAN WG 6G SID RP-251881 [2] has the following objectives:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]
NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Migration options supported in 5G NR includes inter-RAT mobility / cell reselection, DSS, and MRDC.

In 5G study (as from TR 38.801 [2]), various migration options are considered: Option 2, 3/3A, 4/4A, 5, 7/7A. Typical migration options are shown in Table 2 below.

Table 2: Migration options in 5G

| Option | Architecture | Commercialized |
| --- | --- | --- |
| Standalone (Option 2) Note: Inter-RAT mobility and DSS are migration options on top of standalone option. |  | Yes |
| EN-DC (Option 3) |  | Yes |
| eLTE (Option 5) |  | No |
| NG-ENDC (Option 7) |  | No |

Lesson from 5G

The motivation to specify non-standalone architectures in NR (e.g. EN-DC) is to allow early deployment of NR, e.g. to cater for the smooth upgrade of 4G network and inter-operability between 4G and 5G. Consequently, several drops (ASN.1 freeze) were introduced due to the high load in RAN WGs, e.g. for NSA and SA. This causes both specification/network/UE complexity and additional CAPEX for operators. For example, EN-DC was shortly deployed in China, but was quickly phased out for switching to SA, which not only delayed SA deployment but also incurs more CAPEX. This is a lesson to be learned for 6G standardization. It was already agreed in RAN plenary#102 meeting that (in RP-233985 [4]) Release 21 is expected to be delivered with a single drop (i.e., a single code freeze).

Observation: Too many options (SA, DSS, EN-DC, NG-EN-DC) introduced in NR led significant load to WGs, caused both network/UE complexity and additional CAPEX for operators which shall be avoided in 6G.

RAN WG 6G SID RP-251881 [2] clearly states the support of standalone as follows: Single technology framework based on a stand-alone architecture to support the agreed existing and new services. It is therefore natural to support the mobility between 6G and 5G NR, including both CONNECTED mode mobility and cell reselection. Further migration options like MRSS (similar to DSS) can be also considered to provide smooth migration from 5G NR to 6G. MRDC has the issue of complexity of tight interworking at network side and UE side (as discussed above), and also increases the significant WG workload, market fragment, hence it is challenging to consider MRDC for migration from 5G NR to 6G.

On another hand, considering the supported high data rate in 5G, to avoid the impact on user experience, NR has improved the mobility interruption a lot with LTM, etc. We expect that the similar mechanisms will be introduced in 6G as well to meet the mobility requirements. However, the interruption and the data loss during traditional inter-RAT handover, e.g., 4G-5G cannot be avoided. We do see the requirements to improve the inter-RAT handover as well in order to avoid downgrade of the use experience. To minimize interruption and data loss during mobility between 5G and 6G (which is one drawback of inter-RAT HO), but avoid to introduce tight network requirement (e.g. DU complexity) and also high UE complexity, dual stack / registration can be considered as an alternative migration option, where data can be aggregated at core network.

Proposal 5: The RAN architecture shall support interworking between 5G NR and 6G with similar performance as intra-RAT mobility. Considering high performance inter-RAT mobility and aggregation of data flows at core network between 5G NR and 6G. This shall be supported for both collocated and non-collocated site deployments.

Proposal 6: Capture following architecture and migration requirements in the TR 38.814:

The 6G RAN design shall fulfil the following requirements:

-	The RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

-	The RAN architecture shall support internal function split with low control signalling latency.

-	The RAN architecture shall support interworking between 5G NR and 6G with similar performance as intra-RAT mobility.

-	Considering high performance inter-RAT mobility and aggregation of data flows at core network between 5G NR and 6G. This shall be supported for both collocated and non-collocated site deployments.

-	The RAN architecture shall support connectivity through multiple transmission points (e.g. TRPs), either collocated or non-collocated.

-	The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall allow for the operation of Network Slicing.

-	The RAN architecture shall support network sharing of the RAN between multiple operators.

-	The RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The RAN architecture shall support efficient transmission:

-	for diverse data types and QoS requirements;

-	between different end point pairs (UE and RAN, UE and CN, RAN and CN);

-	for diverse services (e.g. AI, sensing, SON/MDT).

-	The RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

-	The RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	RAN-CN interfaces and RAN internal interfaces (both between 6G logical nodes/functions and between 6G and 5G NR logical nodes/functions) shall be open for multi-vendor interoperability.

Note: the highlighted parts are new requirements compared to those defined in 5G.

# Conclusion

In this contribution, we discuss requirements for architecture and migration and propose the following:

Proposal 1: The RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

Proposal 2: The RAN architecture shall support internal function split with low control signalling latency.

Proposal 3: The RAN architecture shall support signalling transfer between UE/RAN and CN for diverse services, e.g., sensing, AI in efficient and scalable way.

Proposal 4: The RAN architecture shall support efficient transmission: 1) for diverse data types and QoS requirements; 2) between different end point pairs (UE and RAN, UE and CN, RAN and CN); 3) for diverse services (e.g. AI, sensing, SON/MDT).

Observation: Too many options (SA, DSS, EN-DC, NG-EN-DC) introduced in NR led significant load to WGs, caused both network/UE complexity and additional CAPEX for operators which shall be avoided in 6G.

Proposal 5: The RAN architecture shall support interworking between 5G NR and 6G with similar performance as intra-RAT mobility. Considering high performance inter-RAT mobility and aggregation of data flows at core network between 5G NR and 6G. This shall be supported for both collocated and non-collocated site deployments.

Proposal 6: Capture following architecture and migration requirements in the TR 38.814:

The 6G RAN design shall fulfil the following requirements:

-	The RAN architecture shall support simplified internal function split with reduced deployment options compared with 5G NR.

-	The RAN architecture shall support internal function split with low control signalling latency.

-	The RAN architecture shall support interworking between 5G NR and 6G with similar performance as intra-RAT mobility.

-	Considering high performance inter-RAT mobility and aggregation of data flows at core network between 5G NR and 6G. This shall be supported for both collocated and non-collocated site deployments.

-	The RAN architecture shall support connectivity through multiple transmission points (e.g. TRPs), either collocated or non-collocated.

-	The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall allow for the operation of Network Slicing.

-	The RAN architecture shall support network sharing of the RAN between multiple operators.

-	The RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The RAN architecture shall support efficient transmission:

-	for diverse data types and QoS requirements;

-	between different end point pairs (UE and RAN, UE and CN, RAN and CN);

-	for diverse services (e.g. AI, sensing, SON/MDT).

-	The RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

-	The RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	RAN-CN interfaces and RAN internal interfaces (both between 6G logical nodes/functions and between 6G and 5G NR logical nodes/functions) shall be open for multi-vendor interoperability.

# 4 References

[1] RP-243327, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, “New SID: Study on 6G Scenarios and requirements”

[2] RP-251881, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

[3] 3GPP TR 38.801, “Study on new radio access technology: Radio access architecture and interfaces”

[4] RP-233985, TSG Chairs, “High-level Considerations for 6G Timeline”

# 5 Annex Requirements for architecture and migration of NR (TR 38.913 clause 8)

The RAN design for the Next Generation Radio Access Technologies shall be designed to fulfil the following requirements:

-	The RAN architecture shall support tight interworking between the new RAT and LTE.

-	Considering high performing inter-RAT mobility and aggregation of data flows via at least dual connectivity between LTE and new RAT. This shall be supported for both collocated and non-collocated site deployments.

-	The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated.

-	The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites.

-	The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	Different options and flexibility for splitting the RAN architecture shall be allowed.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

-	The RAN architecture shall allow for C-plane/U-plane separation.

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall allow for the operation of Network Slicing3GPP TR 23.799 [7].

-	The RAN architecture shall support sharing of the RAN between multiple operators.

-	The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

-	The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	RAN-CN interfaces and RAN internal interfaces (both between new RAT logical nodes/functions and between new RAT and LTE logical nodes/functions) shall be open for multi-vendor interoperability.

-	The RAN architecture shall support operator-controlled sidelink (device-to-device) operation, both in coverage and out of coverage.